| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0637 |
| --- | --- |

| Title: | Crosscheck of JVET-N0078 (CE2-5.6: Simplifications for inherited affine merging candidates at CTU row boundaries) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Han Huang | Email: | hanhuang@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution is a crosscheck report of Test b and Test c in JVET-N0078 (CE2-5.6: Simplifications for inherited affine merging candidates at CTU row boundaries). The simulations were carried out based on the common test condition (CTC). The available test results are matched with the tester results and the software matches the description in the proposal.

# Introduction

In JVET-N0078, it’s proposed to use two subblock motion vectors (MVs) with a fixed distance between the two subblocks for affine candidate inheritance, where an accurate distance is used to derive the affine motion and “CU width and x position information” are not needed anymore.

# Simulation results

Table 1 CE2-5.6.b crosschecker results

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.06% | 0.08% | 101% | 101% |
| Class C | 0.00% | -0.09% | -0.08% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.01% | 0.03% | -0.06% | 98% | 99% |
| Class F | 0.00% | 0.03% | 0.01% | 100% | 100% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.11% | 0.17% | 100% | 102% |
| Class C | -0.01% | 0.11% | 0.00% | 100% | 99% |
| Class E | 0.06% | -0.03% | 0.12% | 100% | 98% |
| Overall | 0.02% | -0.02% | 0.10% | 100% | 100% |
| Class D | 0.03% | 0.29% | -0.29% | 101% | 100% |
| Class F | -0.10% | 0.01% | -0.28% | 101% | 99% |

Table 2 CE2-5.6.c crosschecker results

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | -0.07% | -0.05% | 101% | 101% |
| Class C | 0.02% | 0.00% | -0.03% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.01% | 98% | 100% |
| Class F | 0.00% | 0.05% | -0.02% | 100% | 99% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.15% | -0.33% | 100% | 100% |
| Class C | -0.02% | -0.13% | -0.06% | 100% | 99% |
| Class E | -0.06% | 0.07% | -0.37% | 100% | 99% |
| Overall | -0.02% | 0.03% | -0.25% | 100% | 99% |
| Class D | 0.01% | -0.14% | 0.18% | 100% | 97% |
| Class F | -0.09% | -0.33% | 0.00% | 100% | 99% |

# Patent rights declaration(s)

Qualcomm does not have any current or pending patent rights relating to the technology described in this contribution.