| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0640-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-N0174 (Non-CE8: Exclusive regular merge and IBC merge in one shared merge list area) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Cheng Lin No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 yucheng.lin@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The document reports the results of crosschecking JVET-N0174 proposed by Huawei Technologies. JVET- N0174 proposes a method to disallow simultaneous selection of IBC merge and regular merge modes inside a same shared merge region. The verification task was done, and the BD-rate results match exactly with those in JVET-N0174.

# Verification

Huawei Technologies provided the source code of their proposed method described in JVET-N0174 [1].

The tests were conducted under the the common test conditions (CTC) defined in [2] using the VTM4.0 software [3], and VTM4.0 are used as the anchor except that the anchor enables IBC. Additional tests for class F and TGM sequences are also conducted, and HashME is enabled for IBC when testing these sequences. The BD-rates of our experiments match exactly those in JVET-N0174. Our runtime comparison may be not accurate.

Table 1.  The results of JVET-N0174 Test 1 using VTM4.0 as anchor

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | -0.01% | 100% | 100% |
| Class A2 | 0.01% | -0.14% | 0.02% | 100% | 98% |
| Class B | 0.01% | -0.19% | -0.08% | 100% | 100% |
| Class C | 0.00% | 0.23% | 0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | 0.01% | 100% | 100% |
| Class D | -0.01% | -0.06% | -0.04% | 100% | 100% |
| Class F | -0.02% | 0.00% | -0.02% | 100% | 100% |
| TGM | 0.13% | 0.20% | 0.14% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.11% | 0.06% | 100% | 101% |
| Class C | -0.05% | -0.01% | 0.12% | 100% | 99% |
| Class E | 0.06% | -0.80% | 0.59% | 99% | 100% |
| Overall | -0.01% | -0.16% | 0.22% | 100% | 100% |
| Class D | -0.04% | 1.19% | 0.33% | 99% | 100% |
| Class F | -0.09% | 0.02% | -0.24% | 100% | 101% |
| TGM | 0.11% | 0.17% | 0.14% | 99% | 100% |

# References

- H. Gao, S. Esenlik, B. Wang, A. M. Kotra, and J. Chen, “Non-CE8: Exclusive Regular Merge And IBC Merge in One Shared Merge List Area,” Document of Joint Video Experts Team, JVET-N0174.

- F. Bossen, J. Boyce, K. Sühring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

- VTM4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-4.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).