| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0643-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-N0486 (CE4-Related: Simplification for share merge list and HMVP harmonization process) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Cheng Lin No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 yucheng.lin@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The document reports the results of crosschecking JVET-N0486 proposed by SZ DJI Technology and Peking University. JVET-N0486 proposes a method to simplify the current design of harmonization for HMVP and share merge list. The verification task was done. It is observed that there is 0.01% Cb BD-rate difference in class A1 in random access in Test 2 with those in JVET-N0486. After investigation, it is concluded that the difference is caused by a PSNR calculation accuracy problem. Therefore, the BD-rate results can be regarded as matched.

# Verification

SZ DJI Technology and Peking University provided the source code of their proposed method described in JVET-N0486 [1].

The tests were conducted under the the common test conditions (CTC) defined in [2] using the VTM4.0 software [3]. It is observed that there is 0.01% Cb BD-rate difference in class A1 in random access in Test 2 with those in JVET-N0486. After investigation, we conclude this difference is caused by a PSNR calculation accuracy problem. Therefore, the BD-rate result can be treated as matched. Our runtime comparison may be not accurate. Table 1 and Table 2 show our results.

Table 1.  The results of JVET-N0486 Test 1 using VTM4.0 as anchor

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | -0.03% | 102% | 99% |
| Class A2 | -0.01% | -0.06% | 0.03% | 103% | 104% |
| Class B | 0.01% | 0.01% | 0.06% | 102% | 99% |
| Class C | 0.01% | -0.08% | -0.12% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.01% | 101% | 100% |
| Class D | -0.02% | -0.01% | 0.06% | 99% | 100% |
| Class F | 0.02% | -0.09% | -0.16% | 99% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.09% | 0.00% | 97% | 99% |
| Class C | -0.01% | 0.26% | 0.02% | 98% | 98% |
| Class E | 0.03% | -0.01% | -0.70% | 100% | 100% |
| Overall | 0.00% | 0.12% | -0.17% | 98% | 99% |
| Class D | 0.00% | 0.02% | -0.64% | 99% | 101% |
| Class F | -0.15% | -0.28% | -0.46% | 97% | 102% |

Table 2.  The results of JVET-N0486 Test 2 using VTM4.0 as anchor

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.07% | 0.03% | 103% | 102% |
| Class A2 | -0.02% | -0.09% | 0.05% | 103% | 101% |
| Class B | 0.00% | -0.02% | -0.02% | 103% | 100% |
| Class C | -0.01% | -0.01% | -0.05% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | -0.01% | 102% | 101% |
| Class D | -0.03% | 0.02% | -0.04% | 100% | 98% |
| Class F | 0.01% | -0.08% | -0.14% | 102% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.13% | 0.05% | 100% | 102% |
| Class C | 0.00% | 0.09% | -0.11% | 99% | 99% |
| Class E | -0.06% | -0.16% | -0.25% | 99% | 100% |
| Overall | -0.02% | -0.07% | -0.08% | 100% | 101% |
| Class D | -0.11% | -0.03% | -0.35% | 103% | 103% |
| Class F | -0.28% | -0.01% | -0.41% | 98% | 101% |

# References

- S.H. Wang, X. Zheng, S. S. Wang, and S. W. Ma, “CE4-related: Simplification for share merge list and HMVP harmonization process,” Document of Joint Video Experts Team, JVET-N0486.

- F. Bossen, J. Boyce, K. Sühring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

- VTM4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-4.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).