| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, March 2019 | Document: JVET-N0653 |
| --- | --- |

| Title: | Crosscheck of JVET-N0110 (CE 13.1.1: Convolutional neural network loop filter) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hujun Yin, Rongzhen Yang, Xiaoran Fang, Shoujiang Ma, 2200 Mission College Blvd, Santa Clara, CA, 95054, USA | Tel: Email: | +1 408 410 5081 {Hujun.yin, Rongzhen.yang} @intel.com |
| Source: | Intel Corporation |  |  |

_____________________________

# Abstract

This contribution reports a cross-checking of contribution JVET-N0110 related to convolution neural network loop filter (CNNLF).

It is reported that the result obtained during this cross-checking matches the result described in JVET-N0110 [1].

# Experimental Results

The cross-check test is performed on top of VTM4.0 for RA. The corresponding simulations were conducted on an Intel Xeon cluster with SuSE Linux OS and a GCC 8.2.0 compiler. The test proposed by CE13.1.1a in JVET-N0110 have been crosschecked

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.95% | -8.57% | -13.33% | 100% | 180% |
| Class A2 | -1.47% | -18.33% | -15.72% | 104% | 125% |
| Class B | -1.52% | -23.99% | -21.70% | 100% | 143% |
| Class C | 0.12% | -5.94% | -6.99% | 102% | 119% |
| Class E |  |  |  |  |  |
| Overall | -1.36% | -14.96% | -14.91% | 101% | 139% |

# Conclusions

Results match the one presented by the proponents in JVET-N0110. Encoder and decoder run times provided by the cross-checker is inaccurate.

# Reference

- Y.-L. Hsiao, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “CE13-1.1: Convolutional neural network loop filter” Document of Joint Video Experts Team, JVET-N0110. Mar 2019

.