| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0665 |
| --- | --- |

| Title: | Crosscheck of JVET-N0230 (Non-CE3: Simplified intra mode candidates for ISP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jinho Lee Sung-Chang Lim Hahyun Lee Jungwon Kang 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr sclim@etri.re.kr hanilee@etri.re.kr jungwon@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution reports crosscheck results of simplified intra mode candidates for ISP proposed in JVET-N0230. It only allows Planar, Horizontal, and Vertical mode for intra sub-partitioning (ISP) mode. It is reported that the average luma BD-rate of the proposed method for AI case is 0.01% compared to VTM 4.0.

- Introduction

In VTM 4.0, there are three kinds of MPM list derivation method according to whether intra sub-partitioning (ISP) or multi-reference line (MRL) is enabled. JVET-N0230 removes MPM derivation process when ISP is enabled and limits intra prediction modes to Planar, Horizontal, and Vertical mode for ISP. When ISP is enabled, ISP intra mode flag is signaled and if the flag is equal to 1, Planar mode is used for the ISP block. If the flag is equal to zero, Horizontal mode when ISP split type is horizontal split and Vertical mode when ISP split type is vertical split is selected, respectively.

Experimental results

Experiments were performed based on VTM 4.0 software under JVET common test condition.

Table 1 shows experimental results of the proposed method compared to VTM 4.0. All the results are matched with the ones in JVET-N0230.

Table 1. Results of the proposed method over VTM 4.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.07% | -0.02% | 102% | 100% |
| Class A2 | -0.04% | 0.04% | -0.06% | 101% | 101% |
| Class B | 0.04% | 0.12% | 0.08% | 101% | 100% |
| Class C | 0.06% | 0.06% | 0.03% | 100% | 100% |
| Class E | 0.06% | 0.27% | 0.20% | 100% | 100% |
| Overall | 0.01% | 0.08% | 0.05% | 101% | 100% |
| Class D | -0.07% | 0.06% | -0.07% | 101% | 100% |
| Class F | -0.10% | 0.09% | -0.09% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.15% | -0.14% | 100% | 100% |
| Class A2 | -0.02% | -0.11% | -0.13% | 99% | 100% |
| Class B | 0.04% | 0.08% | 0.00% | 100% | 100% |
| Class C | 0.04% | 0.06% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.01% | -0.06% | 100% | 100% |
| Class D | 0.00% | -0.18% | -0.42% | 100% | 100% |
| Class F | -0.14% | -0.21% | -0.23% | 100% | 100% |