| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0668 |
| --- | --- |

| Title: | Crosscheck of JVET-N0445 (Non-CE6: MTS kernel derivation for efficient memory usage) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaeil Kim | Email: | jaeil1203@sk.com |
| Source: | SK Telecom |  |  |

_____________________________

- Abstract

This document reports cross-check results of JVET-N0445 where a derivation scheme for an Multiple Transform Selection (MTS) kernel is applied to save memory consumption for kernel storage. It produces a complete matrix kernel of DST-7 from only a few numbers of rows of DST-7, a matrix kernel of DCT-8 from that of DST-7 with simple sign change and flipping computations, and a matrix of DCT-2 from a sparse unified matrix by using DST-3 matrices with a flipping and sign changing matrices. It is obvious that the cross-check results are matched with the experimental results provided by the proponent of JVET-N0445 [1] with negligible EncT and DecT differences.

Cross-check results

The source code based on VTM-4.0 was delivered by the proponent provided by the proponent [1]. The experiments were done under the common test conditions (CTC) [2] with MTS-off. As is same to the proponent, tables show BD-rate results of JVET-N0445 for AI, LDB10, RA configuration.

< Simulation results over VTM-4.0 >

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.08% | 0.02% | 100% | 102% |
| Class A2 | 0.01% | 0.02% | 0.04% | 100% | 100% |
| Class B | 0.00% | -0.02% | -0.01% | 100% | 99% |
| Class C | -0.01% | 0.01% | -0.12% | 100% | 99% |
| Class E | -0.02% | -0.01% | 0.05% | 100% | 100% |
| Overall | 0.00% | -0.01% | -0.01% | 100% | 100% |
| Class D | -0.01% | 0.15% | 0.03% | 101% | 99% |
| Class F | -0.01% | 0.06% | 0.07% | 100% | 99% |

|  | Random access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.14% | -0.15% | 99% | 99% |
| Class A2 | 0.02% | 0.00% | 0.02% | 98% | 99% |
| Class B | -0.01% | 0.09% | -0.22% | 100% | 99% |
| Class C | 0.02% | -0.08% | -0.01% | 99% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.10% | 99% | 100% |
| Class D | 0.01% | 0.22% | -0.34% | 99% | 104% |
| Class F | -0.01% | 0.07% | 0.09% | 99% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.12% | 0.15% | 100% | 100% |
| Class C | 0.04% | 0.29% | -0.06% | 100% | 99% |
| Class E | -0.07% | -0.18% | -0.38% | 100% | 100% |
| Overall | 0.00% | 0.00% | -0.05% | 100% | 100% |
| Class D | 0.08% | -0.35% | 0.65% | 100% | 100% |
| Class F | -0.08% | 0.21% | -0.41% | 100% | 100% |

Reference

- S. Shrestha, A. Kumar, B. Lee, Y. Lee, J. Park, “Non-CE6: MTS kernel derivation for efficient memory usage,” JVET-N0445, 14th JVET meeting, Geneva, CH, Mar. 2019

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software      reference configurations,” Document of Joint Video Experts Team, JVET-K1010.