| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0669 |
| --- | --- |

| Title: | Crosscheck of JVET-N0532 (AHG19: Resampling filters for adaptive resolution change (ARC) in VVC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check Report |  |  |
| Author(s) or Contact(s): | Edouard François | Tel: Email: | edouard.francois@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results for contribution JVET-N0532 (AHG19: Resampling filters for adaptive resolution change (ARC) in VVC). Simulation results reportedly match those provided by the proponents.

# Introduction

JVET- N0532 relates to the down/upsampling filters used in adaptive resolution change. Two different set of filters are considered and tested.

The first set of filters, referred as anchor by the proponent, is made of a zero-phase separable 5-tap downsampling filter and an 8-tap upscaling filter. The second set of filters, corresponding to the proposal, is made of a 9-tap downsampling filter and a 4-tap upsampling filter.

# Experimental Results

The windows executable as well as the source code was delivered to the cross-checker. The code was quickly checked, and no inconsistency with the proposal was identified.

The cross-checker has cross-checked simulation results from both versions. Tests were achieved on a different basis than the CTCs. Tests were done in AI configuration, on a 1 second clip, and all frames of the 1s clip were processed.

Simulation results reported in JVET-N0532 are confirmed by the cross-checker.

Results using “anchor” filters version are reported below.

| Sequences | Y | U | V |
| --- | --- | --- | --- |
| BasketballDrive | 33.654 | 40.123 | 39.951 |
| Cactus | 31.595 | 36.825 | 37.899 |
| MarketPlace | 36.745 | 40.848 | 42.035 |
| Campfire | 35.681 | 34.077 | 37.718 |
| CatRobot | 37.941 | 39.63 | 39.43 |
| FoodMarket | 43.933 | 45.2 | 45.396 |
| ParkRunning | 37.892 | 30.618 | 32.716 |

Results using proponent filters version are reported below.

| Sequences | Y | U | V |
| --- | --- | --- | --- |
| BasketballDrive | 33.981 | 40.397 | 40.329 |
| Cactus | 31.629 | 37.085 | 38.166 |
| MarketPlace | 37.191 | 41.047 | 42.19 |
| Campfire | 36.125 | 34.526 | 37.86 |
| CatRobot | 38.209 | 39.779 | 39.747 |
| FoodMarket | 43.972 | 45.258 | 45.502 |
| ParkRunning | 38.75 | 31.119 | 33.198 |

Regarding bit-rates variations, a 10% to 20% increase is observed when encoding downsampled sequences with the proposed filters, compared to encoding downsampled sequences with the anchor filters.