| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 |  |  |  | Document: JVET-N0671-v1 |  |
| --- | --- | --- | --- | --- | --- |
| Title: | Support of 4:4:4 and 4:2:2 chroma formats in VVC |  |  |  |  |  |
| Status: | Input document to JVET |  |  |  |  |  |
| Purpose: | Proposal |  |  |  |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Timofey Solovyev, Roman Chernyak, Jianle Chen, Xin Zhao, Xiang Li, Xiaozhong Xu, Shan Liu, Hongtao Wang, Yung-Hsuan Chao, Wei-Jung Chien, Vadim Seregin, Marta Karczewicz | Email: | alexey.filippov@huawei.com, vasily.rufitskiy@huawei.com, solovyev.timofey@huawei.com, chernyak.roman@huawei.com, jianle.chen@huawei.com, xinzzhao@tencent.com, xlxiangli@tencent.com, xiaozhongxu@tencent.com, shanl@tencent.com, hongtaow@qti.qualcomm.com, yunghsua@qti.qualcomm.com, wchien@qti.qualcomm.com, vseregin@qti.qualcomm.com, martak@qti.qualcomm.com |  |  |  |
| Source: | Huawei Technologies Co., Tencent, Qualcomm Inc. |  |  |  |  |  |

_____________________________

# Abstract

This document proposes modifications of the VTM software and the VVC spec draft to support YUV4:4:4, RGB4:4:4, and YUV4:2:2. For enabling that, straightforward changes of CCLM and affine sub-block motion vector derivation were made. In addition, some software bugs affected YUV 4:4:4 and YUV 4:2:2 were fixed. Besides, a minor editorial change is proposed for ISP. In order to evaluate compression performance, we use the HM-16.18+SCM8.7 software as the anchor. The proposed modifications reportedly show the following overall results with RExt cfg-files for YUV4:4:4 content: -21.8% (Y), - 13.7% (U), -14.2% (V) with encoding time of 1475% and decoding time of 141% for AI configuration, as well as - 31.9% (Y), -23.0% (U), -19.4% (V) with encoding time of 576% and decoding time of 128% for RA configuration. The proposed modifications reportedly show the following overall results with RExt cfg-files for RGB4:4:4 content: -21.7% (Y), - 14.5% (U), -15.3% (V) with encoding time of 2000% and decoding time of 143% for AI configuration, as well as - 31.7% (Y), -19.3% (U), -21.7% (V) with encoding time of 723% and decoding time of 134% for RA configuration. The proposed modifications reportedly show the following overall results with RExt cfg-files for YUV4:2:2 content: -17.8% (Y), - 19.5% (U), - 20.0% (V) with encoding time of 2435% and decoding time of 157% for AI configuration, as well as - 22.1% (Y), -25.9% (U), -23.2% (V) with encoding time of 665% and decoding time of 138% for RA configuration.

# Introduction

The VTM software natively supports YUV 4:2:0 content as input format. However, VTM-4.0 encoder crash happens with the following configuration:

InputChromaFormat             : 444

However, support of other chroma formats (mainly, 4:4:4) is highly desirable and even mandatory for a wide range of applications. Meanwhile, consistency with the current design of VVC would be an attractive feature that can provide its wide deployment.

## CCLM mode

CCLM (Cross-Component Linear Model) is a coding tool of VVC draft 4. The mode uses luma reconstructed samples to predict chroma samples by using a linear model:

predCi,j=α⋅recL'i,j+β,

where recL'i,j is a luma sample values obtained from the reconstructed luma block. The parameters α and β are derived from the reconstructed luma and chroma samples from the top and left neighbouring blocks. For YUV 4:2:0 format, each chroma sample has 4 collocated luma samples, so both luma samples used to derive α  and β and the samples used for calculating predicted chroma samples are down sampled (Fig. 1).

Fig. 1 Luma and Chroma samples involved in CCLM mode in YUV4:2:0 [1]

## Affine mode

In VVC draft 4, for affine mode, the motion vectors of a chroma 4x4 block is defined based on the average motion vector of 2 luma 4x4 blocks (Fig. 2).

Fig. 2 Motion vector derivation of affine chroma block

# Proposal

## CCLM mode

In the case of YUV 4:4:4 format, downsampling filtering is not performed for CCLM mode since luma and chroma CBs have the same size.

Fig. 3 Locations of samples used for CCLM mode for YUV 4:4:4 format

In the case of YUV 4:2:2 format, horizontal [1, 2, 1]/4 downsampling filter is used since chroma CBs are decimated in horizontal direction.

## Affine mode

In the case of YUV 4:4:4 format, it is proposed to use the motion vector(s) of the collocated luma block for each chroma 4x4 block to perform affine prediction, i.e. affine sub-block motion vector derivation process for chroma is same as for luma.

In the case of YUV 4:2:2 format, affine sub-block motion vector derivation process for chroma subblock uses averaging for motion vectors of two horizontally-adjacent luma sub-blocks.

Fig. 4 Motion vector derivation for chroma 4x4 block of YUV 4:4:4 format

## Software fixes

In the function xProcessDMVR(,) where DMVR processing is performed, chroma format 4:2:0 is assumed for calculating chroma pixel position. Thus, in the case of YUV4:4:4, wrong chroma pixel positions is calculated in VTM-4.0. It is proposed to fix this bug by using the values of ComponentScaleX and ComponentScaleY when computing chroma positions.

In the functions to calculate SSE values such as xGetSSE_WTD(), xGetSSE2_WTD(), xGetSSE4_WTD(), xGetSSE8_WTD(), xGetSSE16_WTD(), xGetSSE16N_WTD(), xGetSSE32_WTD(), and xGetSSE64_WTD(), the ‘cShift’ value, which is used to calculate luma pixel positions and step size of pixel lines also assume the chroma format is 4:2:0. We propose to set ‘cShift’ value to 0.

For RGB support, a minor change has been introduced in the function VideoIOYuv::read(). The modification consists in copying converted original picture to the buffer containing the “true original” picture. The need for modification was caused by in-loop reshaper that references the “true original” picture buffer. Without this change, when a non-normative RGB-to-GBR transform of original picture is performed, in-loop reshaper erroneously uses the “R” plane instead of intended “G” plane.

## Editorial change

A minor editorial change is proposed for ISP as shown below:

| if( IntraSubPartitionsSplitType != ISP_NO_SPLIT && treeType = = SINGLE_TREE && subTuIndex = = NumIntraSubPartitions − 1 ) ) |  |
| --- | --- |
| xC = CbPosX[ x0 ][ y0 ] |  |
| yC = CbPosY[ x0 ][ y0 ] |  |
| wC = CbWidth[ x0 ][ y0 ] / SubWidthC2 |  |
| hC = CbHeight[ x0 ][ y0 ] / SubHeightC2 |  |
| } else |  |
| xC = x0 |  |
| yC = y0 |  |
| wC = tbWidth / SubWidthC |  |
| hC = tbHeight / SubHeightC |  |
| } |  |

# Experiment Results

Compression performance for YUV 4:4:4, YUV 4:2:2 and RGB content was evaluated under the JVET CTC conditions [2] using the JCT-VC Range Extension (RExt) CTC [3] test set. The anchor is HM-16.18+SCM8.7 and the test is VTM-4.0 with proposed fixes.

The results reportedly obtained with RExt cfg-files are presented in Table 1 for AI and RA configurations.

Table 1. Experimental results as compared with HM-16.18 Rext

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB 4:4:4 | -21.7% | -14.5% | -15.3% |
| YCbCr 4:4:4 | -21.8% | -13.7% | -14.2% |
| YCbCr 4:2:2 | -17.8% | -19.5% | -20.0% |
| Enc Time[%] | 1933% |  |  |
| Dec Time[%] | 147% |  |  |
|  | Random Access |  |  |
|  | Y | U | V |
| RGB 4:4:4 | -31.7% | -19.3% | -21.7% |
| YCbCr 4:4:4 | -31.9% | -23.0% | -19.4% |
| YCbCr 4:2:2 | -22.1% | -25.9% | -23.2% |
| Enc Time[%] | 655% |  |  |
| Dec Time[%] | 133% |  |  |

# Conclusion

We propose to adopt the proposed modifications into the VTM software and the VVC spec draft.

# References

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 4 (VTM 4)” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M1002, Jan. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, 13th JVET Meeting, Marrakesh, MA, January 2018.

- C. Rosewarne, K. Sharman, D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006, 16th JCT-VC Meeting, San Jose, CA, January 2014.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd, Tencent, and Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).