| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0674 |
| --- | --- |

| Title: | Crosscheck of JVET-N0373 (Non-CE4: HMVP unification between the Merge and MVP list) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Ruoyang Yu | Email: | ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The JVET-N0373 proposes modifications on the usage of HMVP candidates in merge candidate list construction and AMVP candidate list construction.

This contribution reports the cross-check results of the JVET-N0373. The BD-rate results reported in this cross-check match with the results provided by the proponent.

# Results

The following three tests were run under VTM-4.0 RA and LDB common test configurations [1]:

- Test #1 (modification 1): Change the scanning order of HMVP candidate during merge candidate list construction to be from the first entry to the last entry. Remove pruning checks when adding HMVP candidates to the merge candidate list.

- Test #2 (modification 2): Change the maximum number of allowed HMVP candidates to be checked during AMVP candidate list construction to 5.

- Test #3: modification 1+ modification 2.

The BD-rate results reported in this contribution for the three tests match with the results in JVET-N0373.

It should be noted that encoder and decoder run time are inaccurate since the anchor and tests were run on machines with different capabilities.

## Test 1

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.17% | 0.09% | 110% | 92% |
| Class A2 | 0.01% | -0.10% | 0.07% | 108% | 89% |
| Class B | 0.04% | 0.05% | 0.03% | 97% | 85% |
| Class C | 0.06% | 0.06% | 0.02% | 104% | 90% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.05% | 0.05% | 104% | 88% |
| Class D | 0.04% | -0.05% | 0.04% | 102% | 88% |
| Class F | 0.07% | -0.03% | -0.06% | 104% | 81% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.02% | 0.47% | 0.08% | 117% | 102% |
| Class E | 0.04% | 0.54% | 0.12% | 94% | 76% |
| Overall |  |  |  |  |  |
| Class D | -0.02% | 0.06% | -0.06% | 86% | 78% |
| Class F | -0.08% | 0.25% | -0.17% | 98% | 79% |

Note: The low delay simulation for one of the Class B sequences was still running when this report was uploaded. For the other finished Class B simulations, the BD-rate results were matched.

## Test 2

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.01% | 105% | 82% |
| Class A2 | -0.01% | 0.09% | 0.02% | 107% | 81% |
| Class B | 0.00% | 0.02% | 0.08% | 99% | 82% |
| Class C | 0.00% | 0.00% | -0.06% | 95% | 85% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.02% | 101% | 83% |
| Class D | 0.00% | -0.07% | -0.06% | 96% | 75% |
| Class F | 0.00% | 0.04% | -0.01% | 104% | 72% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.08% | -0.24% | 108% | 92% |
| Class C | -0.03% | 0.28% | -0.27% | 99% | 89% |
| Class E | -0.02% | 0.06% | -0.24% | 104% | 83% |
| Overall | -0.02% | 0.07% | -0.25% | 104% | 89% |
| Class D | -0.02% | 0.16% | -0.15% | 95% | 85% |
| Class F | 0.03% | 0.05% | -0.34% | 103% | 81% |

## Test 3

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.18% | 0.07% | 101% | 91% |
| Class A2 | 0.02% | -0.01% | 0.11% | 107% | 89% |
| Class B | 0.05% | 0.02% | 0.00% | 105% | 92% |
| Class C | 0.05% | -0.04% | -0.01% | 102% | 88% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.03% | 0.03% | 104% | 90% |
| Class D | 0.03% | -0.11% | -0.05% | 101% | 81% |
| Class F | 0.07% | -0.05% | -0.07% | 95% | 79% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.00% | -0.34% | 125% | 97% |
| Class C | 0.00% | 0.23% | 0.13% | 116% | 102% |
| Class E | 0.02% | 0.47% | -0.13% | 95% | 78% |
| Overall | 0.01% | 0.19% | -0.13% | 114% | 93% |
| Class D | -0.03% | 0.38% | -0.52% | 89% | 79% |
| Class F | 0.02% | -0.12% | 0.22% | 95% | 79% |

# References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010.