| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0677-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-N0095 (CE8-related: Unified method for coding BVD and MVD) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun | Tel: Email: | +1-8583492818 yuchen.s@alibaba-inc.com |
| Source: | Alibaba Cloud Computing Ltd. |  |  |

_____________________________

- Abstract

This document reports crosscheck results of JVET-N0095 (CE8-related: Unified method for coding BVD and MVD). The crosscheck results match the ones provided by the proponent except one data point is still running.

Introduction

In JVET-N0095, a unified method for coding BVD and MVD is proposed.

Simulation Results

Simulation results with IBC on under AI, RA and LDB configurations over VTM-4.0 anchor are shown below.

Table 1. JVET-N0677 with IBC on

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | -0.05% | 101% | 105% |
| Class A2 | 0.00% | -0.04% | -0.04% | 97% | 100% |
| Class B | -0.01% | -0.05% | -0.03% | 97% | 100% |
| Class C | -0.02% | 0.08% | -0.06% | 99% | 95% |
| Class E | -0.04% | -0.12% | -0.07% | 98% | 96% |
| Overall | -0.02% | -0.03% | -0.05% | 98% | 99% |
| Class D | 0.01% | -0.19% | 0.14% | 99% | 97% |
| Class F (optional) | -0.47% | -0.51% | -0.60% | 97% | 98% |
| TGM | -1.41% | -1.48% | -1.48% | 99% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.20% | -0.11% | 100% | 99% |
| Class A2 | -0.07% | -0.33% | -0.09% | 100% | 100% |
| Class B | -0.03% | -0.22% | -0.41% | 98% | 99% |
| Class C | -0.06% | 0.03% | -0.13% | 98% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.17% | -0.21% | 99% | 98% |
| Class D | -0.03% | -0.23% | -0.01% | 98% | 95% |
| Class F (optional) | -0.41% | -0.57% | -0.69% | 98% | 98% |
| TGM | -1.20% | -1.13% | -1.21% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.04% | -0.11% | 0.04% | 102% | 101% |
| Class E | -0.08% | -1.43% | 0.91% | 100% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.11% | 1.24% | 0.43% | 101% | 101% |
| Class F (optional) | -0.43% | -0.70% | -0.82% | 103% | 104% |
| TGM | -0.99% | -0.98% | -0.99% | 99% | 101% |

References

- S.-T. Hsiang, S.-M. Lei, “CE8-related: Unified method for coding BVD and MVD,” JVET-N0095, 14th Meeting: Geneva, CH, 19–27 March 2019.