| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0685 |
| --- | --- |

| Title: | Crosscheck of JVET-N0492 (CBF flags signalling in VVC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Philippe Hanhart 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

The document reports the results of crosschecking JVET-N0492 which investigates two aspects of the mismatch between VVC 4 specification draft and VTM4.0 SW concerning CBFs flags signalling:

- Removing the hierarchical chroma CBF signalling based on Transform Unit depth;

- Unifying the design of luma CBF flag derivation for all TU partitioning methods.

The first aspect was cross-checked for the RA and LDB configurations. For RA, all the BD-rates results match exactly with those in JVET-N0492. For LDB, only partial results are available currently, but they match exactly with those in JVET-N0492.

For the second aspect, a problem was found by the proponents, so this aspect was not tested. The proponents sent a patch to fix the issue and the modification looks reasonable.

# Verification

The software used in this cross-check is provided by Huawei, as described in JVET-N0492. The cross-checking tests are conducted based on the VTM-4.0.1 reference software [1] and under the JVET common test conditions [2].

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.32% | 0.21% | #DIV/0! | #DIV/0! |
| Class A2 | 0.01% | 0.11% | 0.00% | #DIV/0! | #DIV/0! |
| Class B | 0.02% | 0.13% | 0.04% | #DIV/0! | #DIV/0! |
| Class C | 0.03% | 0.06% | -0.03% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.15% | 0.05% | #DIV/0! | #DIV/0! |
| Class D | 0.00% | -0.03% | -0.01% | #DIV/0! | #DIV/0! |
| Class F | 0.00% | 0.03% | 0.02% | #DIV/0! | #DIV/0! |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.01% | -0.28% | -0.12% | #DIV/0! | #DIV/0! |
| Class E | -0.01% | -0.51% | 0.30% | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.01% | 0.04% | -0.14% | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# References

VTM-4.0.1 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0.1.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Jan. 2019, Marrakech, MA.