| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0696 |
| --- | --- |

| Title: | Availability based context modelling for mtt_split_cu_vertical_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Wieckowski T. Nguyen H. Schwarz D. Marpe T. Wiegand | Tel: Email: | +49 30 31002 833 adam.wieckowski@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The contribution JVET-N0112 proposes a simplification of the context modelling for the mtt_split_cu_vertical_flag syntax element. The proposed approach reduces the number of context models from five to three. This contribution proposes an alternative context model reduction approach. The authors assert that the proposed method presented in this document is simpler, while showing similar performance.

# Introduction

The current context modelling for the mtt_split_cu_vertical_flag syntax element derives the context model based on split availability and the relation of the neighboring block sizes to the current block size. The proposed modification removes the dependency on the neighboring block sizes, and reduces the number of context models from five to three as the consequence.

# Proposed modeling

The proposed context modeling changes the current context modeling of the mtt_split_cu_vertical_flag in the following manner:

dW 		=  widthC /  widthA,
dH 		=  heightC  /  height,
d 		= 	dW  = =  dH ? 0 : ( dW < dH ? 1 : 2 ), and 
ctxId 	= 	ABH + ATH  = =  ABV + ATV ? 0d : (ABH + ATH < ABV + ATV ? 13 : 24 ).

# Complexity analysis

It is asserted that the availability of the splits needs to be evaluated either way because of the adaptive binarization process of the split coding. The availability based context modelling requires that the availability of all four MTT splits is derived, while without the availability based modeling, in the best case only three of the four availability information need to be derived. On the other hand, with the proposed availability only based context modeling, no neighboring blocks information is required.

# Results

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.05% | 100% | 101% |
| Class A2 | 0.00% | 0.13% | -0.02% | 100% | 102% |
| Class B | 0.01% | 0.02% | 0.00% | 100% | 99% |
| Class C | 0.01% | -0.05% | -0.03% | 100% | 100% |
| Class E | 0.01% | 0.02% | -0.03% | 100% | 101% |
| Overall | 0.01% | 0.03% | -0.01% | 100% | 100% |
| Class D | 0.02% | 0.01% | -0.07% | 100% | 99% |
| Class F | 0.01% | -0.08% | -0.17% | 100% | 102% |
|  |  |  | Random Access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.09% | -0.02% | 100% | 99% |
| Class A2 | 0.02% | 0.01% | 0.02% | 100% | 100% |
| Class B | 0.03% | 0.00% | -0.20% | 100% | 99% |
| Class C | 0.04% | -0.12% | -0.10% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.01% | -0.09% | 100% | 100% |
| Class D | -0.02% | -0.28% | -0.22% | 100% | 100% |
| Class F | -0.04% | -0.15% | -0.06% | 100% | 103% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.04% | 0.16% | -0.21% | 100% | 100% |
| Class E | -0.02% | 0.54% | 1.31% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | 0.65% | -0.17% | 100% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).