| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0697 |
| --- | --- |

| Title: | Crosscheck of JVET-N0301 (Simplification of the initialization process for context variables) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Dong | Tel: Email: | +1 408.533.9515 jiedong@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document reports the cross-check of JVET-N0301. The results provided by the proponent were verified, and the implementation in VTM4.0 was inspected as well.

# Introduction

JVET-N0301 proposes to change the output of the initialization process to the linear domain, which is currently in the logarithmic domain. Also, it proposes to use 6-bit representation for the initialization values. The method is described as below.

m = ( ( initValue >> 3 )  * 3 ) - 12

n  = ( ( ( initValue  &  7 ) * 5 ) << 5 ) + 32

preCtxState = m * qp + n

pStateIdx0 = Clip3( 16, 1008, preCtxState )

pStateIdx1 = Clip3( 1, 16383, preCtxState << 4 )

Software, which is VTM4.0 with the proposed method integrated, was delivered by the proponent. The implementation was inspected and confirmed to be consistent with the description in JVET-N0301. Simulations were conducted following the common test conditions. The results match those provided in JVET-N0301. More details can be found in the attached spreadsheets.