| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0698 |
| --- | --- |

| Title: | Crosscheck of JVET-N0381 (Native CABAC initialization) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Dong | Tel: Email: | +1 408.533.9515 jiedong@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document reports the cross-check of JVET-N0381. The results provided by the proponent were verified, and the implementation in VTM4.0 was inspected as well.

# Crosscheck Results

In the current VVC, the mismatch between the output of initialization process and the input of arithmetic coding engine is fixed by using a look-up table (LUT) for mapping. For the mapping process, JVET-N0381 proposes to replace using the LUT with using equations, as below.

From the 8 bit table entry initValue, two 4-bit variables slopeIdx and offsetIdx are derived as follows:

slopeIdx = initValue  >>  4

offsetIdx = initValue & 15

The variables m and n, used in the initialization of context variables, are derived from slopeIdx and offsetIdx as follows:

m = slopeIdx - 8

n = ( offsetIdx  - 8 ) * 25

The variable preCtxStateIdx is derived from TileGroupQpY, which is derived as provided above, as follows:

preCtxStateIdx = m * Clip3( 0, 51, TileGroupQpY) + n

mps = preCtxStateIdx < 0  ? 0 : 1

preCtxStateIdx = 3 * min(255, mps ? preCtxStateIdx  : –preCtxStateIdx-1)

state = ((255 – (   preCtxStateIdx & 127)) * 32) >> (preCtxStateIdx    >> 7)

state = mps ? 16383 – p : p

The values of pStateL and pStateH are derived as follows:

pStateL = state >> 4

pStateH = state

Software, which is VTM4.0 with the proposed method integrated, was delivered by the proponent. The implementation was inspected and confirmed to be consistent with the description in JVET-N0381. Simulations were conducted following the common test conditions. The results match those provided in JVET-N0381. More details can be found in the attached spreadsheets.