| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0700-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-N0167 (Non-CE4: Advanced multi-hypothesis inter prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 man-shu.chiang@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The document reports the results of crosschecking JVET-N0167 proposed by Hikvision. JVET-N0167 proposes advanced multi-hypothesis inter prediction (AMHP), which is a multi-hypothesis prediction for uni-prediction of the inter mode (that requires to signal motion vector differences). The verification task was done. The PSNR values and bit-rates of anchor and test match those in JVET-N0167. The BD-rate results are slightly different from those in JVET-N0167.

# Verification

Hikvision provided the source code of their proposed method described in JVET-N0167 [1]. One verification was carried out by setting the following parameters in the provided code:

#define HIK_AMHP_TEST_SWITCH                              0

#if HIK_AMHP_TEST_SWITCH

#define HIK_AMHP_FOR_SIMPLIFICATION                       1

#endif

#define HIK_AMHP                                          1

#define HIK_AMHP_MODE_NUM                                 3

#define HIK_AMHP_PRECISION_NUM                            2

The test was conducted under the the common test conditions (CTC) defined in [2] using the VTM4.0 software [3]. Table 1 shows the results using VTM4.0 as the anchor and enabling the proposed method in the test. The PSNR values and bit-rates of anchor and test match those in JVET-N0167. The BD-rate results are slightly different from those in JVET-N0167. Our runtime comparison may be not accurate.

Table 1.  The results of JVET-N0167 using VTM4.0 as anchor

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.05% | 0.01% | 104% | 100% |
| Class A2 | -0.03% | -0.03% | -0.02% | 104% | 106% |
| Class B | -0.05% | -0.02% | -0.05% | 104% | 101% |
| Class C | -0.03% | -0.02% | -0.18% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.01% | -0.07% | 103% | 102% |
| Class D | 0.00% | -0.09% | 0.00% | 101% | 103% |
| Class F | -0.04% | -0.01% | -0.14% | 101% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.15% | -0.20% | 99% | 103% |
| Class C | -0.05% | 0.21% | 0.14% | 98% | 102% |
| Class E | -0.04% | 0.32% | -0.37% | 101% | 106% |
| Overall | -0.02% | 0.09% | -0.13% | 100% | 103% |
| Class D | 0.01% | 0.63% | -0.42% | 103% | 108% |
| Class F | -0.17% | -0.07% | -0.54% | 100% | 104% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.95% | -1.29% | -1.30% | 103% | 100% |
| Class C | -0.66% | -0.96% | -1.00% | 103% | 103% |
| Class E | -0.83% | -1.02% | -0.96% | 100% | 104% |
| Overall | -0.82% | -1.11% | -1.12% | 102% | 102% |
| Class D | -0.49% | -0.47% | -0.47% | 103% | 105% |
| Class F | -0.43% | 0.09% | -0.15% | 102% | 103% |

# References

- X. Xiu, Y.-W. Chen, T.-C. Ma, and X. Wang, “Non-CE4: Advanced Multi-hypothesis Inter Prediction,” Document of Joint Video Experts Team, JVET-N0167.

- F. Bossen, J. Boyce, K. Sühring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

- VTM4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).