| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0709 |
| --- | --- |

| Title: | CE2-related: Computation reductions of interweaved affine mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 80 5066 9075 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes the computation reduction method for interweaved affine mode. The interweaved affine mode maintains the memory bandwidth but increases the motion compensation computation. The problem is the different incensement of MC computation for each block size. This contribution shows that the additional computation and gain are trade-off relation by using checker board pattern. The proposed method is implemented on the top of CE2-1.1 software. The result of Y BD-rate is -0.11% under the random access condition with shorter running time.

Introduction

The interweaved affine mode was proposed and tested in the Core Experiments 2. This method maintains the memory band-width because the fetch area is same between the normal and the interweaved affine motion compensation. At the same time, motion compensation computation is increased up to roughly twice.

The incensement of the interweaved MC blocks depends on the CU size. When the CU is small, the incensement is also small. It means that worst case capability is too much for smaller CUs. The following table is the summary of increase ratio for each CU size. 4x4 means the MC unit.

Table 1 CU size and number of 4x4 blocks by the interweaved method

| CU size | # of 4x4 | # of IW 4x4 | Increase ratio |
| --- | --- | --- | --- |
| 8x8 | 4 | 1 | 125% |
| 16x16 | 16 | 9 | 156% |
| 32x32 | 64 | 49 | 176% |
| 64x64 | 256 | 225 | 188% |

Proposed method

This contribution proposed checker board pattern as the interweaved blocks. It shows the trade-off relation between the additional computation and gains. The following table is the summary of increase ration for each CU size with checker board pattern.

Table 2 CU size and number of 4x4 blocks by the checker board pattern

| CU size | # of 4x4 | # of CB 4x4 | Increase ratio |
| --- | --- | --- | --- |
| 8x8 | 4 | 1 | 125% |
| 16x16 | 16 | 4+1 | 133% |
| 32x32 | 64 | 16+9 | 139% |
| 64x64 | 256 | 64+49 | 144% |

Test 1 Uni-prediction

This test applies the proposed method to the uni-prediction condition, which is same as CE2-1.1 condition.

Test 2 Bi-prediction (under running)

This test applies the proposed method to the bi-prediction condition. The simulation is under running.

Simulation results

The proposed method is implemented on the top of CE2-1.1. A test condition follows the JVET common test condition [2]. The summary of BD-rate is shown in the following table. It is confirmed that the running time is reduced by the proposed checker board pattern.

Table 3 Simulation results of random access and Test 1 condition by the proposed method.

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.04% | -0.02% | 101.2% | 100.4% |
| Class A2 | -0.20% | 0.07% | 0.04% | 101.5% | 100.7% |
| Class B | -0.14% | -0.09% | -0.03% | 101.3% | 99.9% |
| Class C | -0.06% | -0.05% | -0.16% | 101.1% | 99.5% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.02% | -0.05% | 101.3% | 100.1% |
| Class D | -0.15% | -0.10% | -0.03% | 101.1% | 100.4% |
| Class F (optional) | -0.15% | -0.09% | -0.11% | 101.2% | 99.9% |

Table 4 Additional results of random access and Test 1 condition without the proposed method.

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.08% | 0.01% | 101.3% | 101.0% |
| Class A2 | -0.53% | 0.17% | 0.16% | 101.7% | 100.9% |
| Class B | -0.37% | -0.14% | -0.07% | 101.4% | 100.4% |
| Class C | -0.18% | 0.05% | -0.10% | 101.2% | 99.8% |
| Class E |  |  |  |  |  |
| Overall | -0.30% | 0.02% | -0.02% | 101.4% | 100.5% |
| Class D | -0.28% | -0.13% | -0.05% | 101.2% | 100.0% |
| Class F (optional) | -0.53% | -0.27% | -0.23% | 101.3% | 100.5% |

Conclusion

In this contribution, we proposed computation reductions method for interweaved affine mode. This contribution showed that the additional computation and gain are trade-off relation by using checker board pattern. The BD-rate results are -0.11%/-0.02%/-0.05% for Y/U/V under the random access condition. The running time is also reduced.

References

- K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, “CE2-1.1: Interweaved Prediction for Affine Motion Compensation”, JVET-N0261, Geneva, CH, 19–27 Mar. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Marrakesh, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).