| cJoint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0712 |
| --- | --- |

| Title: | CE13-related: Adaptive CNN based in-loop filtering with boundary weights |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Yoshitaka Kidani Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-5066-9075 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposed the online CNN based in-loop filtering. The CNN filter model, which is updated by every intra period based on the encoder side learning, is quantized and signalled as side information. A residual signal is generated by the CNN based filter and is added to the reconstructed signal considering boundary weights. The CTU level switching is also applied. Simulation results reportedly show the BD-rates for luma by X.xx% under the random access condition.

Introduction

Several convolutional neural network-based in-loop filterers were proposed in the recent meetings. CNN model generation is categorized into two types. One is off-line learning and the other is on-line learning. The former means the model is fixed for every sequences. No model signaling is required but model size is large due to support various content types. The latter means the model is varying for each group of pictures like intra period. Model signalling is required but model size is small. From the decoder viewpoint, small model size is reasonable because the complexity is also small.

This contribution focuses on the weight for addition of generated signal by the CNN model. CNN-based in-loop filter can be categorized into smoothing filter. From the PSNR viewpoint, CNN-based in-loop filter is too strong. Outside control is required for provide better performance in both objective and subjective quality.

Proposed weight derivation

The weight is used for sum of residual signals by the CNN filter and the signals after deblocking filter. In the introduction, a benefit of an outside control of CNN model is mentioned. The CNN filter is the same as JVET-N019 [1], but the learning conditions might be different. The fair comparison is difficult under the current situation.

In this contribution, we proposed the boundary based weight derivation. The basic weight is derived from the distance from the boundary. We reuse the boundary definition and its strength derivation from the deblocking filter operation. When the boundary strength is larger than 0, weight is further modified by the block types like intra or inter coding. Figure 1 shows the actual weights for both intra and inter blocks. Thick and thin lines illustrates the block and pixel boundary.

Figure 1 Derived weights for intra (left) and inter (right) blocks when the BS is significant.

Simulation results

The proposed method is implemented on VTM4.0.1. A test condition follows the JVET common test condition [2]. The summary of BD-rate is shown in the following table. It is noted that the CPU implementation is naïve C++ representation. Further optimization can be applied to reduce the running time. Encoding running time does not include the learning time.

Table 1 The simulation results

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.05% | -4.62% | -13.65% | 100.8% | 224.2% |
| Class A2 | -1.06% | -15.80% | -11.43% | 100.5% | 187.6% |
| Class B | -0.49% | -22.87% | -20.71% | 100.6% | 219.8% |
| Class C | -0.12% | -13.15% | -10.05% | 100.6% | 158.2% |
| Class E |  |  |  |  |  |
| Overall | -0.82% | -15.21% | -14.60% | 100.6% | 195.8% |
| Class D | -0.35% | -8.84% | -9.75% | 100.8% | 133.3% |
| Class F (optional) | -0.15% | -0.09% | -0.11% | 101.2% | 99.9% |

Conclusion

This contribution proposed the adaptive CNN-based in-loop filter with boundary based weight. The decoding running time is reasonable even though the CPU implementation. It is requested the proposed method to be studied in the further Core Experiments.

References

- Y.-L. Hsiao, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, and S.-M. Lei, “AHG9: Convolution neural network loop filter,” JVET-M0159, Marrakech, MA, January. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, "JVET common test conditions and software reference configurations for SDR video," JVET-M1010, Marrakesh, MA, 9-18 Jan. 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).