| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0714-v2 |
| --- | --- |

| Title: | Crosscheck of JVET-N0381 (Native CABAC initialization) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Heiner Kirchhoffer Einsteinufer 37 D-10587 Berlin, Germany | Tel: Email: | +49 30 31002 184 heiner.kirchhoffer@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Crosscheck results

This document presents crosscheck results for proposal JVET-N0381. The implementation in the provided software matches the description. The BD rates as reported in version 1 of the proposal were reproduced. The runtime behavior is similar and as expected (given the algorithmic changes).

## Further observations

Custom window size values (shiftIdx) have been modified.

# Experimental results

The following BD rates have been observed by running the provided software. Runtimes are not accurate due to a heterogenous cluster and a varying cluster load.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,02% | 0,09% | -0,06% | 95% | 94% |
| Class A2 | -0,02% | 0,15% | -0,04% | 96% | 94% |
| Class B | -0,01% | -0,09% | 0,01% | 96% | 95% |
| Class C | 0,01% | -0,09% | -0,09% | 97% | 97% |
| Class E | -0,02% | -0,03% | -0,02% | 96% | 95% |
| Overall | -0,01% | -0,01% | -0,04% | 96% | 95% |
| Class D | 0,01% | -0,05% | -0,23% | 97% | 95% |
| Class F | 0,03% | -0,11% | -0,25% | 98% | 97% |

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,03% | -0,07% | -0,06% | 91% | 92% |
| Class A2 | -0,03% | 0,13% | -0,02% | 91% | 93% |
| Class B | -0,02% | 0,01% | -0,10% | 92% | 91% |
| Class C | -0,04% | -0,15% | -0,23% | 93% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0,03% | -0,02% | -0,11% | 92% | 92% |
| Class D | -0,03% | -0,40% | -0,28% | 93% | 93% |
| Class F | -0,01% | -0,18% | -0,14% | 93% | 93% |

|  |  |  | Low Delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,05% | -0,35% | -0,58% | 92% | 92% |
| Class C | -0,13% | -0,02% | -0,42% | 92% | 91% |
| Class E | -0,16% | 1,20% | 0,53% | 89% | 93% |
| Overall | -0,10% | 0,15% | -0,25% | 91% | 92% |
| Class D | -0,09% | 0,29% | -0,97% | 93% | 94% |
| Class F | -0,14% | 0,21% | -0,24% | 91% | 88% |

|  |  |  | Low Delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,05% | -0,25% | -0,72% | 90% | 90% |
| Class C | -0,06% | -0,58% | -0,81% | 92% | 92% |
| Class E | -0,12% | 1,16% | 1,24% | 84% | 87% |
| Overall | -0,07% | -0,01% | -0,26% | 89% | 90% |
| Class D | 0,00% | 0,96% | 0,50% | 94% | 95% |
| Class F | -0,05% | -0,43% | -0,40% | 90% | 86% |