| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0716-v1 |
| --- | --- |

| Title: | CE1-related: Simplified and robust LIC parameter derivation unified with CCLM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Jianle Chen Christian Helmrich, Heiko Schwarz, Detlev Marpe, Thomas Wiegand | Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com jianle.chen@huawei.com christian.helmrich@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de detlev.marpe@hhi.fraunhofer.de thomas.wiegand@hhi.fraunhofer.de |
| Source: | Huawei Technologies Co., Fraunhofer Institute for Telecommunications – Heinrich Hertz Institute |  |  |

_____________________________

# Abstract

This contribution proposes to use CCLM MinMax method to derive linear model parameters for LIC. It is also proposed to improve performance of both LIC and CCLM by calculating average values and slope of linear model with extended set of input data.

It is reported the proposed method reduce the number of sequential computations, simplifies VVC specification draft and reduces hardware complexity.

# Introduction

In all the methods of CE1-5 linear model parameters of LIC (a;b) are derived using LMS method  with the reference and reconstructed samples of the L-shape around the current block. To avoid integer divisions, the number of samples should be a power of 2 so that the divisions are carried out with right shifting. The maximum number of samples used by LIC (in case both sides are available):

N=2×minwidthPU, heightPU

a= N·∑Ln·Cn-∑Ln·∑CnN·∑Ln·Ln-∑Ln·∑Ln

b= ∑Cn-a·∑LnN.

where N is the number of samples, C(n) are neighboring reconstructed samples of the current block and  L(n) is neighboring reconstructed samples  of the reference block.

Predicted samples are updated using the derived LIC parameters (a;b) as follows:

pred'x,y=a×predx,y+b

Calculation of sum of squared values implies certain constraints on the hardware implementation and the input bit depths of samples on which LIC may operate. As proposed in JVET-L0191, instead of determining the straight line from all available luma-chroma sample pairs, this proposal fits the line only between the data point A with the minimum luma value and the data point B with the maximum luma value.

B'

A'

y=α∙x+β

A

B

Fig. 1.	Straight-line fitting in VVC’s simplified CCLM predictor.  Illustration based on that in JVET-L0191.
The red line indicates the adopted approach, while the blue line indicates the proposed improvement.

This reduces the number of operations (particularly multiplications) required by the algorithm, leaving the search for the local luma minimum and maximum as the major operations of the CCLM method. In particular, values of the updating parameters a and b are obtained using the found minimum and maximum values as follows:

a=yB-yAxB-xA

b=yB-a⋅xA

In these formulas, a minimum value () and a maximum value () selected from the samples belonging to an L-shaped template of a reference block. Two corresponding neighboring (reconstructed) samples are selected from the reconstructed template in the positions A and B of corresponding minimum value () and maximum value () of reference samples. Note that the actual CCLM prediction, which uses more operations than the model calculation at medium and large CUs (since it is carried out for each chroma sample in the CU) is not affected by the simplification.

As specified in JVET-M0064, division operation is performed by taking log2() values from nominator and denominator, and using difference and LUT fetch operations to determine the result of the division in terms of multiplier and shift values.

In JVET-M0401, it was suggested to use mean values to increase the accuracy of parameter b:

b=ymean-a*xmean

# Proposal

This contribution suggests using unified mechanism of CCLM parameters derivation in LIC and CCLM. Specifically, it is proposed to derive parameters a and b for LIC using the same derivation process as CCLM does, and to increase the accuracy of parameter b as it is suggested in JVET-M0401 (test CE3-2.5.1).

There are several aspects that are tested in this proposal:

- Template downsampling: Since the MinMax method doesn’t require that the number of processed template samples would be a power of 2, it can be skipped in both cases (i.e. for current and reference blocks).

- Template filtering: filtering can be applied to template to remove outliers using the smoothing FIR filter [1, 2, 1]/4.

- Mean values: mean values ymean and xmean can be calculated by performing averaging separately for each side and both results are averaged with each other.

- Slope value: not only for the smallest luma value A and largest luma value B but also for the second-smallest luma value A' and the second-largest luma value B' are searched as shown in Fig. 1.  Then, the trend line can be fitted between the average of the data points at A and A' (by using (A + A') >>  1) and the average of the data points at B and B' (by using (B + B') >> 1), leading to the blue colored line in Fig. 1, which generally leads to a better data-fit.

Aspect 1 and aspect 2 could be applied to LIC modification. Aspect 3 and aspect 4 are applicable to both LIC and CCLM parts.

Table 1. Summary of the proposed tests

| Test # | Description |
| --- | --- |
| 1 | LIC modification: MinMax method of CCLM with aspects 1-4 applied. CCLM is not modified |
| 2 | CCLM modification: aspects 3 and 4. LIC is not modified |
| 3 | Includes modifications of Test 1 and Test 2 |

# Results

The proposal is implemented using the source code of test CE1-5.1b based on VTM-4.0 software. Results of the Test1 are given in Table 2. Test 1 does not modify intra part. Hence, only RA and LDB results are provided  since AI results coincide with the ones provided by VTM4-0.

Table 2: Results for Test 1 over VTM-4.0.

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.23% | -0.07% | -0.12% | 113% | 100% |
| Class C | -0.21% | -0.33% | -0.17% | 115% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.08% | -0.16% | -0.15% | 113% | 99% |
| Class F | -0.61% | -0.55% | -0.60% | 109% | 100% |
|  | Low Delay B |  |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class E | 0.04% | -0.03% | 0.12% | 101% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.05% | -0.42% | -0.85% | 117% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Test 2 reportedly demonstrates the following performance (Table 3):

Table 3: Results for Test 2 over VTM-4.0.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -0.65% | -0.37% | 100% | 101% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.09% | -0.55% | -0.78% | 99% | 101% |
| Class C | -0.05% | -0.60% | -0.54% | 99% | 98% |
| Class E | -0.03% | -0.46% | -0.65% | 100% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.03% | -0.46% | -0.61% | 99% | 95% |
| Class F | -0.02% | -0.17% | -0.30% | 100% | 101% |
|  | Random Access |  |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.37% | -0.60% | -1.01% | 112% | 100% |
| Class C | -0.31% | -0.77% | -0.69% | 115% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.16% | -0.61% | -0.36% | 113% | 100% |
| Class F | -0.78% | -0.60% | -0.66% | 108% | 101% |
|  | Low Delay B |  |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.43% | -0.08% | -0.46% | 119% | 102% |
| Class E | -0.07% | 0.66% | 0.41% | 101% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.16% | 0.26% | -0.58% | 116% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Results of the Test 3 are given in Table 4.

Table 4: Results for Test 3 over VTM-4.0.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -0.65% | -0.37% | 100% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.09% | -0.55% | -0.78% | 100% | 101% |
| Class C | -0.05% | -0.60% | -0.54% | 99% | 99% |
| Class E | -0.03% | -0.46% | -0.65% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.03% | -0.46% | -0.61% | 99% | 95% |
| Class F | -0.02% | -0.17% | -0.30% | 100% | 100% |
|  | Random Access |  |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.24% | -0.84% | -0.78% | 116% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.08% | -0.47% | -0.38% | 114% | 100% |
| Class F | -0.67% | -0.69% | -0.66% | 109% | 100% |
|  | Low Delay B |  |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.32% | -0.31% | -0.30% | 122% | 100% |
| Class E | 0.02% | 0.32% | 0.41% | 101% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.09% | -0.08% | -0.36% | 119% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# Conclusion

We propose to adopt this contribution to the VVC spec draft and the VTM software.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd and Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).