| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0719r2 |
| --- | --- |

| Title: | Crosscheck of JVET-N0313 (Low latency intra sub-partitions) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check Report |  |  |
| Author(s) or Contact(s): | Rémi Jullian | Tel: Email: remi.jullian@technicolor.com |  |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results for contribution JVET-N0313 (Low latency intra sub-partitions). The simulation results reportedly match those provided by the proponents.

# Introduction

JVET-N0313 proposes a low latency ISP scheme. In the proposed scheme, a sub-partition is predicted using the prediction samples from its previous adjoining sub-partition. Two versions are tested, one with applying this scheme to 8×4 and 4×8 CUs, the other one with applying this scheme to all CU sizes.

Both versions were cross-checked.

# Experimental Results

The following two tests are considered:

- : Low latency ISP applied only to 8×4 and 4×8 CUs, and regular ISP applied to larger CUs.

- : Low latency ISP applied to all CU sizes permitted by ISP.

Tests are conducted for both the AI and RA configurations. AI and RA results for Test (1) are provided in Table 1 and Table 2, respectively, and results for Test (2) are provided in Table 3 and Table 4.

For all the simulations, a nearly perfect match is obtained compared to the proponent results : unique difference has been found for Test (2) ; it concerns RA configuration, Class B, BD-rate PSNR-V (0.04 % vs 0.03 %). The difference was due to using ascii output and not Hex PSNR for reporting the psnr figures.

Eventually, when using Hex PSNR, results perfectly match.

Table 1. AI results for Test (1) where Low latency ISP is applied only to 8×4 and 4×8 CUs.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | 0.00% | 101% | 97% |
| Class A2 | 0.00% | 0.10% | 0.00% | 101% | 98% |
| Class B | 0.00% | -0.01% | 0.05% | 101% | 98% |
| Class C | 0.04% | -0.15% | -0.05% | 101% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 97% |
| Overall | 0.01% | -0.02% | 0.00% | 101% | 98% |
| Class D | 0.02% | 0.02% | -0.06% | 101% | 97% |
| Class F | 0.01% | 0.03% | -0.18% | 100% | 100% |

Table 2. RA results for Test (1) where Low latency ISP is applied only to 8×4 and 4×8 CUs.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.15% | -0.04% | 100% | 99% |
| Class A2 | -0.03% | 0.06% | -0.02% | 100% | 101% |
| Class B | 0.01% | -0.06% | -0.08% | 100% | 100% |
| Class C | 0.03% | 0.02% | -0.01% | 99% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | -0.04% | 99% | 99% |
| Class D | -0.03% | -0.18% | -0.21% | 99% | 96% |
| Class F | 0.02% | -0.16% | -0.20% | 98% | 96% |

Table 3. AI results for Test (2) where Low latency ISP is applied to all CU sizes permitted by ISP.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.09% | 0.08% | 100% | 97% |
| Class A2 | 0.10% | 0.23% | 0.11% | 101% | 98% |
| Class B | 0.13% | 0.15% | 0.16% | 100% | 98% |
| Class C | 0.22% | 0.21% | 0.24% | 100% | 98% |
| Class E | 0.19% | 0.29% | 0.13% | 99% | 97% |
| Overall | 0.15% | 0.19% | 0.15% | 100% | 98% |
| Class D | 0.12% | 0.25% | 0.08% | 100% | 98% |
| Class F | 0.17% | 0.17% | 0.12% | 100% | 100% |

Table 4. RA results for Test (2) where Low latency ISP is applied to all CU sizes permitted by ISP.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.01% | 0.11% | 100% | 100% |
| Class A2 | 0.06% | 0.26% | 0.11% | 100% | 99% |
| Class B | 0.09% | 0.18% | 0.03% | 99% | 100% |
| Class C | 0.11% | 0.15% | 0.04% | 98% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.15% | 0.07% | 99% | 99% |
| Class D | 0.06% | -0.05% | -0.12% | 99% | 96% |
| Class F | 0.05% | 0.07% | -0.06% | 98% | 95% |