| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0724-v1 |
| --- | --- |

| Title: | BoG report on general high level syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill.boyce@intel.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met on 19 Mar 2019 from 17:30 to 10:00.

The BoG recommends the following:

- Adopt a Decoder Parameter Set (DPS), from JVET-N0349 and previously proposed in JVET-M0101

# Contribution Review

### General high-level syntax (31)

#### NAL unit header and general aspects of parameter sets (6)

JVET-N0050 AHG17: On Forbidden-Zero Bit in NAL Unit Header [S. Wenger, B. Choi, S. Liu (Tencent)]

The authors propose removing the “forbidden_zero_bit” from the NAL unit header and re-allocating that bit for other purposes, such as an extension of the NAL unit type numbering space, unless compelling reasons are found and documented that continue to justify the bit’s existence.  We also wonder whether the restriction of the values for the temporal ID in the HEVC NAL unit header, and the complex language regarding SODB/RBSP (section 7.4.2.3 of JCTVC-M1001-v5) can be released/removed, as the authors’ admittedly dim recollection is that such restriction was introduced for essentially the same purpose.

It was suggested that avoiding PES start code emulation was a motivation for the existence of the forbidden_zero_bit in earlier codecs.

JVET-N0067 is also related.

JVET-N0051 AHG17: On NAL Unit header design for VVC [S. Wenger, B. Choi, S. Liu (Tencent)]

This contribution proposes an update to the NAL unit header design for VVC.  At the expense of an—the authors assert insignificant—increase in processing power demands, and certain limitations in extensibility, the NAL Unit header length can be kept at 8 bits.  Extensibility concerns can be addressed by using an extension mechanism similar to the one proposed in JVET-M0520 of the Marrakech meeting.

The reduction of the NAL unit header size from 16 bits to 8 bits results in bitrate savings.  Quantifying those bitrate savings is difficult as they depend highly on the number of tile groups per coded picture, number of SEI messages and marker NAL units per picture, GOP structure and resulting parameter set NAL units for GOP, and so forth.

It was suggested that this proposal could be combined with JVET-N0067 to free up some additional NUT values.

It was suggested that a reserved NUT value could be used to enable an extension of the NUH length, such as to be able to carry layer_id, but it was noted that it couldn’t be used in combination with another NUT value.

JVET-N0067 AHG17: On the first byte of the NAL unit header [M. M. Hannuksela, K. Kammachi-Sreedhar (Nokia)] [late]

The contribution proposes to remove forbidden_zero_bit from the NAL unit header. It is asserted that MPEG-2 Program Elementary Stream (PES) start code emulation is avoided with the proposed syntax element arrangement in the first byte of the NAL unit header. The proposal can be summarized as follows:

- The following first byte of the NAL unit header syntax is proposed:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| zero_tid_required_flag | u(1) |
| nuh_temporal_id_plus1 | u(3) |
| nal_unit_type_lsb | u(4) |
| … // remaining part of the NAL unit header |  |
| } |  |

- NalUnitType is set equal to ( zero_tid_required_flag << 4 ) + nal_unit_type_lsb.

- The assignment of NAL unit type values to types is rearranged. zero_tid_required_flag shall be equal to 1 for DPS, SPS, EOS, EOB, and IRAP NAL units, which therefore have NalUnitType in the range of 16 to 31, inclusive. The proposed number and assignment of reserved NAL unit types differs from that in VVC draft 4.

- When zero_tid_required_flag is equal to 1, nuh_temporal_id_plus1 shall be equal to 1 (i.e., TemporalId shall be equal to 0).

The contribution leaves the discussion of the content of the remaining part of the NAL unit header beyond the first byte, if any, to other contributions. If no decision on the remaining part of the NAL unit header is reached, it is proposed to add one additional reserved bit to nuh_reserved_zero_7bits, i.e. make it nuh_reserved_zero_8bits.

Avoid values of 188 to 255, to avoid PES start code values.

It was noted that with this proposal, all unspecified values would require temporal id equal to 0. It was suggested that using a value as an escape to add extra bytes eliminates the possibility of using the types.

Is it possible to use a bit as an extension? Could an extension use a partial bit instead of the full bit? It was suggested that new NUT values could use the extension.

This proposal allows reclaiming of the temporal id bits for NUT values that don’t need temporal id, e.g. because they are required to have temporal id equal to 0.

The question of temporal sub-bitstream extraction for this contribution was raised.

It was suggested to review the proposals on layer id before making any recommendations.

The basic philosophy of N0067 and N0051 is to be more efficient in the use of NUH values by not requiring simple assignment of contiguous integer bits to syntax elements. There was support for this general concept.

Revisit.

JVET-N0278 AHG17: On VVC HLS relevant to MPEG requirements on immersive media delivery and access (N18134) [J. Boyce (Intel)] [late]

TBP

JVET-N0511 AHG17: Signalling leading picture information in the NAL unit header structure [L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)] [late]

This contribution is a continuation of JVET-N0072 to signal the leading picture related information in the NAL unit header structure. The information can be used by file writer, middle box, or system applications. The suggested syntax and semantics are provided.

Syntax elements in the NUH are proposed to replace reserved bits: num_leading_pictures u(4), and leading_picture_poc_cnt u(5).

The need for including this information in the NUH was questioned. It was suggested that the tile group header might be a more suitable location. The sps_max_dec_pic_buffering syntax element might serve a similar purpose already.

No action.

JVET-N0349 AHG17: On Parameter Set Design for VVC [K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

This contribution proposes to keep following the design principle of using the same picture parameter set for all NAL units of a picture. It also proposes to adopt the decoder parameter set (DPS) that was proposed in JVET-M0101 and discussed in the BoG on high level syntax, but was accidentally missed in track review.

This is relevant to the coded picture region category of combinations. The current VVC Draft 4 design already requires that the same PPS be used for all NAL units of the picture.

The DPS applies to an entire bitstream, beyond a single CVS. It was suggested that the two reserved syntax elements in the syntax could be combined.

It was suggested that syntax elements in the DPS could be included in VUI or SEI. It was noted that contributions at this meeting propose conditional syntax in other NAL units based on constraint flags in the DPS.

BoG recommends to adopt the DPS, as proposed here and in M0101.

#### Reference picture management (4)

JVET-N0058 AHG17: On decoded picture buffer management [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

TBP

JVET-N0100 AHG17: On Reference Picture List Signalling [S. Deshpande (Sharp)]

Modifications are proposed to reference picture list signalling. Following is proposed:

- Proposal 1: It is proposed to allow signalling information about long-term reference picture POC LSB in reference picture list syntax structure or collocated with corresponding long-term reference picture’s delta POC MSB related syntax elements in the tile group header.

- Proposal 2: A conditional signaling is proposed for signaling of num_ref_idx_active_override_flag. An inference rule is proposed when this flag is not signaled.

- Proposal 3: A bug-fix is proposed for calculation of FullPocLt[ i ][ RplsIdx[ i ] ][ j ]. It is asserted that without this bug-fix, the incorrect value of poc_lsb_lt signalled in ref_pic_list_struct( listIdx, rplsIdx ) is used for the calculation of FullPOCLt – which provides full POC for a long term-reference picture.

Proposal 1 saves for long term reference picture list signaling for some conditions. It was questioned if long term reference pictures are even needed anymore. It was questioned if the VTM software encoder has the capability to use long term reference pictures.

It was suggested to consider if signaling in the SPS is necessary.

BoG recommends conditional adoption of Proposal 1 subject to offline review of the text. Revisit to check that text has been reviewed.

Proposal 2 saves a bit in the tile group header in some cases, when override flag isn’t useful.

It was suggested that the condition could be simplified.

Revisit proposal 2 after offline study to simplify the condition if possible.

Proposal 3 is included within Proposal 1.

JVET-N0135 AHG17: On reference picture list [Y. Fujimoto, T. Suzuki (Sony)] [late]

JVET-N0136 AHG17: On non-referenced picture [Y. Fujimoto, T. Suzuki (Sony)] [late]

#### APS and tile group header (8)

JVET-N0065 AHG17 & AHG9: Comments on carriage of coding tool parameters in Adaptation Parameter Set [M. Li, P. Wu (ZTE)]

JVET-N0069 AHG16/AHG17: Proposed Cleanup for Reshaper High Level Syntax [B. Heng, M. Zhou, W. Wan (Broadcom)]

JVET-N0117 AHG17: Signalling of reshaper parameters in APS [Y.-K. Wang, Hendry, J. Chen (Huawei), P. Yin, T. Lu, F. Pu, S. McCarthy (Dolby)]

JVET-N0138 AHG17: Carriage of reshaper model parameters [W. Choi, K. Choi, K. Choi (Samsung)]

JVET-N0284 AHG17: On ALF and Reshaper Signaling [S.Paluri, J. Zhao, J. Lim, S. Kim (LGE)]

JVET-N0290 AHG17: Conditional Signaling of ALF and In-Loop Reshaper Model [S.Paluri, J. Zhao, J. Lim, S. Kim (LGE)]

JVET-N0293 AHG17: Reshaper Reset Signaling [S. Paluri, J. Zhao, J. Lim, S. Kim (LGE)]

JVET-N0297 AHG17: Reshaper Model Signaling for Intra Coded Tile Group [S. Paluri, J. Zhao, J. Lim, S. Kim (LGE)]

#### Interoperability and capability points definition and signalling (1)

JVET-N0276 AHG15: On interoperability point signalling [J. Boyce (Intel)]

#### High efficiency random access (4)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0119 AHG17: EDR - external decoding refresh [Y.-K. Wang (Huawei)] [late]

JVET-N0244 AHG17: Video coding based on cross RAP referencing (CRR) [H. Yu, L. Yu (Zhejiang Univ.)] [miss] [late]

JVET-N0494 AHG17: Dependent random access point pictures in VVC [M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

JVET-N0072 AHG17: New NAL unit types for VVC [L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

The DRAP part of this document belongs to this agenda item.

#### Miscellaneous general HLS topics (8)

JVET-N0061 AHG19: Signalling temporal IDs and levels for HFR backwards compatible bitstreams [V. Drugeon (Panasonic)]

JVET-N0063 AHG17: Separate essential display information from non-essential display information in the Video Usability Information [V. Drugeon (Panasonic)]

JVET-N0120 AHG17: Misc. HLS clean-ups [Y.-K. Wang (Huawei)] [late]

JVET-N0227 Maximum transform size signaling in HLS [L. Li, M. Koo, J. Nam, J. Heo, J. Lim (LGE)] [miss] [late]

JVET-N0234 [AHG12/AHG17] Signaling of virtual boundaries [A. DSouza, C. Pujara, R. Gadde, W. Choi, K. Choi, K.P. Choi (Samsung)] [late]

JVET-N0438 AHG12: Loop filter disabled across virtual boundaries [S.-Y. Lin, L. Liu, J.-L. Lin, Y.-C. Chang, C.-C. Ju (MediaTek), P. Hanhart, Y. He (InterDigital)]

JVET-N0288 AHG17: Comments on High-Level Syntax of VVC [S. Deshpande (Sharp)]

JVET-N0352 AHG17: Conformance Window [K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]