| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0724-v8 |
| --- | --- |

| Title: | BoG report on general high level syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill.boyce@intel.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met on 19 Mar 2019 from 17:30 to 20:00. The BoG met again on 20 Mar 2019 from 09:00 to 13:00. The BoG met again on 22 Mar 2019 from 18:15 to 21:00. The BoG met again on 24 Mar 2019 from 15:00 to 20:10. The BoG met again on 25 Mar 2019 from 11:00 to 18:00. The BoG met again on 26 Mar from 18:20 until 20:00.

The BoG recommends the following:

- Adopt a Decoder Parameter Set (DPS), from JVET-N0349 and previously proposed in JVET-M0101

- Adopt signalling information about long-term reference picture POC LSB in reference picture list syntax structure or collocated with corresponding long-term reference picture’s delta POC MSB related syntax elements in the tile group header, from JVET-N0100

- Adopt loop filtering disabling aross virtual boundaries, from JVET-N0438

- Adopt 5 new constraint flags, from JVET-N0276

- Adopt adding a syntax element in the profile_tier_level( ) syntax structure for sub-profile indication, as specified by ITU-T T.35,

- Adopt a VUI design that contains parameters for aspect ratio, colours description, field seq flag, chroma location, overscan info, and full range flag, as reflected section 4 of in a –v4 version of the document, from JVET-N0063, with the removal of video_format.

- Editorial improvements to sections 8.1.1, 8.1.2, and 8.1.3 to clarify the difference between a CVS and a bitstream, from JVET-N0120

- Adopt syntax for conformance window, following HEVC, from JVET-N0352

- Adopt a CRA decoding processs with support for reference picture lists, from JVET-N0101

- Adopt conditional signaling of num_ref_idx_active_override_flag, with an inference rule, Proposal 2 from N0100

- Adopt APS types, and define APS types for ALF parameters and LMCS (reshaper) parameters, and signal the LMCS APS_ID in the TGH when LMCS is enabled, from option 2 of JVET-N0805.

- Adopt a GRA solution as described in JVET-N0865 v3, with the following characteristics:

- One flag gra_enabled_flag in the SPS that specifies whether GRA pictures may be present or not

- One new VCL NAL unit type, GRA_NUT, that indicates a GRA picture

- One syntax element, recovery_poc_cnt, in the tile group header that is a copy of the recovery_poc_cnt syntax element from the recovery point SEI of HEVC.

- A process to start a CVS with a GRA picture similar to how a CVS is started with a CRA picture

- Adopt copying the HEVC dependent random access indication SEI message (with modifications as appropriate to adapt to VVC), from JVET-N0494

- Adopt restructuring of buffering period and picture timing SEI messages as proposed, except removal of bp_seq_parameter_set_id in buffering period SEI, from JVET-N0353

- Adopt a starting point for HRD design as captured above, from an editorial combination of JVET-N0423 and JVET-N0350, using the most recent version of HEVC as the basis.

- CPB and DPB

- AU but not DU based operation

- Filler data NUT and SEI

- picture timing SEI and buffering period SEI

- Sub-layer DPB parameters in SPS

- VUI HRD parameters (exact location TBD) (revisit once N0353 is reviewed)

- TGH pic output flag

- TGH no_ouptut_of prior_pics_flag for all IRAP

- Sub-layer HRD CPB

- Not include alternative timing at this stage (further study encouraged, especially for potential new NUTs)

- Updated definitions in Section 3

- Adopt decoded picture hash SEI message copied from HEVC, from JVET-N0706.

- Adopt a NUH design which assigns a wider range of NUT values while still avoiding MPEG-2 PES start code emulation while eliminating the forbidden_zero_bit and recapturing the bits allocated to temporal id for NUTs which require temporal id equal to 0, from either JVET-N0823 of JVET-N0066.

  - JVET-N0066 syntax is as follows:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| zero_tid_required_flag | u(1) |
| nuh_temporal_id_plus1 | u(3) |
| nal_unit_type_lsb | u(4) |
| … // remaining part of the NAL unit header |  |
| } |  |

  - JVET-N0823 syntax is as follows:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| nuh_first_octet | u(8) |
| … // remaining part of the NAL unit header |  |
| } |  |

    - nuh_first_octet specifies the type of RBSP data structure contained in the NAL unit and a temporal identifier for the NAL unit as specified in Tables XYZ1, XYZ2, and XYZ3.

- Add an editor’s note to the HRD to note the need to study to consider if there is a bug for the concatenation flag in equation C-10.

- Adopt HRD paramters for temporal scalability for each AU in single picture timing SEI message, without the need for scalable nesting SEI messages, from JVET-N0867.

- Adopt a constraint on dps_id equal to 0, from JVET-N0869.

# Contribution Review

### General high-level syntax (31)

#### 16.18.2.1 NAL unit header and general aspects of parameter sets (6)

JVET-N0050 AHG17: On Forbidden-Zero Bit in NAL Unit Header [S. Wenger, B. Choi, S. Liu (Tencent)]

The authors propose removing the “forbidden_zero_bit” from the NAL unit header and re-allocating that bit for other purposes, such as an extension of the NAL unit type numbering space, unless compelling reasons are found and documented that continue to justify the bit’s existence.  We also wonder whether the restriction of the values for the temporal ID in the HEVC NAL unit header, and the complex language regarding SODB/RBSP (section 7.4.2.3 of JCTVC-M1001-v5) can be released/removed, as the authors’ admittedly dim recollection is that such restriction was introduced for essentially the same purpose.

It was suggested that avoiding PES start code emulation was a motivation for the existence of the forbidden_zero_bit in earlier codecs.

JVET-N0067 is also related.

JVET-N0051 AHG17: On NAL Unit header design for VVC [S. Wenger, B. Choi, S. Liu (Tencent)]

This contribution proposes an update to the NAL unit header design for VVC.  At the expense of an—the authors assert insignificant—increase in processing power demands, and certain limitations in extensibility, the NAL Unit header length can be kept at 8 bits.  Extensibility concerns can be addressed by using an extension mechanism similar to the one proposed in JVET-M0520 of the Marrakech meeting.

The reduction of the NAL unit header size from 16 bits to 8 bits results in bitrate savings.  Quantifying those bitrate savings is difficult as they depend highly on the number of tile groups per coded picture, number of SEI messages and marker NAL units per picture, GOP structure and resulting parameter set NAL units for GOP, and so forth.

It was suggested that this proposal could be combined with JVET-N0067 to free up some additional NUT values.

It was suggested that a reserved NUT value could be used to enable an extension of the NUH length, such as to be able to carry layer_id, but it was noted that it couldn’t be used in combination with another NUT value.

JVET-N0067 AHG17: On the first byte of the NAL unit header [M. M. Hannuksela, K. Kammachi-Sreedhar (Nokia)] [late]

The contribution proposes to remove forbidden_zero_bit from the NAL unit header. It is asserted that MPEG-2 Program Elementary Stream (PES) start code emulation is avoided with the proposed syntax element arrangement in the first byte of the NAL unit header. The proposal can be summarized as follows:

- The following first byte of the NAL unit header syntax is proposed:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| zero_tid_required_flag | u(1) |
| nuh_temporal_id_plus1 | u(3) |
| nal_unit_type_lsb | u(4) |
| … // remaining part of the NAL unit header |  |
| } |  |

- NalUnitType is set equal to ( zero_tid_required_flag << 4 ) + nal_unit_type_lsb.

- The assignment of NAL unit type values to types is rearranged. zero_tid_required_flag shall be equal to 1 for DPS, SPS, EOS, EOB, and IRAP NAL units, which therefore have NalUnitType in the range of 16 to 31, inclusive. The proposed number and assignment of reserved NAL unit types differs from that in VVC draft 4.

- When zero_tid_required_flag is equal to 1, nuh_temporal_id_plus1 shall be equal to 1 (i.e., TemporalId shall be equal to 0).

The contribution leaves the discussion of the content of the remaining part of the NAL unit header beyond the first byte, if any, to other contributions. If no decision on the remaining part of the NAL unit header is reached, it is proposed to add one additional reserved bit to nuh_reserved_zero_7bits, i.e. make it nuh_reserved_zero_8bits.

Avoid values of 188 to 255, to avoid PES start code values.

It was noted that with this proposal, all unspecified values would require temporal id equal to 0. It was suggested that using a value as an escape to add extra bytes eliminates the possibility of using the types.

Is it possible to use a bit as an extension? Could an extension use a partial bit instead of the full bit? It was suggested that new NUT values could use the extension.

This proposal allows reclaiming of the temporal id bits for NUT values that don’t need temporal id, e.g. because they are required to have temporal id equal to 0.

The question of temporal sub-bitstream extraction for this contribution was raised.

It was suggested to review the proposals on layer id before making any recommendations.

The basic philosophy of N0067 and N0051 is to be more efficient in the use of NUH values by not requiring simple assignment of contiguous integer bits to syntax elements. There was support for this general concept.

Discussed again after track review of BoG report about JVET-N0823 recommendation.

This contribution has fewer reserved NUTs than N0823 would enable.  There are still about 13 total NUT values remaning if the different categories are added together, plus 4 unspecified. We may want to adjust the ranges of VCL and non-VCL values at a later time if additional NUT values are assigned.

In this contribution, the 3 bits allocated to TID are still retained as fixed bit fields, but are sometimes required to be equal to 1.

The BoG recommends to adopt either this contribution or N0823.

JVET-N0823 AHG17: First octet of NAL unit header without forbidden_zero_bit [S. Wenger, B. Choi (Tencent), M. M. Hannuksela, K. Kammachi-Sreedhar (Nokia)] [late]

This contribution is a hybrid between JVET-N0051 (Tencent), JVET-N0067 (Nokia), as well as ideas and opinions captured duing the discussion in the HLS Breakout meeting that took place March 19 afternoon and March 20, and also addresses the concerns voiced in JVET-N0050 (Tencent).  Specifically, proposed is

a) to replace the forbidden_zero_bit as the mechanism to avoid MPEG-2 stream layer PES start code emulation with the mechanism aligned with the spirit of JVET-N0067 (basically disallowing values for the first octet of the Nal Unit header between 188 and 255);

b) using the bits previously employed for temporal ID (TID) as a demultiplexing point for those NAL unit types where temporal ID is meaningless, according to JVET-N0051;

c) creating NAL unit types for reference and non-reference trailing and STSA picture NAL units (according to real-time discussions in the HLS break-out, March 20 in the morning). The semantics of the non-reference trailing and STSA NAL units are left outside of the scope of this contribution.

This document focusses on syntax and semantics on the proposed solution; for justification and discussion please see the aforementioned documents and the HLS breakout meeting reports.  Note that the documents assumes the APS having a meaningful TID, and it also proactively reserves NAL unit types for VPS and DPS even if those are not yet agreed.

Assigned so far are 10 NAL unit types (NUTs) where the TID matters, (reference/non-reference trailing/STSA NUTs, RASL/RADL NUTs, AUD, APS, prefix and postfix SEIs),  9 NUTs where TID does not matter (parameter sets, markers).  Further we propose to set aside 8 codepoints NUTs as “unspecified” for external use.

Proposed herein is the interpretation of the first octet of the NAL unit header (NUH).  Whether or not a second or more octets in the NUH are necessary for all or some NUHs is left open.

It was suggested that this design doesn’t include a gating mechanism for an optional second byte. An example of how an escape code could be used was discussed using a NUT value, that would add two extra bits. It was suggested that there could be two different escape codes, one code for a one byte extension and another code for a two byte extension.

Several minor editorial issues were identified in the provided text.

It was commented that any software implementation of this would also require a change to the parcat executable.

Contribution includes additional NUTs for TRAIL_N and STSA_N, for non-reference, but didn’t clearly define what its behavior should be.

BoG recommends to adopt, with the removal of the TRAIL_N and STSA_N and VPS values, which will be further studied, to be reflected –v2 version.

Further discussed following track review.

The following changes were implemented in version 2 of the contribution:

- A range of reserved VCL NAL unit types was assigned to IRAP VCL NAL unit types, and another range was assigned to non-IRAP VCL NAL unit types.

- Non-reference trailing and STSA picture NAL unit types as well as VPS NAL unit type were excluded, as agreed in the HLS BoG review of version 1 of the contribution.

- Editorial changes were done, with the intent to align to the style used in the VVC specification.

- VVC Draft 4 specification text on nal_unit_type and nuh_temporal_id_plus1 was incorporated as much as possible. For example, informative notes were carried over. The proposed text is now asserted to be suitable for replacing the entire semantics of nal_unit_type and nuh_temporal_id_plus1.

JVET-N0278 AHG17: On VVC HLS relevant to MPEG requirements on immersive media delivery and access (N18134) [J. Boyce (Intel)] [late]

Reviewed in the BoG on coded picture regions.

JVET-N0511 AHG17: Signalling leading picture information in the NAL unit header structure [L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)] [late]

This contribution is a continuation of JVET-N0072 to signal the leading picture related information in the NAL unit header structure. The information can be used by file writer, middle box, or system applications. The suggested syntax and semantics are provided.

Syntax elements in the NUH are proposed to replace reserved bits: num_leading_pictures u(4), and leading_picture_poc_cnt u(5).

The need for including this information in the NUH was questioned. It was suggested that the tile group header might be a more suitable location. The sps_max_dec_pic_buffering syntax element might serve a similar purpose already.

No action.

JVET-N0349 AHG17: On Parameter Set Design for VVC [K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

This contribution proposes to keep following the design principle of using the same picture parameter set for all NAL units of a picture. It also proposes to adopt the decoder parameter set (DPS) that was proposed in JVET-M0101 and discussed in the BoG on high level syntax, but was accidentally missed in track review.

This is relevant to the coded picture region category of combinations. The current VVC Draft 4 design already requires that the same PPS be used for all NAL units of the picture.

The DPS applies to an entire bitstream, beyond a single CVS. It was suggested that the two reserved syntax elements in the syntax could be combined.

It was suggested that syntax elements in the DPS could be included in VUI or SEI. It was noted that contributions at this meeting propose conditional syntax in other NAL units based on constraint flags in the DPS.

BoG recommends to adopt the DPS, as proposed here and in M0101.

#### 16.18.2.2 Reference picture management (4)

JVET-N0058 AHG17: On decoded picture buffer management [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

The contribution proposes the following items to improve the decoded picture buffer management, by addressing the temporal sub-layer adaptation:

Proposal 1: TRAIL_NUT, STSA_NUT, RASL_NUT and RADL_NUT are re-specified as (TRAIL_N, TRAIL_R), (STSA_N, STSA _R), (RASL_N, RASL_R) and (RADL_N, RASL_R) to indicate non-referenced pictures in the same sub-layer. Accordingly, non-referenced pictures having the temporal ID equal to that of the current picture are included in the RefPicList[ i ].

Proposal 2: sps_max_dec_pic_buffering_minus1 is signaled for each highest temporal id in SPS.

Proposal 3: The list of unused reference pictures for each highest temporal id is signaled in tile group header.

Proposal 4: All reference pictures with TemporalId greater than HighestTid are marked as "unused for reference", when the value of HighestTid is not equal to sps_max_sub_layers_minus1.

It was noted that the proposed definition of non reference pictures differs from what HEVC used, where it was a sub-layer non-reference picture.

It was suggested that it is limiting to restrict all references, rather than just sub-layer non-reference.

It was suggested to only provide separate NUT values for TRAIL.

It was suggested to use a fixed position in the tile group header to indicate non-reference, such as the discardable_flag in HEVC’s slice header.

Should a non-reference indication be per sub-layer or global?

It was suggested that if tile group header signaling were used, there would be room to include indication for both global and per sub-layer.

So far, we are using 16 of the 32 NUT values in the design (counting a new one for DPS).

Proposal 2 is HRD related, and should be considered in that discussion.

An issue with the syntax of Proposal 3 was raised.

For Proposal 4, it isn’t clear that the specification would need to change to realize the asserted benefit.

Further discussed 24 Mar to review –v5 version with revision to Proprosal 1. Proposes redefinition of TRAIL_N and TRAIL_R for non-reference and reference.

Proposing a definition of TRAIL_N

Should the TRAIL_N and STSA_N non-reference indication mean that it is global non-reference or sub-layer non-reference?  One option would be add separate types for global and sub-layer non-reference, although we had been attempting to minimize the number of NUTs. Flags in the the tile group header is another option to indicate non-reference.

For STSA_N, should be sub-layer non-reference.

For further study.

JVET-N0100 AHG17: On Reference Picture List Signalling [S. Deshpande (Sharp)]

Modifications are proposed to reference picture list signalling. Following is proposed:

- Proposal 1: It is proposed to allow signalling information about long-term reference picture POC LSB in reference picture list syntax structure or collocated with corresponding long-term reference picture’s delta POC MSB related syntax elements in the tile group header.

- Proposal 2: A conditional signaling is proposed for signaling of num_ref_idx_active_override_flag. An inference rule is proposed when this flag is not signaled.

- Proposal 3: A bug-fix is proposed for calculation of FullPocLt[ i ][ RplsIdx[ i ] ][ j ]. It is asserted that without this bug-fix, the incorrect value of poc_lsb_lt signalled in ref_pic_list_struct( listIdx, rplsIdx ) is used for the calculation of FullPOCLt – which provides full POC for a long term-reference picture.

Proposal 1 saves for long term reference picture list signaling for some conditions. It was questioned if long term reference pictures are even needed anymore. It was questioned if the VTM software encoder has the capability to use long term reference pictures.

It was suggested to consider if signaling in the SPS is necessary.

BoG recommends adoption of Proposal 1.

Proposal 2 saves a bit in the tile group header in some cases, when override flag isn’t useful.

It was suggested that the condition could be simplified.

BoG recommends adoption of proposal 2 as reflected in the –v3 version.

Proposal 3 is included within Proposal 1.

JVET-N0135 AHG17: On reference picture list [Y. Fujimoto, T. Suzuki (Sony)] [late]

In the current VVC design, ref_pic_list_idx and ref_pic_list_struct are signaled in tile_group_header. However, when more than one tile_group exist, the same value must be duplicated in each tile group. When ref_pix_list_struct is signaled, only long term reference is updated in many cases. However, in the current design, both long term and short term references must be updated. When more than one tile_group exist, whole set of long term and short term must be duplicated in all tile_group. It is redundant.

This contribution proposes 1) new NAL unit to send parameters frequently changed, e.g. ref_pic_list_idx and ref_pic_list_struct. The parameters in this NAL unit are not changed within AU. 2) the syntax change to update long term reference picture list, which is defined in ref_pic_list_struct in SPS. Only long term reference can be updated in AU.

It was suggested that there are some advantages to allowing the flexibility of list orders in different tile group.

It was suggested that the second identified problem can be addressed by JVET-N0100 Proposal 1.

JVET-N0136 AHG17: On non-referenced picture [Y. Fujimoto, T. Suzuki (Sony)] [late]

In the current VVC design, there is no syntax to indicate whether the current picture is reference picture or non-reference picture. If there is no such syntax element at higher layer of syntax, it is hard to identify if the current picture can be discarded. This contribution proposes to include a flag to identify whether current picture is non-reference picture. Three methods are proposed.

It was suggested that if the second option of adding ref_pic_flag to the AUD, were used with multi-layer coding, wouldn’t allow different values for different layers.

It was commented that the HEVC design of nal_ref_idc indicated referencing or not within a temporal sub layer. HEVC has NUT values that distinguish between reference and non-reference.

It was suggested that a fixed position in the tile group header could be used for this purpose.

It was suggested that temporal_id could be used for this purpose.

It was suggested that the two trailing picture NUT values be used to differentiate between reference and non-reference.

The contribution asserts that in HEVC if temporal sub-layers are used, the HRD parameters must be sent per sub-layer. This could be changed for VVC.

JVET-N0058 was suggested to be related.

#### 16.18.2.3 APS and tile group header (8)

JVET-N0065 AHG17 & AHG9: Comments on carriage of coding tool parameters in Adaptation Parameter Set [M. Li, P. Wu (ZTE)]

In 13th JVET meeting, Adaptation Parameter Set (APS) has been introduced into Versatile Video Coding (VVC) standard text. A few years ago, the APS was once adopted into High Efficiency Video Coding (HEVC) standard for carrying coding tool parameters, such as Adaptive Loop Filter (ALF) parameters; but the APS was removed from HEVC standard along with the removal of ALF as a coding tool at a final HEVC standardisation stage. The APS was designed to carry coding tool parameters as a picture level adaptive nature and alternatively the APS data can also be maintained as unchanged for the whole video sequence for avoiding resending them unnecessarily. This contribution is for information only and it is commented and recommended to use the APS to carry the coding tool data, such as neural networks parameters and affine linear weighted intra prediction parameters, etc., if some associated coding tools are indeed to be adopted into VVC standard. If more coding tool data need to be carried by APS in the future, it commented that further study on updating parameters of multiple tools using APS would be needed.

The contribution suggests that the APS would be an appropriate place to signal parameters for a CNN loop filter, if such a tool were adopted.

Contribution is for information.

JVET-N0069 AHG16/AHG17: Proposed Cleanup for Reshaper High Level Syntax [B. Heng, M. Zhou, W. Wan (Broadcom)]

The following document discusses potential issues with the high-level syntax for the reshaper tool. It is asserted that the current syntax has undefined behavior, and that it would cause issues for random access, temporal scalability, and trick-modes (fast-forward, reverse, etc…).

This contribution proposes to:

- Define the behavior when the reshaper is enabled but a model is not present in a tile-group.

- Add a default reshaper model in the SPS.

- Send additional reshaper syntax only if the reshaper is actually enabled for a tile-group.

The current design includes reshaper parameters in the tile group header. The reshaper parameters signaled in the tile group header are used by other pictures, which is not consistent with our usual HLS design principles, which would use a parameter set for signaling of parameters.

This contribution proposes including default reshaper parameters in the SPS. It was also suggested that the PPS might be a more appropriate location. It may be possible to include them in more than one parameter set location, with an override in the tile group header.

There is agreement that default reshaper parameters should be sent in a parameter set rather than propagated from tile group header.

The size of reshaper parameters is approx. 100 bits.

JVET-N0117 AHG17: Signalling of reshaper parameters in APS [Y.-K. Wang, Hendry, J. Chen (Huawei), P. Yin, T. Lu, F. Pu, S. McCarthy (Dolby)]

This contribution asserts that the properties of the reshaper parameters can be characterized as follows:

- The size of a set of reshaper parameters contained in the tile_group_reshaper_model( ) syntax structure is typically around 60 to 100 bits for video coded with a 10-bit internal bit depth.

- The reshaper model is typically updated by the encoder about once per second.

- Within a bitstream, many sets of reshaper parameters may be applied.

Based on the above asserted properties of the reshaper parameters, it is asserted that signalling of the reshaper parameters in the APS is more desirable than in any of the following: SPS; PPS; picture header (if it exists); and tile group header, and therefore it is proposed to signal the reshaper parameters in the APS.

In addition, to avoid the partial-update problem, it is also proposed that an APS NAL unit shall not contain more than one type of parameters; i.e., either ALF parameters or reshaper parameters, never both.

In the CTC for RA, the reshaper parameters are signaled once per random access period.

It is asserted that there would be a coding efficiency benefit to adjusting reshaper parameters more frequently, but no experimental results are available.

In the contribution, an APS type is proposed, which can be used for ALF and reshaper parameters. More than one aps_id can be signaled in the tile group header, based on which tools are enabled.

The design currently allow 32 values of aps_id. If this contribution were adopted, we might want to increase the number.

Some semantic constraint would be needed to disallow signaling of aps_id in the TGH for the wrong type of APS.

The proposal creates different APSs without the need to use additional NUT values.

This concept could be further extended for other APS types, such as for CNN loop filters, if they were adopted.

JVET-N0138 AHG17: Carriage of reshaper model parameters [W. Choi, K. Choi, K. Choi (Samsung)]

Similar in design to JVET-N0117. Proponent considers the JVET-N0117 design able to address the identified problem. No presentation of this contribution requested.

JVET-N0284 AHG17: On ALF and Reshaper Signaling [S.Paluri, J. Zhao, J. Lim, S. Kim (LGE)]

This contribution proposes a design of the Adaptation Parameter Set (APS) for the carriage of Adaptive Loop Filter (ALF) and In-Loop reshaper model parameters. In the latest VVC WD, the ALF parameters are signaled in the APS while the reshaper model is signaled in the tile group header. This contribution proposes to signal in-loop reshaper in the APS. It is considered beneficial to signal both these parameters in the APS.

Proposes signaling default reshaper parameters in SPS (in N0293), and update reshaper parameters in the APS. Conditionally signal a reshaper aps_id in the TGH based on a flag.

JVET-N0290 AHG17: Conditional Signaling of ALF and In-Loop Reshaper Model [S.Paluri, J. Zhao, J. Lim, S. Kim (LGE)]

This contribution proposes a design of the Adaptation Parameter Set (APS) for the carriage of Adaptive Loop Filter (ALF) and In-Loop reshaper model parameters, while still being present conditionally in the tile group header. This is advantageous because, it can provide the flexibility of partial updates of the ALF parameters and the in-loop reshaper model either through the APS or through the tile group header itself. A new flag proposed in the tile group header is used to control if the tile group should refer to an APS or would carry its own ALF and in-loop reshaper model.

JVET-N0293 AHG17: Reshaper Reset Signaling [S. Paluri, J. Zhao, J. Lim, S. Kim (LGE)]

This contribution proposes a design for the reset of the In-Loop reshaper model at Intra Random Access Point (IRAP) picture. The In-Loop reshaper model is conditionally present in the tile group header. In the existing design, the reshaper parameters are controlled by two flags, i.e., tile_group_reshaper_model_present_flag and tile_group_reshaper_enable_flag, which indicate whether the reshaper model is present in the current tile group and whether it is to be used in the current tile group. These two flags work independently of one another. During an intra refresh, the model may not be present, however the model use may be enabled for the current and subsequent tile groups. This may not be a desirable option at a random access point and may cause dependency across random access point. Therefore, it is proposed to reset the reshaper to a default reshaper at IRAP picture if reshaper is not present. The default reshaper can be linear mapping or it can be signaled in the Sequence Parameter Set.

JVET-N0297 AHG17: Reshaper Model Signaling for Intra Coded Tile Group [S. Paluri, J. Zhao, J. Lim, S. Kim (LGE)]

This contribution proposes to signal the use of an In-loop reshaper model for an intra coded tile group. In particular it imposes the condition that a reshaper model be present in the intra coded tile group header and allows the reshaper model to be optionally present in other tile group headers.

Overall discussion:

APS has the same temporal id as the AU which it is sent in.

Need to have a restriction that can’t reference an aps_id which had a higher temporal id value than the current picture temporal id. Check to see if the spec text has any language for that.

It was noted that parameter set activation text is missing in VVC Draft 4.

It was noted that the current design uses a fixed length code for the aps_id, which may be sub-optimal.

Suggested solution:

- optionally send reshaper parameters in the SPS

- define APS types for ALF and reshaper parameters. Each APS has only one type.

- can send reshaper parameters in APS

- if reshaper tool enabled, have a flag in the TGH to indicate whether or not reshaper aps_id is present. If not signaled, the SPS parameters are used.

  - need to add semantic constraint to always have something valid that is referred to when tool enabled

Proponents are asked to submit a late contribution to implement this solution. Hendry to coordinate.

JVET-N0805 AHG17: Design for signalling reshaper model [B. Heng, M. Zhou, W. Wan, P. Yin, Hendry, S. Paluri, J. Zhao, J. Lim, S. Kim, W. Choi, K. Choi, K. Choi (multi-company ??)] [miss] [late]

Further discussed after track level discussion. The notes from Track A are as follows:

“From the discussion in track A, it was suggested that it would be better to avoid sending LMCS parameters in SPS. This can easily be resolved by specifying that at least one corresponding APS has to be available when LMCS is enabled.

Proponents are asked to update the proposal accordingly.”

BoG recommends to adopt option 2, to send the reshaper APS in the TGH.

#### 6.18.2.4 Interoperability and capability points definition and signalling (1)

JVET-N0276 AHG15: On interoperability point signalling [J. Boyce (Intel)]

Chaired by S. Deshpande

1. Five new constraint flags are proposed, primarily for tools adoptions at the prior meeting, and small modifications of existing constraint flags to align with renaming or syntax reordering in the VVC Draft 4 specification.

This part is recommended by BoG to be adopted.

2. It is also proposed to have the presence of tool enable flags in the SPS, PPS, and tile group header be conditioned on the value of the corresponding constraint flags in the general_constraint_info( ) syntax structure.

It was pointed by proponent that this results in parsing dependency between parameter sets.

And in rewriting case there may be union needed.

It was commented that this might unnecessarily increase test vectors and parameter parsing dependency is an issue.

No action on this point.

3. Additionally, it proposed to add a syntax element in the profile_tier_level( ) syntax structure for sub-profile indication.

The semantics for this are as follows:

general_sub_profile_idc indicates interoperability metadata registered as specified by X Recommendation ITU-T T.35, the contents of which are not specified in this Specification.

It was commented that previously there was a comment that 4 bytes might be considered instead of 3 bytes.

It was asked if there is a way not tos end this info if not needed. It was commented that this is in DPS and parmeter sets dont need to be transmitted in-band.

It was asked if this may lead to – too many sub-profiles. It was answered that there is anyway nothing to preclude it.

It was commented that decoder generally cares about constraint flags instead of sub-profile.

This part is recommended by BoG to be adopted.

#### Inloop filter disabling

JVET-N0234 [AHG12/AHG17] Signaling of virtual boundaries [A. DSouza, C. Pujara, R. Gadde, W. Choi, K. Choi, K.P. Choi (Samsung)] [late]

The in-loop filters in a conventional video coding system are used to reduce undesired artifacts and enhance coding efficiency. For a 360° video, the layout for a specific projection format can contain one or more discontinuous edges. Applying in-loop filters across these discontinuous edges directly may result in poor visual quality and coding efficiency because of the pixels involved (to be referenced and/or filtered) across discontinuous edges are jointly processed (filtered).

This contribution demonstrates the need to disable in-loop filters that not only span across the entire frame but also at arbitrary discontinuous edges within a picture. An alternate frame packing arrangement is demonstrated in JVET-N0233 that shows better compression efficiency as compared to the current frame packing method. The discontinuous boundaries in the alternate frame packing arrangement do not span across the entire picture. To address such problems, it is proposed to disable the in-loop filters at virtual boundaries at tile group level in the picture.

Each tile group can signal up to 2 vertical and two horizontal boundaries. First the CTU containing a boundary is signaled. Each CTU can signal one vertical and one horizontal boundary at which not to do inloop filters, at arbitrary minCbY aligned positions.

This approach doesn’t require that boundaries span the entire picture, but supports other patterns. An example was provided for an alternative layout of cube faces.

Some OMAF formats have boundaries that do not span across the entire picture.

If the boundary was aligned with a tile boundary, the signaling would be for the later tile.

Parameters are signaled in the PPS.

It was suggested that a decoder would need to either store a table at the lowest granularity, or the condition would need to be checked at each block position.

JVET-N0438 AHG12: Loop filter disabled across virtual boundaries [S.-Y. Lin, L. Liu, J.-L. Lin, Y.-C. Chang, C.-C. Ju (MediaTek), P. Hanhart, Y. He (InterDigital)]

The in-loop filters in a conventional video coding system are used to remove undesired artifacts and enhance image quality. For a 360° video, the layout for a specific projection format may have one or more discontinuous edges. Applying the in-loop filters on these discontinuous edges directly may result in poor visual quality and decreased coding efficiency because the accessed pixels (to be referenced and/or filtered) across discontinuous edges are jointly processed (filtered). To address this problem, it is proposed to disable the in-loop filters across vertical and/or horizontal virtual boundaries in the picture.

Experimental results provided for an example with one virtual boundary at a 360 video face row boundary. Still images with subjective differences show reduction of subjective artifacts across boundaries.

Allows disabling in-loop filters at 8 pixel granularity. The boundaries must span the entire picture either vertically or horizontally. Parameters are signaled in the SPS.

Storage may be able to be done with two 1-D arrays rather than a 2-D array as in JVET-N0234.

Proposal limits to 3 horizontal and 3 vertical boundaries per picture.

BoG recommends to adopt, with inclusion of an additional semantic constraint of at most one horizontal and one vertical boundary per CTU .

JVET-N0275 AHG14/AHG8/AHG17: Sections for intra refresh and inloop filter disabling [J. Boyce, R. Lei, L. Xu (Intel)]

Chaired by S. Deshpande.

A capability is proposed to divide a picture into two sections:  a special section, and a remainder section of the picture outside of the special section. The special section can be flexibly used for any of several encoder-selected features applied within the special section or at the boundaries between the special section and the remainder section, including intra coding, QP adaptation, or disabling of in-loop filters. The proposed section boundaries would not be associated with tiles, and the mapping would be signaled in the PPS or tile group header at an encoder-selected granularity as small as MinCbSizeY. The regions of the picture contained within the special section need not be contiguous. This general purpose solution can be used for a variety of use cases, including progressive intra refresh, 360 video coding with cube maps, and region-of-interest coding, and could potentially be extended for other use cases.

It was asked if this is bit efficient way to signal such info. Proponent explained that bit counts for examples are provided.

It was commented that for progressive intra refresh part, this contribution is not proposing normative gradual decoding refresh decoding process but rather signaling to help enable such a decoder to operate in that mode.

A software implementation is not done yet.

It was commented that ramifications about a new partitioning scheme such as this should be carefully considered and some way to bound it and bound complexity will be useful. There is also some storage requirement.

For in-loop filtering aspect it was asserted that the complexity will be somewhat similar to N0234 proposal.

It was asked if it is like fine grained tiles. It was answered that there are no tiles in this proposal.

It was commented that since there is only one partitioning, at any time this signaling can only enable one use case (e.g. disabling of in-loop filters for cubemap layout could not be done at the same time as progressive intra refresh).

It was commented than no normative action is specified for clean versus dirty areas, it only signals VUI type indicators.

It was remarked than this can enable an efficient encoder support for intra-refresh same way as existing implementation.

It was asked by proponent if having the additional flexibility such as this for specifying virtual boundaries is better or having specific solutions is preferred.

-It was answered that it is a bit unclear if we need additional flexibility. This comment is not really specific to this proposal but about what are our underlying needs for each of those problems to be solved. It was commented that for disabling in-loop filtering at non-tile boundaries is well known issue that this can help solve.

-It was commented that it is a bit difficult to answer until we know how many such solutions we need to support.

-It was commented that MinCbSize granularity is needed for all the features, e.g. for some of the features CTU granularity may be sufficient.

About further study of this following aspects may be useful to study:

1. Consider use cases, finding out what is the optimal solution  for them

2. Or finding a particular use case that this is a good solution for.

Further Study encouraged.

In loop filtering disabling discussion:

It was suggested that any inloop filter disabling be consistent with the design for tile group boundaries.

It was suggested to limit to have only one vertical and only one horizontal boundry per CTU.

#### 6.18.4 Low latency random access (10)

JVET-N0072 AHG17: New NAL unit types for VVC [L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

The GDR part of this document was first discussed in the track.

This contribution is a part of continuation of JVET-M0161-v2. The new network abstraction layer (NAL) types of GDR_NUT and DRAP_NUT are proposed for the NAL unit header. The corresponding properties of gradual decoding refresh (GDR) picture and dependent random access point (DRAP) picture follow up those of HEVC. The suggested syntax and semantics are provided.

It was suggested that DVB mandates the use of the HEVC SEI message for DRAP indication. This was used by DVB for an issue with a constraint on ordering of leading and trail pictures, could be possibly addressed with a relaxation of the constraint.

Would make the NUH different lengths.

N0494 and N0495 are related.N0119 may also be related.

Discuss with the related contributions.

For GDR, proposes a NUT value. This would be in place of the HEVC recovery point and refreshed region SEI messages. It was suggested that higher layers such as Systems may wish to use this information.

It was suggested that SEI could be used instead of a NUT if normative decode behavior not defined.

JVET-N0101 AHG14/ AHG17: On Gradual and Clean Random Access [S. Deshpande (Sharp)]

This document provides proposals related to gradual random access and clean random access. Following is proposed:

- Proposal 1: Signalling is proposed for a gradual random access (GRA) picture related information and to allow support for random access to be performed at a GRA picture. Two alternative syntax options are proposed for this.

- Proposal 2: Decoding process for generating unavailable reference pictures is proposed. This process is proposed to be invoked for decoding of CRA picture with NoRaslOutputFlag equal to 1. Additionally if GRA functionality is agreed then it is also proposed to be invoked for GRA picture decoding.

Doesn’t change the NUH length. Proposes changing normative decoding process.

Gives the encoder flexibility to decide later when the picture is fully clean.

Changes to the normative decoder process would define conforming output during the recovery period, and the bitstream would be considered to be conforming, which was asserted to affect interoperability. HRD, DPB behavior may be impacted.

Having even infrequent I frames impacts low latency operation. Some encoders never use I frames except for at scene changes.

Option 2 proposes a flag in the tile group header, conditioned by a PPS flag. It follows variable length coded syntax elements, which may make it harder to change.

N0115 is related.

Proposal 2 proposes a normative CRA decoding process, similar to what was in HEVC but updated for reference picture lists, and also extending for GRA.

BoG recommends adoption of a CRA decoding process, with support for reference picture lists, without include the seting of the value of PicOutputFlag.

JVET-N0495 AHG14: Recovery point indication NAL unit [R. Sjöberg, M. Pettersson, M. Damghanian (Ericsson)]

This contribution proposes to add a recovery point indication NAL unit to the VVC specification, using a new non-VCL NAL unit type. The position of the recovery point indication NAL unit in the bitstream specifies the start of the recovery point period. The last picture of the recovery point period, the recovery point picture, is specified by a recovery_poc_cnt UVLC code word. The recovery point indication NAL unit syntax also contains information for generating the unavailable reference pictures for the recovery point. It is proposed that a coded video sequence (CVS) may start with the access unit containing the recovery point indication NAL unit, using mechanisms similar to how a CRA picture may start a CVS. It is further proposed that starting the decoding at a recovery point is supported through the following process:

- decode a recovery point indication NAL unit that defines a recovery point begin (RPB) access unit, the start and end picture of the recovery point period and the list of reference pictures

- generate unavailable reference pictures from syntax elements in the recovery point indication NAL unit

- decode the recovery point begin picture and the pictures that follow it in decoding order

- optionally suppress output of the recovery point begin picture and all pictures that follow it in decoding order until the last picture of the recovery point period is decoded

This proposal is a follow-up contribution of JVET-M0529. In contrast to JVET-M0529 this contribution proposes to generate unavailable reference pictures directly from syntax in the recovery point indication NAL unit instead of waiting for the RPL signalled in the tile group header.

It was asked why the reference picture information needs to be in the recovery point indication rbsp, and why it couldn’t use the RPL info? This would require an additional reference picture marking process.

Proposed to be a non-VCL NAL unit.

Can an application specification mandate the use of the recovery point SEI as an alternative?

This contribution makes this type of bitstream conforming, which is not the case with the current design.

This contribution has more changes to the design than the earlier contribution in JVET-M0529.

General discussion:

Several options were discussed:

- Keep SEI message

- Add NUT

- Normative CVS GDR operation

- Low level tools changes

It was remarked that the recovery point SEI message is commonly used now, although it is also sometimes not present in a bitstream when it should be.

It was suggested that having a NUT for recovery point could be useful, even for non-GDR usages.

It was suggested that if a NUT for recovery point header were used, the recovery poc count could be placed in the tile group header.

Suggested design goals:

- Clarifying use of recovery point indication, with syntax location other than in SEI

  - Should be applicable to use cases where GDR period is equal to 0

- Have GDR bitstreams without IRAP pictures not be non-conforming

  - A CVS could start with a picture corresponding to one of these GDR indicators

- No need to define conforming output during the recovery period, but should be conforming starting with the recovery point

- Minimum changes to the specification

Two options for non-SEI syntax:

- Non-VCL NAL unit in same access unit as the recovery point beginning picture

- VCL NUT value for the recovery point beginning picture

Probably coded picture within the recovery period wouldn’t be leading pictures, because that wouldn’t be low delay.

Offline discussion to try to design a solution for these design goals, coordinate by S. Deshpande. (involving proponents of N0101, N0495, N0115, N0072)

Revisit.

JVET-N0865 AHG17: Gradual Random Access

This document provides a proposed text related to gradual random access based on the discussions in HLS BoG. The proposal is a result from offline discussions at the Geneva meeting involving proponents of JVET-M0529, JVET-N0101, JVET-N0495, JVET-N0115 and JVET-N0072. The proposed text contains the following elements that the authors claim is a minimal set of specification text changes to enable a CVS to start with a gradual random access (GRA) picture that is not necessarily all Intra coded:

- One flag gra_enabled_flag in the SPS that specifies whether GRA pictures may be present or not

- One new VCL NAL unit type, GRA_NUT, that indicates a GRA picture

- One syntax element, recovery_poc_cnt, in the tile group header that is a copy of the recovery_poc_cnt syntax element from the recovery point SEI of HEVC.

- A process to start a CVS with a GRA picture similar to how a CVS is started with a CRA picture

Defines a CVSS pic and access unit and GRA pic and access unit. Defines conformance starting at the recovery picture.

BoG recommends to adopt a GRA solution as described in JVET-N0865 v3, with the following characteristics:

- One flag gra_enabled_flag in the SPS that specifies whether GRA pictures may be present or not

- One new VCL NAL unit type, GRA_NUT, that indicates a GRA picture

- One syntax element, recovery_poc_cnt, in the tile group header that is a copy of the recovery_poc_cnt syntax element from the recovery point SEI of HEVC.

- A process to start a CVS with a GRA picture similar to how a CVS is started with a CRA picture

Editors are encouraged to find another name which doesn’t include “gradual” in the name, as it can apply for other use cases with recovery poc count equal to 0. Recovery Point Access (RPA) is suggested as an option.

#### 6.18.2.5 High efficiency random access (4)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0119 AHG17: EDR - external decoding refresh [Y.-K. Wang (Huawei)] [late]

This contribution proposes a design for support random accessing a bitstream from inter-coded pictures without a recovery period. Such random accessing needs a small number of reference pictures to be provided by an external means, thus named external decoding refresh (EDR).

The proposed design is summarized as follows:

- An EDR picture is a random accessible picture for which one or more external pictures are needed when random accessing from the picture.

- An EDR picture may start a CVS (hence may also start a bitstream).

- External pictures are provided in the form of coded pictures instead of decoded pictures.

- There is no change to RPL syntax, semantics, and derivation process.

- A new NAL unit type (EDR_NUT) is defined for EDR pictures.

- For each EDR picture, the POC MSB values of the external pictures and the MSB values are signalled in the tile group header.

- A flag is signalled in the SPS to specifies whether EDR pictures may be present in the CVS.

The proposed detailed spec text changes are provided in an attachment.

Proposes an externally provided IRAP coded reference picture for the current picture.

It was suggested that a rewriting process with a long term reference picture could be used to create a valid bitstream so that the decoder would be unaware.

The SHVC external base layer concept was suggested to be conceptually related, on a spatial scalability basis vs. a temporal scalability basis in this contribution, but differs in that this externally provides a coded picture rather than a decoded picture.

Required changes are a new NUT, an SPS flag, and modification to TGH to indicate POC MSB values of the external pictures.

It was commented that any externally provided reference would be required to have the same SPS.

Could this capability be support provided at the system layer instead of the codec layer?

Some concerns were expressed about system implementation complexity.

We have generally been trying to reduce the number of NUT values.

For further study with systems experts.

JVET-N0244 AHG17: Video coding based on cross RAP referencing (CRR) [H. Yu, L. Yu (Zhejiang Univ.)] [miss] [late]

Withdrawn.

JVET-N0494 AHG17: Dependent random access point pictures in VVC [M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

This contribution proposes to include dependent random access point (DRAP) pictures to the VVC draft, similar to the DRAP pictures in HEVC. A DRAP picture is a RAP picture that may use the previous IRAP picture in decoding order for temporal prediction.

In contrast to DRAP pictures in HEVC, the following is proposed for VVC:

- To signal DRAP indication as a VCL NAL unit type in the NAL unit header

- To signal a flag in SPS whether the CVS may contain DRAP pictures or not

HEVC has an SEI message with a DRAP indication. Semantics for this NUT are similar to the SEI message.

N0119 is related, but has a conforming decoder behavior.

It was suggested that using temporal id values might address some of the use cases.

Without this change, encoders are free to use these coding pictures, but there may be systems or decoder benefits from having the indicator.

ISOBMFF spec has support for DRAP, independent of the codec type. The HEVC SEI message is used for setting the ISOBMFF field.

This contribution doesn’t include changes to conforming decoder behavior.

BoG recommends to copy the HEVC dependent random access indication SEI message (with modifications as appropriate to adapt to VVC)

JVET-N0072 AHG17: New NAL unit types for VVC [L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

The DRAP part of this document belongs to this agenda item.

Additional use cases were discussed, which asserts some systems layer benefits.

#### 6.18.2.6 Miscellaneous general HLS topics (8)

JVET-N0061 AHG19: Signalling temporal IDs and levels for HFR backwards compatible bitstreams [V. Drugeon (Panasonic)]

Chaired by S. Deshpande.

An issue about signalling of maximum temporal ID and level of sub-bitstreams is reported when using temporal scalability to enable backwards compatibility of High Frame Rate (HFR) bitstreams to standard frame rate receivers. Two alternative solutions are suggested that both consists in signaling the number of frame rate versions of the content within the bitstream, together with the maximum temporal ID value and level for each frame rate version. The first solution signals the necessary information within the profile_tier_level syntax structure. The second solution signals the necessary information at the beginning of the Sequence Parameter Set and includes one profile_tier_level syntax structure for each frame rate version of the content.

It was commented if the second solution is only for the purpose of signaling level idc.  It was agreed that was the case.

It was agreed by the proponent that second solution will need more bits that the first proposal. First proposal needs a constraint.

It was asked about the scenarios where the temporal sub-layers are being dropped.

It was asked if you envision change from SFR to HFR at the boundary of CVS. This was agreed by the proponent.

It was remarked that we have a max sub layers indicator in video layer and there is another one or more related syntax in system layer such as DVB.  It was remarked that decoder can remove sub-layers anytime. It was asked how that would be expressed. It was commented by proponent that in some systems dropping temporal sub-layers is not common.

It was asked why in this case SFR is not just in temporal sub-layer. It was answered that additionally trick play may want to supported.

It was asked why current vvc signaling of sub-layer level info not sufficient.

Regarding signaling level indicator it was commented that broadcasters typically fix this level. It was also asked that in a program max TID for SFR could change and may need to be supported to be frame accurate.

It was asked what is the benefit fort he system if this information can change at elementary stream layer on CVS basis.

It was asked if this can handle a situation where we can change from 2 to 3 sub-bitstreams and if how system will know the mapping between operation points.

It was remarked that these are signalled code points that content authors would like decoders to use when it operates at some display frame rates. So some frame rate info. Signalling will be useful.

It was commented that why is this in PS and not in SEI.

It was commented why you would mandate a constraint as proposed. Also was commented that constraint as proposed is not useful when trick play is additionally intended.

For further study.

JVET-N0063 AHG17: Separate essential display information from non-essential display information in the Video Usability Information [V. Drugeon (Panasonic)]

This proposal suggests including and reorganising a VUI-like syntax structure for VVC, although not necessarily described within the VVC specification itself. It is proposed that essential display information should be signalled somewhere within the video elementary stream and that this essential information contains only the following type of information: Sample aspect ratio, Colour description, Chroma location, Fields coding or frame coding for interlaced content, and Timing information. Additional information about the content may be signalled, but separately from the above mentioned essential information.

It was suggested to bring a revision of the document where just the parameters the proponent recommends be included in the VUI.  It was suggested to have the picture timing and HRD be put into a different syntax structure. There are other contribution making HRD proposals.

There was discussion in the BoG about which syntax elements to include in the VUI. The BoG is recommending only those syntax elements for which someone expressed support for inclusion. This should be considered to be a starting point for the VUI design, which may be further modified based on future input contributions.

BoG recommends to adopt a VUI design that contains parameters for aspect ratio, colours description, field seq flag, chroma location, overscan info, and video format and full range flag, as reflected section 4 of in a –v4 version of the document.  HRD and picture timing parameters are expected to be included in the design in some way, but are deferred for consideration in other contributions.

JVET-N0120 AHG17: Misc. HLS clean-ups [Y.-K. Wang (Huawei)] [late]

This contribution proposes the following aspects:

- To add some constraints on IRAP, RASL, and RADL pictures, which are asserted to be technically identical as in HEVC for IRAP, RASL, and RADL pictures.

- To add text to the empty clause on order of VCL NAL units and association to coded pictures, such that decoding order of VCL NAL units within a coded picture is clearly specified.

- To change the description of the general decoding process, such that the term "the bitstream" in the specification does not refer to just one CVS.

It was suggested that the constraint on the order of leading and trailing pictures for HEVC has been a problem for coding of interlaced content with field pictures.

Proposal 1 further discussed on 26 March. It was suggested that the constraint might conflict with a coding structure that is sometimes used to carry interlaced content. Further study encouraged.

It was noted that the parameter set activation process is still not in the specification.

Proposal 2 is trying to reflect the decision in Marrakesh. The constraint on tileGroupAddrA was questioned. There is still some confusion about what the design should be in this area.

Further study encouraged once the tile design is stabilized.

Proposal 3 seems to be motivated by the existence of the DPS, to clarify the difference between a CVS and a bitstream. It was noted that NoIncorrectPicOutFlag was previouslycalled NoRaslOutputFlag. It was asserted to be editorial in nature. It was suggested that order could be changed so that bitstream was defined before it was used. BoG recommends this an editorial improvement to sections 8.1.1, 8.1.2, and 8.1.3, with details delegated to the editor.

JVET-N0227 Maximum transform size signaling in HLS [L. Li, M. Koo, J. Nam, J. Heo, J. Lim (LGE)] [miss] [late]

No presentation requested. Replaced by JVET-N0808 to be reviewed with CE6.

JVET-N0288 AHG17: Comments on High-Level Syntax of VVC [S. Deshpande (Sharp)]

Reviewed in coded picture regions BoG

JVET-N0352 AHG17: Conformance Window [K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

This contribution proposes to re-introduce the conformance window with the same syntax and semantics as in HEVC to allow coding of pictures that are non-multiple of the minimum CU size.

BoG recommends to adopt.

#### 6.18.5 HRD (7)

JVET-N0062 AHG19: Signalling HRD models for HFR backwards compatible bitstreams [V. Drugeon (Panasonic)]

An issue about decoding time stamps is reported when using temporal scalability to enable backwards compatibility of High Frame Rate (HFR) bitstreams to standard frame rate receivers. It is suggested to signal CPB removal delays and DPB output delays for each temporal sub-layer contained in the bitstream to solve this issue.

JVET-N0077 Hypothetical Reference Decoder in VVC [K. Kazui (Fujitsu)]

It is proposed to incorporate Annex C of the HEVC standard [2] into the VVC draft [1] as the starting point for designing the hypothetical reference decoder (HRD) model for VVC. It is also proposed to add HRD-related parameters to the sequence parameter set (SPS), picture parameter set (PPS) and tile group header, and to define supplemental enhancement information (SEI) messages for signalling HRD parameters as the starting point.

In version 2, section 3 is removed since the author realized that it was already agreed in spirits. Moreover, the proposal on Recovery point SEI is removed since it is not directly related to the HRD model.

The HEVC design was copy-pasted into the specification. Adaptation to VVC is still needed. Proposes including the following SEI messages:

- Buffering period SEI

- Picture timing SEI

- Recovery point SEI

- Decoding unit info SEI

- Filler payload SEI

It was suggested to not include the recovery point SEI in the design now until the GDR proposals are decided upon. It was commented that recovery point SEI is not really part of the HRD design.

The question was raised if picture timing SEI with interlace support should be included. There was support for including it, but it should be checked if adaptation for VVC would be required.

It was suggested that N353 is related.

JVET-N0350 AHG17: HRD starting point [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

This contribution provides a stripped down basic Hypothetical Reference Decoder (HRD) design as starting point for VVC. It is based on the HRD model of HEVC, with the following simplifications:

- No sub-picture timing

- No alternative timing for removal of RASL pictures

- Bug-fix for splicing

Decoded picture buffer conformance text is not included, because the referenced syntax elements either do not exist in the VVC specification (e.g. no_output_of_prior_pics_flag) yet or are replaced by different techniques (e.g. RPS).

It was suggested to defer the design for sub-picture timing until the general specification design for sub-pictures is stabilized.

It was also suggested to defer the design for alternative timing, until the NAL unit types design is stabilized.

Addition of a filler data NUT value.

Based on the stripped down HEVC design. Doesn’t include the DPB design.

Further discussed on 26 Mar.

A potential bug in the HEVC HRD design was identified. It was recommended to bring a contribtuion to JCT-VC to consider modifying the HEVC HRD design.

BoG recommends to add an editor’s note in the adoption of N0350/N0423 to note the need to study to consider if there is a bug for the concatenation flag in equation C-10.

JVET-N0351 AHG17: HRD parameters for temporal scalability [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

The contribution proposes to indicate CPB removal times for operation points using a different number of temporal sub-layers within the buffering period SEI message or picture timing SEI message. Since removal times follow a pattern, these patterns can be included in the buffering period SEI or directly coded in the picture timing SEI.

Temporal scalability requires a smart sub-bitstream extraction process for rewriting of HRD for temporal scalability.

It was asserted that in HEVC the scalable nesting SEI was needed to get the desired behavior. For VVC, could temporal id be used?

Two options were presented.

Offline  discussion to attempt to harmonize the N0351 and N0062 proposals. Asked to provide some bit counts for RA CTC.

JVET-N0353 AHG17: Parsing of HRD related SEI messages [K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

The contribution proposes to add HRD parameters, Buffering Period, Picture Timing SEI messages to the VVC specification. General syntax and semantics are based on the corresponding HEVC structures. Parsing of SEI messages is made possible independent of the presence of the SPS.

Proposes reordering of syntax elements and some replication without changing processs, to avoid parsing dependency between SEI messages.

BoG recommends to adopt restructuring of buffering period and picture timing SEI messages as proposed, except removal of bp_seq_parameter_set_id in buffering period SEI.

JVET-N0423 AHG17: Hypothetical Reference Decoder [S. Deshpande (Sharp)]

Specification text is proposed for Hypothetical Reference Decoder (HRD) for VVC as a starting point.

Compared to HEVC HRD specification text, at high-level following modifications were made to the proposed starting point text for VVC.

- Specification text corresponding to VPS is not included as VVC WD does not currently include VPS.

- Specification text corresponding to BLA pictures is not included as VVC WD does not currently include BLA pictures.

- Operation point definition is adapted to VVC WD and thus not using a target layer identifier list.

- Specification text is updated to not include layer set, layer id list aspect.

- Specification text is updated to not include filler data NAL unit aspect as that NAL unit is not currently included in VVC WD.

- Specification text corresponding to decoding unit based/ sub-picture based operation is not included for now. It is expected that since decoding unit based operation can achieve low latency, this should be added in future.

- Specification text for RPS related aspects is modified to refer to method used in VVC WD.

- The conditions which use NAL unit type for conditional signalling are updated.

- Specification text for cropping related aspect is not currently included. It is asserted that it could be straightforward to include this aspect later.

It was suggested that filler data could be included.

This contribution included DPB data.

It was suggested that having teamporal sub-layer DPB could be included.

It was suggested that conformance cropping could be included.

The IBC tool is in the current specification, but it differs from the current picture referencing feature in HEVC SCC. It wasn’t considered in this design, but it might not need any modifications to support, and should be studied.

The starting of the HRD design should include:

- CPB and DPB

- AU but not DU based operation

- Filler data NUT and SEI

- picture timing SEI and buffering period SEI

- Sub-layer DPB parameters in SPS

- VUI HRD parameters (exact location TBD) (revisit once N0353 is reviewed)

- TGH pic output flag

- TGH no_ouptut_of prior_pics_flag for all IRAP

- Sub-layer HRD CPB

- Not include alternative timing at this stage (further study encouraged, especially for potential new NUTs)

- Updated definitions in Section 3

Editorial improvements to align with the latest version of HEVC should be incorporated.

We expect to include some type of DU operation mode, but defer until coded picture regions design stabilized.

BoG recommends to adopt a starting point for HRD design as captured above, from an editorial combination of N0423 and N0350. The most recent HEVC HRD specification text will be used as the basis.

JVET-N0706 AHG17: Decoded Picture Hash SEI [K. Sühring, R. Skupin, Y. Sanchez (HHI)] [late]

### Contribution proposes to include decoded picture hash SEI from HEVC in design.

### BoG recommends to adopt.

JVET-N0867 Harmonized proposal on HRD parameters for temporal scalability [V. Drugeon (Panasonic), R. Skupin, Y. Sanchez (HHI)] [late]

This document presents a harmonized proposal of JVET-N0062 and JVET-N0351 for signalling of HRD parameters for temporal scalable bitstreams.

It is asserted that the HEVC solution for specifying and signalling the HRD parameter for temporal scalability (non-scalable nested SEI messages per AU for each temporal layer) comes at an undesirable signalling and implementation burden. Furthermore, it led external organizations such as DVB to avoid the specified solution and impose further constraints on bitstream structure that are detrimental to coding efficiency. Detailed problem descriptions can be found in documents JVET-N0062 and JVET-N0351.

The proposal is to specify and signal HRD parameters in temporal scalable VVC bitstreams for each AU in a single picture timing SEI message, without the need for scalable nesting SEI messages.

BoG recommends to adopt as proposed.

### JVET-N0869, Decoding Parameter Set with default ID equal to zero, S. Wenger and W. Wan

This document contains the authors’ attempt to represent the compromise regarding the Decoding Parameter Set (DPS) reached on March 23 in the JVET Track B (joint meeting with systems).  A DPS with a DPS ID equal to 0 is defined in the VVC specification as a DPS which does not set any constraints on the bitstream.  It’s a requirement of bitstream compliance that, if a DPS with an ID equal to 0 is activated, it complies with that set of restrictions.

Defining a DPS with ID equal to 0, today, is asserted to be reasonably straightforward and provided below, with the exception of tiers, profiles, and the DPS extension mechanism.  Some thoughts are provided regarding potential specification problems related to tiers and profiles.  Solutions to these problems may well be better devised once we have a more thorough understanding of future Annex A, and therefore proposed for further study.

Future proponents to DPS-related technologies are encouraged/requested/required to provide text reflecting the values of proposed codepoints for the DPS with ID equal to zero.

### A related document JVET-N0349 was originally reviewed in the track.

### The specific language in N0869 expressing the constraint could be more simply expressed as follows:

- dps_id shall not be equal to 0 in DPS. When the value of dps_id  in the SPS is equal to 0, no DPS is activated.

### BoG recommends to adopt with the modification to the semantic restriction as listed above.