| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0644-v1 |
| --- | --- |

| Title: | Cross-check of JVET-N0092 (CE7-related: Unified Rice parameter derivation of abs_remainder and dec_abs_level syntax elements) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | aramasub@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The results of a cross-check of JVET-N0092-v2 is reported. It is reported that the cross-checker has conducted a cross-check of the bit-rates and the PSNRs reported for the AI, RA and LDB configurations under CTC, and the results obtained match the values reported by the proponent. The software implementation that was provided by the proponents to the cross-checkers was used for the cross-check.

# Introduction

The syntax elements of coefficient coding are parsed using three coding passes: the first pass parses the syntax elements sig_coeff_flag, abs_level_gt1_flag, par_level_flag, and abs_level_gt3_flag; the second pass parses abs_remainder and dec_abs_level; and the third pass parses coeff_sign_flag. The Rice parameter derivations of the syntax elements abs_remainder and dec_abs_level are different.

# Cross-checked test

This proposal proposes the following changes in the coding of transform coefficients:

–	Combine the syntax elements abs_remainder and coeff_sign_flag into a new syntax element abs_remainder_with_sign in cases where abs_remainder is signalled currently; the syntax element coeff_sign_flag is not signalled in these cases.

–	Signal the syntax element coeff_sign_flag in the second pass along with abs_remainder_with_sign and dec_abs_level.

–	Modify the Rice parameter derivation of the syntax element abs_remainder_with_sign; the modified process is similar to the Rice parameter derivation of dec_abs_level; the modified process also considers the transform_skip_flag in the Rice parameter derivation which is not considered in the case of dec_abs_level.

# Software

A bug is reported in the software when sign data hiding is enabled. However, since sign data hiding is not enabled in the common test conditions, this does not affect the results obtained. No other deviation was observed in the software implementation compared to the description provided in JVET-N0092-v2.

# Results

The PSNR numbers and bit rates numbers for AI, RA and LDB configurations were tested under CTC, and it is reported that they match the bitrates and PSNRs reported by the proponent.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).