| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0737_r1 |
| --- | --- |

| Title: | Non-CE2: Combination of affine MV clip and prediction refinement with optical flow |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li, Guichun Li, Xiaozhong Xu, Shan Liu | Tel: Email: | {xlxiangli, guichunli, xiaozhongxu, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes a combination of affine MV clip and affine prediction refinement with optical flow. It is reports that the combination brings additional coding gain over VTM-4 while keeping the worst-case memory bandwidth of bi-predicted affine the same as that of regular bi-predicted inter block.

# Introduction

In JVET-N0398 [1], affine MV clip was proposed to reduce the worst-case memory bandwidth of 4x4 sub-block affine. Essentially, the MVs of four 4x4 sub-blocks within one 8x8 block are constrained such that the max difference between integer parts of the four 4x4 sub-block MVs is no more than a threshold, such as 1 pixel. Consequently, the worst-case memory bandwidth is reduced to (8+7+1)×(8+7+1)/(8×8)×2=8 luma samples per pixel. When the threshold is set to 0, the worst-case memory bandwidth is further reduced to (8+7+0)×(8+7+0)/(8×8)×2=7.03125, which is the same as regular 8x8 bi-prediction.

In JVET-N0236 [2], affine prediction refinement with optical flow (PROF) was proposed to compensate the distortion introduced by affine MV rounding at 4x4 level.

Although MV clip didn’t show much negative coding efficiency impact, it sometimes introduces additional MV rounding error, which can be compensated by PROF. Therefore, the combination of the two methods are reported in this contribution.

# Simulation results

To check the performance of the combination, three tests were performed on top of VTM-4.0 under common test conditions [2] as follows.

- Test 1: PROF with Tx=1023, Ty=1023 (No MV clip)

- Test 2: Test 1 + Tx=1, Ty=1 (cfg setting: --AffineIntx=1 --AffineInty=1)

- Test 3: Test 1 + Tx=0, Ty=0 with distance based MVmin and MVmax determination on (cfg setting: --AffineIntx=0 --AffineInty=0 --DistBasedClip=1)

## Test 1

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | 0.02% | 0.03% | 108% | 103% |
| Class A2 | -0.81% | 0.38% | 0.32% | 110% | 103% |
| Class B | -0.61% | -0.24% | -0.12% | 109% | 105% |
| Class C | -0.31% | -0.09% | -0.29% | 108% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.47% | -0.03% | -0.05% | 109% | 104% |
| Class D | -0.63% | -0.23% | -0.11% | 108% | 103% |
| Class F | -0.82% | -0.36% | -0.44% | 109% | 103% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.29% | 0.15% | -0.12% | 106% | 105% |
| Class E | -0.33% | -0.23% | -0.06% | 113% | 109% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.48% | 0.10% | -0.12% | 107% | 108% |
| Class F | -1.34% | -0.71% | -1.16% | 107% | 106% |

## Test 2

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | 0.05% | 0.02% | 109% | 102% |
| Class A2 | -0.74% | 0.45% | 0.32% | 110% | 103% |
| Class B | -0.60% | -0.28% | -0.01% | 109% | 105% |
| Class C | -0.30% | -0.01% | 0.03% | 108% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.46% | 0.00% | 0.07% | 109% | 104% |
| Class D | -0.58% | -0.33% | -0.23% | 106% | 103% |
| Class F | -0.80% | -0.36% | -0.36% | 109% | 103% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.25% | -0.08% | 0.01% | 107% | 108% |
| Class E | -0.51% | 0.45% | -0.22% | 112% | 109% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.52% | -0.12% | -0.24% | 106% | 107% |
| Class F | -1.35% | -1.31% | -1.22% | 109% | 104% |

## Test 3

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.05% | 0.18% | 110% | 104% |
| Class A2 | -0.48% | 0.39% | 0.28% | 110% | 104% |
| Class B | -0.51% | -0.12% | 0.00% | 109% | 105% |
| Class C | -0.24% | -0.05% | -0.19% | 109% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.35% | 0.03% | 0.04% | 109% | 105% |
| Class D | -0.52% | -0.33% | -0.15% | 107% | 104% |
| Class F | -0.54% | -0.19% | -0.31% | 109% | 103% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.24% | 0.15% | 0.03% | 107% | 111% |
| Class E | -0.46% | -0.06% | -0.89% | 112% | 106% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.49% | -0.43% | -0.49% | 106% | 108% |
| Class F | -1.30% | -1.19% | -1.40% | 108% | 105% |

It should be noted that in this test the worst-case memory bandwidth of affine mode (8x8 block size) is the same as that of regular 8x8 bi-prediction. It is further emphasized that in this case two 4x4 affine blocks can be interpolated simultaneously with SIMD implementation, which is similar to 8x4 interpolation.

# Conclusions

This contribution reports the combination of JVET-N0398 and JVET-N0236. It is reported in test 3 that the combination brings additional coding gain over VTM-4 while keeping the worst-case memory bandwidth of affine mode the same as that of regular inter 8x8 bi-prediction.

# References

- X. Li, G. Li, X. Xu, S. Liu, “CE2-4.4: Affine block memory bandwidth reduction by MV clip,” JVET-N0398, 14th meeting, Geneva, CH, 19–27 March 2019.

- J. Luo, Y. He, “CE2-related: Prediction refinement with optical flow for affine mode”, JVET-N0236, 14th meeting, Geneva, CH, 19–27 March 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

# Patent rights declaration

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).