| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0742-v2 |
| --- | --- |

| Title: | CE8-related: Joint test of JVET-N0094 and CE8-4.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Olena Chubach, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan Sunmi Yoo, Jungah Choi, Jin Heo Jangwon Choi, Ling Li, Jaehyun Lim 19 Yangjaedaero-11gil, Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +886-3-5670766 {mohammed.sarwer, olena.chubach, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com sunmi.yoo@lge.com |
| Source: | MediaTek Inc., LG Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution shows the results of combined test of the CE8-4.1c in JVET-N0428 and the Test 3 in JVET-N0094. The CE8-4.1 reorders residuals for intra predicted and transform skipped blocks. JVET-N0094 proposed to derive the context models for coefficient coding from three previously decoded neighboring coefficients located at the right, bottom, and bottom-right positions of the current to-be-decoded coefficient. A separate context set is defined for transform skip mode. Simulation results of this joint test are reported as follows.

Luma BD-rate for Class F sequences = -1.84% (AI), -1.45% (RA), -1.00% (LB)

Luma BD-rate for SCC sequences = -2.46% (AI), -1.65% (RA), -1.16% (LB)

Luma BD-rate for CTC sequences = -0.11% (AI), -0.05% (RA), -0.06% (LB)

# Proposed methods

## Tests in JVET-N0428:

JVET-N0428 [3] proposed three different tests.

- CE8-4.1a: To maintain the statistics of the syntax elements for residual coding, it is proposed to flip a residual block horizontally or vertically according to the intra mode. In this method, the intra prediction mode can be divided into three categories – using mainly top-reference modes, left-reference modes, and others.

  - Top-reference modes: VER_IDX -3 to 80

  - Left-reference modes: -14 to HOR_IDX + 3, Planar mode, DC mode

- CE8-4.1b: In this method, the residual blocks are rotated by 180 degrees when the current block is a transform skipped and intra predicted block. It is applied from 4×4 to 32×32 TS blocks as well.

- CE8-4.1c: In this test, the methods described above, flipping and rotating are combined together. In detail, 4×4 transform skip (TS) blocks are flipped as in test a, and larger TS blocks are always rotated if they are coded as intra modes.

## Tests in JVET-N0094:

JVET-N0094 [2] proposed context modelling for coefficient coding of transform skip mode. Context indexes are derived from three neighbors (right, bottom, and bottom-right). Three tests are proposed.

- Test 1: In this test only the number of neighbors for transform skip mode is reduced. Same context sets are used for both transform and transform skipped transform block. Therefore, the number of contexts are exactly the same as that in VTM4.0 (i.e, 232).

- Test 2: In this test, separate context sets for transform skip mode for sig_coeff_flag, par_level_flag, abs_level_gt1_flag, and abs_level_gt3_flag are implemented on top of test 1. The number of contexts is 265 whereas in VTM4.0, the number of contexts is 232.

- Test 3: In this test, separate context sets for last_sig_coeff_x_prefix and last_sig_coeff_y_prefix: are implemented on top of test 2. The number of contexts is 295 whereas in VTM4.0, the number of contexts is 232.

# Simulation results

In this contribution, JVET-N0094 is implemented on top of CE8-4.1. Table 1 shows the results of combining CE8-4.1c in JVET-N0428 and Test 3 in JVET-N0094.

Table 1:  Results of proposed joint test

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.03% | 0.03% | 102% | 100% |
| Class A2 | 0.00% | -0.04% | -0.01% | 102% | 99% |
| Class B | -0.04% | -0.02% | -0.05% | 102% | 100% |
| Class C | -0.33% | -0.19% | -0.29% | 102% | 99% |
| Class E | -0.07% | -0.06% | -0.06% | 101% | 100% |
| Overall | -0.11% | -0.06% | -0.09% | 102% | 100% |
| Class D | -0.29% | -0.33% | -0.20% | 101% | 98% |
| Class F | -1.84% | -1.25% | -1.42% | 101% | 98% |
| TGM | -2.46% | -1.56% | -1.57% | 101% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.25% | 0.13% | 100% | 95% |
| Class A2 | -0.02% | -0.03% | 0.00% | 100% | 99% |
| Class B | -0.01% | -0.17% | -0.29% | 100% | 96% |
| Class C | -0.16% | 0.33% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.02% | -0.08% | 100% | 98% |
| Class D | -0.12% | -0.08% | 0.15% | 101% | 97% |
| Class F | -1.45% | -0.98% | -1.14% | 100% | 96% |
| TGM | -1.65% | -0.75% | -0.79% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.53% | 0.51% | 101% | 100% |
| Class C | -0.09% | 0.09% | 0.13% | 101% | 99% |
| Class E | -0.11% | -0.18% | 0.45% | 101% | 102% |
| Overall | -0.06% | 0.20% | 0.37% | 101% | 100% |
| Class D | -0.06% | 2.58% | 1.42% | 100% | 99% |
| Class F | -1.00% | -0.72% | 0.05% | 100% | 100% |
| TGM | -1.16% | -0.76% | -0.71% | 100% | 101% |

# Conclusions

Compared to VTM4.0, this combined test reportedly achieves BD-rates of -0.05%/-0.02%/-0.08% for Y/Cb/Cr, respectively, under RA. In case of TGM sequences under RA, BD-rates are -1.65%/-0.75%/-0.79% for Y/Cb/Cr, respectively.

# References

[1] VTM4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

[2] M. G. Sarwer, O. Chubach, C.-W. Hsu, Y.-W. Huang, and S.-M. Lei, “CE8-related: Context modelling of transform skip mode,” Document of Joint Video Experts Team, JVET-N0094

[3] S. Yoo, J. Choi, J. Heo, J. Choi, L. Li, J. Lim, and S. Kim, “CE8-4.1: Rearrangement of the residual block for transform skip,” Document of Joint Video Experts Team, JVET-N0428.

[4] X. Xu, Y-H Chao, Y-C Sun, and J. Xu , “Description of Core Experiment 8 (CE8): Screen content coding tools,” Document of Joint Video Experts Team, JVET-M1028.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).