| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0743 |
| --- | --- |

| Title: | Crosscheck of JVET-N0388 (Non-CE6: Shape Adaptive Transform Selection for ISP, SBT and MTS) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Pierrick Philippe | Email: | pierrick.philippe@orange.com |
| Source: | Orange, bcom |  |  |

_____________________________

# Abstract

This document reports the crosscheck of JVET-N0388. It is reported that the average BD-rates match those provided by the proponents. No discrepancies have been found between the provided software and the description of the tests. The execution times are within the measurement error range.

# Description

In this proposal, a new design of the transform coding in which the transform pair of Intra Subpartitions  and Subblock transform is the same as the one for Implicit MTS. Additionally, the Implicit MTS transform pair is embedded within the MTS design, so that it becomes an encoder option without signaling. The last MTS pair being replaced with the pair of transforms determined for the implicit MTS.

The software was verified and the elements described above are consistently implemented.

Consequently, the software matches the description made in JVET-N0388.

# Experimental Results

Experiments were performed based on VTM4.0, in All Intra configuration, using the common test conditions described in JVET-M1010.

## Method 1

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.02% | 0.07% | 93% | 105% |
| Class C | 0.02% | -0.08% | -0.05% | 93% | 109% |
| Class E | -0.01% | 0.09% | -0.14% | 93% | 107% |
| Overall |  |  |  |  |  |
| Class D | 0.01% | 0.02% | -0.18% | 93% | 109% |
| Class F |  |  |  |  |  |

The average BD-rates match those provided by the proponents. Note that the execution times are NOT reliable as the measurement on the experiment and the anchors were performed on different computing architectures. According to the modifications in the software, the encoding and decoding runtimes should be in the range of 100%

# Patent rights declaration

Orange and b<>com may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).