| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0753 |
| --- | --- |

| Title: | AHG12: Design decision questions on independently coded picture regions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | For information |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela (Nokia) Sachin Deshpande (Sharp) |  |  |
| Source: | Nokia, Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

As a result of a scan of the design choices made in the input contributions, this contribution contains a set of questions that are believed to be applicable to all types of approaches for independent coded picture regions. This contribution does not cover design choices that are applicable to a specific type of an approach only (e.g. only to layer-based approaches).

# Design questions

Note: the contribution references are likely to be incomplete.

## On extraction, repositioning, and merging

Background: "Extracting VCL NAL units of a subset of MCTSs from one VVC bitstream and reposition them to another VVC bitstream without substantial difficulty (e.g., without VLC-level modifications)" has already been agreed as a design goal.

- Should the VVC design support merging of independent picture regions originating from different bitstreams?

- Yes (N0055)

- No

## On region dimensions and layout

- Are the width and height of independent picture regions integer multiples of CTUs?

- Yes

- No (N0055). (Use case: matching of independent picture region width/height with face or face row width/height of 360 video)

- Are the decoded independent picture regions of the same POC arranged to a single decoded picture?

- Yes, always (…)

- Encoder controls if sub-pictures are arranged to a single decoded picture or output separately. (N0045)

- No, sub-pictures are output separately. (N0278)

- When independent picture regions of the same POC are arranged to a single decoded picture, do they appear in a grid crossing the entire picture (similar to tiles)?

- Yes

- No (N0044, N0045, N0046, …)

- When independent picture regions of the same POC are arranged to a single decoded picture, is it required that an independent picture region is used as prediction reference is collocated with the current independent picture region?

- Yes (N0073)

- No

- When independent picture regions of the same POC are arranged to a single decoded picture, is the decoded picture entirely occupied by independent picture regions?

- Yes

- No (N0046, N0073)

- What information should be included to indicate a layout when independent picture regions of the same POC are arranged to a single decoded picture?

- Sub-picture sizes and locations within the decoded picture. (N0073, N0107)

- Sub-picture sizes only, the layout is determined by the decoding order of sub-pictures: the sub-picture takes the next such unoccupied location in CTU raster scan order within a decoded picture that is large enough to fit the sub-picture (N0046)

- Independent coded region boundary IDs, a pair of matching identifiers expresses that two of independent coded regions are neighbours along a certain border (N0411)

- Location to indicate region dimensions and layout?

- VPS

- SPS

- PPS

- Tile group header

- SEI

## On boundaries of independent picture regions

- Treating of the boundaries of independent picture regions (for inter prediction)

- A topic of CE12

- Should the encoder be able to control whether loop filtering over independent picture region boundaries is enabled/disabled, or is the loop filtering over independent picture region boundaries always disabled?

- Encoder controlling (N0107)

- Always disabled

## On level and conformance

- Specifying level and conformance for a specified subset of independent picture region sequences originating from the same bitstream. This helps when extracting a viewport from a 360° video and decoding just that.

- Yes (N0044, N0354)

- No

- Specifying level and conformance for independent picture region sequences originating from several bitstreams. This helps in viewport-dependent 360° video streaming.

- Yes (N0044)

- No

## Other

- Enable IRAP and non-IRAP independent picture regions having the same POC?

- Yes (N0047, N0055, N0108)

- No

- Adaptive resolution changing on independent picture region basis?

- Use cases:

  - Partial intra updating with open GOP structures for viewport change in region-wise mixed-quality viewport-dependent 360° streaming.

  - Partial intra updating for viewport change in region-wise mixed-resolution viewport-dependent 360° streaming.

- Yes (N0044)

- No

- Should VVC fulfil MPEG's requirements on decoding interface for immersive media access and delivery?

- Yes (N0043)

- No

- To do flexible/ hierarchical tiling using independent picture regions?

- To do flexible/ hierarchical tiling using independent picture regions? Yes/ No

- To do flexible/ hierarchical tiling using tile group level signaling (N0066, N0111, N0348): Yes/ No