| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0754-v1 |
| --- | --- |

| Title: | BoG Report on Coded Picture Regions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Email: | sdeshpande@sharplabs.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met on 20 Mar 2019 from 14:30 to 20:45.

The BoG met on 21 Mar 2019 from 9:00 to 10:00.

Approaches for independently coded picture regions are categorized as follows:

- Motion-constrained tile sets (MCTSs): Encoding is constrained in a manner that a rectangular set of tile groups can be extracted and repositioned without affecting their decoding or decoding of other tile groups.

Independent rectangular tile groups (IRTGs): Encoding and decoding are modified to treat IRTG boundaries like picture boundaries. Implications on IRTG boundaries are handled in low-level decoding process.

MCTS and IRTG are similar except how they treat boundaries. The term MCTS is used below for this category

- Sub-pictures (non-layer approach): Sub pictures are treated like pictures in the low-level decoding process. Sub-pictures of the same timestamp are spatially arranged onto an output picture.

- Independent sub-picture layers: It can be controlled if decoded pictures of the same timestamp and of different layers are composed onto the same output picture or output separately.

The BoG reached following high-level recommendations so far:

- Should VVC coding spec support defining profile/tier/level (PTL) individually for each independent coded region.

  - Yes, but not mandated to be signalled.

- Should VVC coding spec support HRD model individually for each independent coded region.

  - Yes, but not mandated to be signalled.

- Should VVC coding spec specify a combined HRD operation (e.g. CPB, DPB operation) for combined independent coded regions.

  - Yes, for MCTS, but not mandated to be signalled.

  - Yes, for sub-pictures (non-layer based approach) but not mandated to be signalled.

  - For sub-pictures with layers approach, CPB is layer-wise.

- Should VVC coding spec specify spatial layout of independent coded regions in the combined picture.

  - Yes, for MCTS and sub-picture non-layer based approach.

  - Yes, but not mandated to be signaled for sub-picture layer based approach.

# Contribution Review

### Coded picture regions (34)

#### Tiles and tile groups (11)

JVET-N0056 AHG12: On tile design [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-N0057 AHG12: On tile group design [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-N0064 AHG17: Signalling of only one tile group per picture [V. Drugeon (Panasonic)]

JVET-N0066 AHG12: On top-to-bottom tile partitioning [Y. He, A. Hamza (InterDigital)]

JVET-N0071 AHG12: Unified signalling of tile grouping information [L. Chen, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0111 AHG12: Flexible tiling [Y.-K. Wang, Hendry, M. Sychev (Huawei)]

JVET-N0124 AHG12: On Tile Group Signalling [S. Deshpande (Sharp)]

JVET-N0348 AHG12: Hierarchical tiling for VVC [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

JVET-N0496 AHG12: Rectangular tile group address signaling [R. Sjöberg, M. Damghanian, M. Pettersson (Ericsson)]

JVET-N0497 AHG12: Dependent tiles [R. Sjöberg, M. Pettersson, M. Damghanian (Ericsson)]

JVET-N0498 AHG12: On Uniform Tile Partitioning [R. Sjöberg, M. Pettersson, M. Damghanian (Ericsson)]

#### Independently coded picture regions (17)

JVET-N0042 AHG12: Comments on MPEG draft requirements on immersive media access and delivery [M. M. Hannuksela, E. B. Aksu, A. Hourunranta (Nokia)]

The contribution categorizes approaches for independently coded picture regions to the following:

- Motion-constrained tile sets (MCTSs). Encoding is constrained in a manner that a rectangular set of tile groups can be extracted and repositioned without affecting their decoding or decoding of other tile groups.

- Independent rectangular tile groups (IRTGs). Encoding are decoding are modified to treat IRTG boundaries like picture boundaries. Implications on IRTG boundaries are handled in low-level decoding process.

- Sub-pictures, which are treated like pictures in the low-level decoding process. Sub-pictures of the same timestamp are spatially arranged onto an output picture.

- Independent sub-picture layers. It can be controlled if decoded pictures of the same timestamp and of different layers are composed onto the same output picture or output separately.

The contribution analyzes how the approaches meet the design goals that have been presented to JVET earlier or are part of MPEG's draft requirements on immersive media delivery and access.:

|  | MCTS / IRTG |  | Sub-pictures | Independent layers |
| --- | --- | --- | --- | --- |
| Design goals summarized in this contribution |  |  |  |  |
| Extraction and repositioning | Fulfilled, with PPS and SPS changes only, provided that tile_group_address is selected uniquely for all possible merging combinations among all the bitstreams from which MCTSs can be merged. |  | Fulfilled, with PPS and SPS changes only. | Fulfilled, with VPS changes only. PPS and SPS changes might be needed if the same IDs had been used for parameter sets of different content. |
| Grid of independently decodable regions not crossing the entire picture | Fulfilled (most proposals) |  | Fulfilled | Fulfilled |
| More efficient than MCTSs in terms of rate-distortion penalty. <This row has dependency on CE12 related documents and conclusion there.> | MCTS: no. It was commented that CE12 proposals bring compression efficiency over existing HEVC style MCTS. | IRTG: fulfilled | Fulfilled (unless specifically constrained) | Fulfilled (unless specifically constrained) |
| Partial intra updating for viewport change in region-wise mixed-quality viewport-dependent 360° streaming | Could be enabled |  | Could be enabled | Fulfilled |
| Partial intra updating for viewport change in region-wise mixed-resolution viewport-dependent 360° streaming | Could be enabled |  | Could be enabled | Fulfilled |
| Partial intra updating with open GOP structures for viewport change in region-wise mixed-quality viewport-dependent 360° streaming <This row has a dependency on ARC related discussion.> | Not possible |  | Not possible | Could be enabled (with reference picture resampling) |
| Level and conformance for a subset of MCTSs / sub-picture sequences originating from the same bitstream | Can be realized e.g. like in HEVC motion-constrained tile sets extraction information sets SEI message and through picture timing enclosed in HEVC motion-constrained tile sets extraction information nesting SEI message. |  | Can be realized e.g. like in HEVC motion-constrained tile sets extraction information sets SEI message and through picture timing enclosed in HEVC motion-constrained tile sets extraction information nesting SEI message | Fulfilled. Layer-wise level and CPB operation. |
| Level and conformance for MCTSs / sub-picture sequences originating from several bitstreams <This is level and conformance for the combined picture.> | Not possible, requires systems support for indicating properties cross multiple bitstreams. <It was commented that such MCTS-wise CPB operation is also possible.> |  | Not possible, requires systems support for indicating properties cross multiple bitstreams. <It was commented that sub-picture wise CPB operation can also be enabled similar to the layer-wise operation.> | Fulfilled. Layer-wise level and CPB operation. |
| MPEG N18134 |  |  |  |  |
| 1. The decoding interface shall allow multiple independent video objects to be decoded <It was commented by a decoder manufacturer that this section of decoder requirements is largely out of scope of VVC decoder design.> | Fulfilled, when encoding MCTSs/IRTGs jointly |  | Fulfilled | Fulfilled |
| 1.1. The decoding interface shall not require rewriting of the bitstream(s) corresponding to these objects, by the user of the decoding resources | Fulfilled, with PPS and SPS changes only, provided that tile_group_address is selected uniquely for all possible merging combinations among all the bitstreams from which MCTSs can be merged. |  | Fulfilled, with PPS and SPS changes only. | Fulfilled, with VPS changes only. PPS and SPS changes might be needed if the same IDs had been used for parameter sets of different content. |
| 1.2. It shall be possible to maintain absolute, frame-level sync between these decoded objects | Fulfilled |  | Fulfilled | Fulfilled |
| 1.3. The maximum number of objects shall be a function of the total number of macroblocks/CTU/samples/pixels per second that the decoding resources can process | Up to profile and level specifications |  | Up to profile and level specifications | Up to profile and level specifications |
| 1.4. It shall be possible to simultaneously decode objects with different parameters: | A single SPS is activated, causing e.g. chroma format to be identical in all objects. |  | A single SPS is activated, causing e.g. chroma format to be identical in all objects. | Fulfilled. A separate SPS may be activated per each layer. |
| i. different object sizes and, for rectangular objects, aspect ratios | Fulfilled only when tile groups "fill in" the picture exactly <It was commented that this could be supported if we add signaling on unoccupied areas.> |  | Could be enabled by allowing unoccupied areas in decoded pictures | Fulfilled |
| ii. different framerates | Not possible <It was commented that this can be enabled with copy slices but may have processing penalty.> |  | Not possible <It was commented that this can be enabled with copy slices but may have processing penalty.> | Fulfilled |
| iii. Different resolutions (sampling density) | Fulfilled |  | Fulfilled | Fulfilled |
| 1.5. The interface shall support decoding objects that have different image parameters, such as color space and bit depth. <An example is for v-pcc with texture is 4:2:0 coded and geometric image is 4:0:0 coded.> | Not possible. <If we define sample arrays in spec to have diff. representation formats then it is possible.> |  | Not possible. <If we allow sub-pics to have different SPS it is possible, however then it becomes almost like a layer. Other option is to signal such info in just a PPS. But output process may need to be defined carefully.> | Fulfilled |
| 1.6. The interface shall support maintaining the association of metadata with the decoded object | Up to how decoder output is specified |  | Up to how decoder output is specified | Up to how decoder output is specified |
| 1.6.1. It shall be possible to keep objects in absolute (frame-level) sync with their associated metadata | Up to how decoder output is specified |  | Up to how decoder output is specified | Up to how decoder output is specified |
| 1.7. The conformance requirements for the decoding interface shall include a well-defined maximum decoding latency for all objects on a per-frame basis | Not (easily) possible, since a single CPB is used <Comment related to mcts-wise cpb operation apply here> |  | Not (easily) possible, since a single CPB is used <Comment related to sub-picture wise cpb operation apply here> | Fulfilled, since layer-wise CPB is used. |
| 2. It shall be possible to do all of the above with protected content | Enabled (potential parts that need rewriting can be left non-encrypted). Outside of VVC scope. |  | Enabled (potential parts that need rewriting can be left non-encrypted). Outside of VVC scope. | Enabled (potential parts that need rewriting can be left non-encrypted). Outside of VVC scope. |
| 2.1. It should be possible to mix protected and non-protected content | Outside of VVC scope. |  | Outside of VVC scope. | Outside of VVC scope. |
| 3. The decoding interface shall allow efficient handling by graphics processors | The approaches are believed to have no crucial differences for handling by graphics processors. |  |  |  |
| JVET-N0042 |  |  |  |  |
| It shall be possible to control whether decoded objects of the same timestamp are output separately or spatially arranged onto the same output picture. | Not possible <MCTS could be outputted to enable this> |  | Not possible <Sub-pictures could be outputted to enable this> | Fulfilled |
| The decoding interface shall provide a possibility to output decoded objects in decoding order (possibly out of presentation order). | Up to how decoder output is specified |  | Up to how decoder output is specified | Up to how decoder output is specified |
| When video bitstreams have a non-contradicting decoding order, the decoding interface shall provide a possibility to output decoded objects in presentation order with frame-level sync across video objects. | Up to how decoder output is specified |  | Up to how decoder output is specified | Up to how decoder output is specified |

It is asserted that the independent sub-picture layer approach:

- provides a wider support of design goals than the other approaches

- does not require changes in the signal processing part of the codec unlike the IRTG approach; and

- has similar implementation cost than the sub-picture approach.

The contribution therefore concludes that the independent sub-picture layer approach is preferred for realizing independently coded picture regions.

Discussion:

It was commented that MCTS and IRTG are similar except how they treat boundaries.

It was asked why layers are needed for independent bitstream operation. It was answered that it is a simple way to multiplex multiple bitstreams into a single bitstream.

It was commented by one participant that it will be useful to specify a level, conformance for the combined bitstream.

It was commented that keeping sub-pictures synchronized in decoded output has issues and pixel rate is not the only issue for combined decode capability. For supporting multiple decodes there is currently no guarantee that their output will be frame synchronized. For example, it may not be easy to specify how many 1080p decodes could be done by a 4k decoder.

It was discussed if spatial relation between objects/ sub-pictures/ mcts should be specified at video layer and/or could it be done at system layer with metadata. There were disparate views on this. With some participants preferring this to be done only at system layer and some preferring to additionally specify it at video layer.

It was agreed to try to make rewriting process for VVC (irrespective of which solution we pick) to be simpler than in HEVC to enable common use cases.

Questions:

- Should VVC coding spec support defining profile/tier/level (PTL) individually for each independent coded region.

Yes, but not mandated to be signalled.

- Should VVC coding spec define information to help derive combined profile/tier/level (PTL) for each combination of independent coded regions.

It was commented by one participant that for MCTS the answer is yes and indeed this is also supported in HEVC for MCTSes.

It was commented by that we don’t need to define PTL for combined mcts/ sub-pictures/ independent coded regions.

It was commented that this does not scale well for “each combination” aspect.

It was also commented that specifying this may not be feasible as it is not as simple as pixel rate.

It was commented that this may be able to specified for constrained encoder.

It was commented that it will be useful to have information which can be used to normatively derive the PTL information for combination.

There is request from system side to enable such ability.

However, it seems currently this may not be easy and no clear solution has been proposed for this yet.

- Should VVC coding spec support HRD model individually for each independent coded region.

Yes, but not mandated to be signalled.

- Should VVC coding spec specify a combined HRD operation (e.g. CPB, DPB operation) for combined independent coded regions.

Yes, for MCTS but not mandated to be signalled.

Yes, for sub-pictures (non-layer based approach) but not mandated to be signalled.

For sub-pictures with layers approach, CPB is layer-wise.

- Should VVC coding spec specify a mechanism for signaling combined HRD parameters for each combination of independent coded regions originating from different bitstreams.

This has the combinatorial issue. The combined parameters can not be obtained without recalculating.

- Do we need only one solution or it is ok to have multiple solutions catered for different use cases.

Preference is to specify just one solution, however if it is not possible then revisit.

- Should VVC coding spec specify an output process for combined picture

- Should VVC coding spec specify spatial layout of independent coded regions in the combined picture.

Yes, in MCTS and sub-picture non-layer based approach.

Yes, but not mandated to be signalled for sub-picture layer based approach.

# Design questions (From M0753)

Note: the contribution references are likely to be incomplete.

## On extraction, repositioning, and merging

Background: "Extracting VCL NAL units of a subset of MCTSs from one VVC bitstream and reposition them to another VVC bitstream without substantial difficulty (e.g., without VLC-level modifications)" has already been agreed as a design goal.

- Should the VVC design support merging of independent picture regions originating from different bitstreams?

- Yes

- No

Yes, if can be done without substantial difficulty.

## On region dimensions and layout

- Should we make specification changes to support width and height of independent picture regions that are not multiples of CTUs?

- No

- Yes (N0055). (Use case: matching of independent picture region width/height with face or face row width/height of 360 video)

- Are the decoded independent picture regions of the same POC arranged to a single decoded picture? <No agreed common answer.>

- Yes, always

- Bitstream parameters control if sub-pictures are arranged to a single decoded picture or output separately. (N0045)

- No, sub-pictures are output separately. (N0278)

- When independent picture regions of the same POC are decoded to a single picture, do they have to appear in a grid crossing the entire picture (similar to tiles)?

- Yes

- No (N0044, N0045, N0046, …) but this point may not be very important.

- [skip] When independent picture regions of the same POC are arranged to a single decoded picture, is it required that an independent picture region is used as prediction reference is collocated with the current independent picture region?

- Yes (N0073)

- No

- [skip] When independent picture regions of the same POC are arranged to a single decoded picture, is the decoded picture entirely occupied by independent picture regions?

- Yes

- No (N0046, N0073)

- [Too detailed so skip] What information should be included to indicate a layout when independent picture regions of the same POC are arranged to a single decoded picture?

- Sub-picture sizes and locations within the decoded picture. (N0073, N0107)

- Sub-picture sizes only, the layout is determined by the decoding order of sub-pictures: the sub-picture takes the next such unoccupied location in CTU raster scan order within a decoded picture that is large enough to fit the sub-picture (N0046)

- Independent coded region boundary IDs, a pair of matching identifiers expresses that two of independent coded regions are neighbours along a certain border (N0411)

- [Too detailed so skip] Location to indicate region dimensions and layout?

- VPS

- SPS

- PPS

- Tile group header

- SEI

JVET-N0043 AHG12: VVC design goals related to MPEG requirements on immersive media access and delivery [M. M. Hannuksela, E. B. Aksu (Nokia)]

JVET-N0044 AHG12: Comparison of approaches for independently coded picture regions [M. M. Hannuksela, A. Aminlou, K. Kammachi-Sreedhar (Nokia)]

JVET-N0045 AHG12: Sub-picture layers for realizing independently coded picture regions [M. M. Hannuksela, K. Kammachi-Sreedhar, A. Aminlou (Nokia)]

JVET-N0046 AHG12: Sub-picture-based picture partitioning and decoding [M. M. Hannuksela, A. Aminlou (Nokia)]

JVET-N0055 AHG12: On sub-picture design [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-N0073 AHG17: [SYS-VVC] Signalling subpicture coded video sequence [L. Chen, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0099 AHG12/AHG17 On sub-picture parameter set [Y. He, A. Hazma (InterDigital)]

JVET-N0107 Sub-picture based video coding [Y.-K. Wang, Hendry, J. Chen (Huawei)]

JVET-N0191 [AHG17/AHG12] On APS id for bitstream merging for VVC [N. Ouedraogo, E. Nassor, J. Taquet, G. Kergourlay, F. Maze (Canon)]

JVET-N0192 [AHG17/AHG12] Bitstream extraction and merging with variable initial Qp [N. Ouedraogo, E. Nassor, G. Kergourlay, F. Maze (Canon)]

JVET-N0278 AHG17: On VVC HLS relevant to MPEG requirements on immersive media delivery and access (N18134) [J. Boyce (Intel)] [late]

This contribution discusses how the VVC high level syntax can address some of the issues raised in M46578, “On the decoding interface for Immersive Media.” The proposals can be summarized as follows:

- Include bitstream and decoder support for multiple independent layers in first version of VVC

- 7-bit nuh_layer_id syntax element in the two-byte NAL Unit header

- Introduce a Video Parameter Set (VPS), possibly combined with the DPS, with the following features:

  - Indication of the maximum number of layers

  - Flag to indicate that all layers are independent, using “reserved” name

  - Tool constraint flags in the general_constraint_info( ) syntax structure signalled once in the VPS and applied to all layers in the bitstream, rather than signalled per layer in the profile_tier_level( ) syntax structure

  - The –v2 version of this document adds a loop of syntax elements vps_included_layer_id[i]

- The –v2 version of this document updates specification text for the sub-bitstream extraction process to extract a single layer, LIdTarget.

vps_included_layer_id[ i ] specifies layers that may be present and is asserted to help invoking decoding process for layers and sub-bitstream extraction.

The value of 1 for vps_reserved_zero_1bit may be used to indicate support for inter-layer prediction in a future extension.

VPS does not have total combined PTL, and each layer has its own PTL.

In future HRD needs to be specified.

No decoding process is specified for combination of layers. System layer could describe how a combined picture presentation can be done. Some SEI message could also carry such description. But this is not mandated here now.

The asserted benefit is all the sub-bitstreams are interleaved together and have separate layer id and timing.

It was commented that interleaving would need to be deinterleaved at client in typical implementations.

It was commented that term layer may be changed to something like representation or something else. This aspect of using a different term was agreed by the proponent.

If layer id values in two bitstreams were same then layer id will need to be rewritten to be different.

It was commented that current VVC WD has u(v) coded fields which are also easier to parse.

There is no ability to specify filtering across edges in this approach. Out of loop post filtering could be done.

It was commented that MCTS can co-exist with this approach.

Each layer can have different representation format. It was asked how such diverse layers can be handled by one decoder. It was answered that it is possible by today’s decoders e g. by time sharing of decoder. Decoding process is currently not defined. But for spec it is expected that decoding process for each layer is invoked separately.

There is general_constraint_info() included. It is expected that the constraint syntax currently in PTL will instead be in VPS and it is a union of all those constraints. When combining bitstreams where one has for example a tool on and another has it off, something needs to be done for SPS signaling.

Will there be a profile supporting only one layer or should main profile support multiple layers? Proponent answered that it is not attempted to define a combined PTL for everything in VVC spec. Maybe this can be done by system spec.

VVC spec may just require decoding one layer but it may have layer id other than 0 and it needs to be able to handle having NALUs with other layer ids in the bitstream. This needs some HRD aspect study.

Would all the layers start at the same time. The syntax does not limit this but the topic could be further studied for HRD and related aspects.

SPS and PPS could be reused across layers. Another contribution (JVET-N0045) exists which treats layer id 0 specially and allows sharing.

If used together with MCTS each layer bitstream itself could be rewritten.

It was commented that this approach is bigger than sub-picture aspect and should be dealt with as such.

Enhancement layers (e.g. spatial scalable or view scalable or anything inter-layer predicted) will be included in extension.

JVET-N0354 AHG12: Conformance of temporally independent tile groups [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

JVET-N0411 AHG12: Layout signalling of independent coded regions [E. Thomas (TNO)]

JVET-N0528 AHG12/AHG19/SYS-VVC: Sub-picture decoding design: a comparison of proposed solutions [E. Thomas (TNO)] [late]

JVET-N0047 AHG12/AHG17: Merging IRAP and non-IRAP VCL NAL units into the same coded picture [M. M. Hannuksela (Nokia)]

JVET-N0108 AHG12/AHG17: Allowing mixed IRAP and non-IRAP NAL unit types within a picture [Y.-K. Wang, Hendry (Huawei)]

JVET-N0753 AHG12: Design decision questions on independently coded picture regions [M.M. Hannuksela (Nokia), S. Deshpande (Sharp)]

#### Wavefront parallel processing (5)

JVET-N0060 AHG12: Improved parallel processing capability with WPP [Y. Fujimoto, M. Ikeda, T. Suzuki (Sony)]

JVET-N0559 Crosscheck of JVET-N0060 (AHG12: Improved parallel processing capability with WPP) [T. Ikai (Sharp)] [late]

JVET-N0149 AHG12: Signalling on wavefront parallel processing [T. Ikai, S. Deshpande, T. Chujoh, E. Sasaki, T. Aono (Sharp)] [late]

JVET-N0150 AHG12: One CTU delay wavefront parallel processing [T. Ikai (Sharp)]

JVET-N0646 Crosscheck of JVET-N0150 (AHG12: One CTU delay wavefront parallel processing) [M. Ikeda (Sony)] [late]