| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0755 |
| --- | --- |

| Title: | Crosscheck of JVET-N0113 (AHG16: Subblock-based chroma residual scaling) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jinho Lee Sung-Chang Lim Hahyun Lee Jungwon Kang 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr sclim@etri.re.kr hanilee@etri.re.kr jungwon@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution reports crosscheck results of subblock-based chroma residual scaling proposed in JVET-N0113. Method 1 is proposed to reduce delay of chroma reconstruction by subblock-based chroma residual scaling, and Method 2 is proposed to reduce complexity of Method 1. It is reported that the average luma BD-rate of the Method 1 and Method 2 for RA case is 0.00% and 0.01% compared to VTM 4.0, respectively.

- Introduction

In VTM 4.0, chroma residual coding is applied to chroma TBs by using average value of corresponding luma prediction samples. Therefore, chroma reconstruction process is delayed until the entire corresponding luma block is available. To reduce the delay, Method 1 in JVET-N0113 proposes subblock-based chroma residual scaling. In addition, Method 2 only uses one luma region to calculate chroma residual scale value, and applies the scale value to entire chroma blocks to reduce complexity.

Experimental results

Experiments were performed based on VTM 4.0 software under JVET common test condition.

Table 1 and Table 2 show experimental results of the Method 1 and Method 2 compared to VTM 4.0, respectively. All the results are matched with the ones in JVET-N0113.

Table 1. Results of the Method 1 over VTM 4.0

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.00% | 100% | 101% |
| Class A2 | -0.02% | 0.00% | -0.04% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.08% | 101% | 101% |
| Class C | 0.00% | 0.15% | 0.07% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | 0.04% | 100% | 101% |
| Class D | 0.01% | 0.06% | 0.02% | 101% | 100% |
| Class F | 0.01% | 0.02% | -0.05% | 101% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.18% | 0.18% | 101% | 101% |
| Class C | 0.00% | 0.24% | 0.22% | 101% | 101% |
| Class E | 0.02% | 0.36% | -0.17% | 101% | 101% |
| Overall | 0.01% | 0.25% | 0.11% | 101% | 101% |
| Class D | -0.03% | -0.15% | -0.46% | 101% | 101% |
| Class F | -0.09% | -0.16% | 0.02% | 101% | 101% |

Table 2. Results of the Method 2 over VTM 4.0

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.09% | 0.04% | 100% | 100% |
| Class A2 | -0.01% | -0.07% | 0.00% | 100% | 100% |
| Class B | 0.01% | -0.04% | 0.02% | 101% | 100% |
| Class C | 0.02% | -0.02% | -0.10% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.02% | -0.01% | 100% | 100% |
| Class D | 0.00% | 0.07% | 0.09% | 101% | 101% |
| Class F | -0.01% | 0.02% | -0.02% | 101% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.15% | -0.24% | 100% | 101% |
| Class C | -0.03% | 0.18% | 0.13% | 101% | 101% |
| Class E | 0.00% | 0.33% | -0.52% | 101% | 100% |
| Overall | -0.01% | 0.08% | -0.19% | 101% | 101% |
| Class D | -0.02% | 0.23% | -0.30% | 101% | 101% |
| Class F | 0.00% | 0.15% | 0.16% | 101% | 101% |