| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0772 |
| --- | --- |

| Title: | Cross-check of JVET-N0311 and JVET-0600 (Context Modeling Simplification and Reduction in VVC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Muhammed Coban | Tel: Email: | +1 858 658 3973 mcoban@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document reports the crosscheck results of “Context Modeling Simplification and Reduction in VVC (JVET-N0311, JVET-N0600)”. The source code was analyzed, and it matches the description provided in JVET-N0311. The results provided by the proponent were verified.

# Crosscheck results

The simulation was carried out under the JVET common test condition [1]. Technicolor provided the source code of their proposal and results [5]. The results match those provided by the proponent.

# Results

The proposed scheme is implemented on top of VTM-4.0 software [2].  The simulations were carried out under the JVET common test conditions [1] [3].

The test results 24ctx for test case for JVET common test conditions is tabulated in Table 1.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0-(CTC+TGM(IBC1)) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.31% | 0.36% | 99% | 111% |
| Class A2 | 0.01% | 0.47% | 0.35% | 98% | 114% |
| Class B | 0.00% | 0.23% | 0.25% | 81% | 93% |
| Class C | 0.01% | 0.12% | 0.14% | 81% | 93% |
| Class E | 0.01% | 0.25% | 0.12% | 80% | 93% |
| Overall | 0.00% | 0.26% | 0.24% | 86% | 99% |
| Class D | 0.02% | 0.01% | -0.11% | 81% | 91% |
| Class F | 0.04% | 0.07% | 0.11% | 83% | 91% |
| Class TGM | 0.05% | 0.12% | 0.12% | 112% | 110% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0-(CTC+TGM(IBC1)) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.46% | 0.18% | 107% | 120% |
| Class A2 | -0.02% | 0.10% | 0.23% | 104% | 115% |
| Class B | 0.00% | 0.08% | -0.03% | 81% | 94% |
| Class C | 0.00% | 0.10% | 0.10% | 83% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.16% | 0.10% | 91% | 103% |
| Class D | 0.00% | -0.35% | -0.30% | 83% | 96% |
| Class F | -0.01% | 0.15% | -0.04% | 83% | 98% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0-(CTC+TGM(IBC1)) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.08% | -0.33% | 81% | 94% |
| Class C | -0.05% | 0.30% | 0.05% | 80% | 92% |
| Class E | -0.07% | 1.75% | 0.78% | 82% | 96% |
| Overall | -0.04% | 0.51% | 0.07% | 81% | 94% |
| Class D | 0.00% | 0.85% | -0.23% | 82% | 91% |
| Class F | -0.11% | 0.33% | -0.14% | 84% | 96% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 1: Average Bjøntegaard delta rate savings of the proposed method (24ctx) relative to VTM-4.0.

The test results 54ctx for test case for JVET common test conditions is tabulated in Table 2.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0-(CTC+TGM(IBC1)) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.30% | 0.42% | 101% | 114% |
| Class A2 | 0.05% | 0.42% | 0.35% | 101% | 118% |
| Class B | 0.02% | 0.17% | 0.13% | 82% | 92% |
| Class C | 0.04% | 0.01% | 0.05% | 83% | 92% |
| Class E | 0.00% | 0.27% | 0.02% | 82% | 93% |
| Overall | 0.03% | 0.22% | 0.18% | 88% | 99% |
| Class D | 0.04% | 0.00% | -0.10% | 83% | 91% |
| Class F | -0.03% | 0.06% | -0.02% | 84% | 90% |
| Class TGM | -0.04% | 0.11% | 0.11% | 115% | 111% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0-(CTC+TGM(IBC1)) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.13% | 0.22% | 108% | 117% |
| Class A2 | 0.03% | 0.17% | 0.26% | 105% | 118% |
| Class B | -0.01% | 0.03% | 0.08% | 82% | 94% |
| Class C | 0.01% | -0.06% | -0.05% | 84% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.05% | 0.11% | 91% | 104% |
| Class D | 0.02% | -0.12% | -0.05% | 84% | 97% |
| Class F | 0.00% | -0.01% | 0.07% | 84% | 98% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0-(CTC+TGM(IBC1)) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.11% | 0.08% | 81% | 94% |
| Class C | -0.08% | 0.22% | -0.25% | 81% | 94% |
| Class E | 0.04% | 0.30% | -0.11% | 82% | 92% |
| Overall | -0.04% | 0.10% | -0.08% | 81% | 94% |
| Class D | -0.04% | 0.67% | -0.37% | 82% | 89% |
| Class F | -0.21% | -0.12% | -0.46% | 84% | 98% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 2: Average Bjøntegaard delta rate savings of the proposed method (54ctx) relative to VTM-4.0.

The test results 66ctx for test case for JVET common test conditions is tabulated in Table 3.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0-(CTC+TGM(IBC1)) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.33% | 0.35% | 102% | 112% |
| Class A2 | 0.07% | 0.46% | 0.35% | 101% | 115% |
| Class B | 0.04% | 0.15% | 0.16% | 83% | 93% |
| Class C | 0.06% | 0.07% | 0.06% | 84% | 96% |
| Class E | 0.02% | 0.16% | 0.20% | 82% | 92% |
| Overall | 0.05% | 0.22% | 0.21% | 89% | 100% |
| Class D | 0.05% | -0.02% | -0.01% | 84% | 93% |
| Class F | -0.02% | 0.06% | 0.07% | 86% | 91% |
| Class TGM | 0.01% | 0.18% | 0.13% | 116% | 111% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0-(CTC+TGM(IBC1)) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.38% | 0.22% | 109% | 118% |
| Class A2 | 0.01% | 0.13% | 0.14% | 106% | 117% |
| Class B | 0.02% | 0.22% | 0.03% | 82% | 94% |
| Class C | 0.05% | 0.05% | -0.02% | 84% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.19% | 0.08% | 92% | 104% |
| Class D | 0.03% | -0.21% | -0.08% | 84% | 97% |
| Class F | -0.02% | 0.12% | 0.00% | 84% | 98% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0-(CTC+TGM(IBC1)) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.04% | -0.47% | 81% | 93% |
| Class C | -0.08% | 0.13% | 0.02% | 82% | 94% |
| Class E | 0.06% | 0.82% | 0.05% | 83% | 95% |
| Overall | -0.04% | 0.23% | -0.18% | 82% | 94% |
| Class D | -0.08% | 0.08% | -0.33% | 83% | 91% |
| Class F | -0.20% | -0.27% | -0.14% | 84% | 98% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 3: Average Bjøntegaard delta rate savings of the proposed method (66ctx) relative to VTM-4.0.

The test results 83ctx for test case for JVET common test conditions is tabulated in Table 3.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0-(CTC+TGM(IBC1)) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.26% | 0.33% | 101% | 114% |
| Class A2 | 0.12% | 0.51% | 0.35% | 101% | 118% |
| Class B | 0.08% | 0.15% | 0.13% | 83% | 94% |
| Class C | 0.10% | -0.02% | 0.16% | 83% | 93% |
| Class E | 0.06% | 0.19% | 0.05% | 83% | 93% |
| Overall | 0.09% | 0.20% | 0.19% | 89% | 100% |
| Class D | 0.10% | 0.14% | -0.15% | 83% | 90% |
| Class F | 0.04% | 0.12% | 0.15% | 84% | 91% |
| Class TGM | 0.11% | 0.26% | 0.23% | 115% | 111% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0-(CTC+TGM(IBC1)) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.24% | 0.21% | 107% | 119% |
| Class A2 | 0.11% | 0.13% | 0.02% | 105% | 117% |
| Class B | 0.05% | 0.21% | 0.06% | 82% | 93% |
| Class C | 0.08% | 0.09% | -0.15% | 83% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.17% | 0.03% | 91% | 103% |
| Class D | 0.07% | -0.08% | -0.10% | 85% | 96% |
| Class F | 0.04% | 0.02% | 0.26% | 84% | 98% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0-(CTC+TGM(IBC1)) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.27% | -0.13% | 82% | 96% |
| Class C | -0.04% | 0.12% | -0.09% | 82% | 93% |
| Class E | 0.09% | 0.65% | 0.63% | 82% | 94% |
| Overall | 0.00% | 0.09% | 0.07% | 82% | 94% |
| Class D | -0.10% | 0.21% | -0.04% | 83% | 91% |
| Class F | -0.05% | 0.02% | -0.57% | 85% | 97% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 4: Average Bjøntegaard delta rate savings of the proposed method (83ctx) relative to VTM-4.0.

# References

- H. Schwarz, M. Coban, C. Auyeung, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding,” JVET-M1027, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, January 2019.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4)”, JVET-M1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, January 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git

- “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, January 2019.

- Y. Chen, F. Le Leannec, T. Poirier (Technicolor), “Context Modeling Simplification and Reduction in VVC “, JVET-N0311, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).