| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0785 |
| --- | --- |

| Title: | Cross-check of JVET-N0493: CE1-related: Multiplication-free bilateral loop filter; with additional tests on RDO |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | K.Reuze, D.Rusanovskyy 10160 Pacific Mesa Blvd San Diego, CA 92121 (USA) | Email: | dmytror@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This document reports the crosscheck results for the Bilateral Filter (BIF) at the “loop filter” position, test CE1-related input JVET-N0493. Additional test results are reported by cross-checker for design of JVET-N0493 w/o filtering during the RDO process.

# Results

The method proposed in N0493 deploys proposed BIF design at the loop filter position, BIF is applied as conventional in-loop filter prior to deblocking - (no restriction on block size or block type, samples outside the block can be used). For the test1, proponent reported coding gain of 0.46% / 0.60% / 0.58% of BD-rate reduction for AI/RA/LDB.

For partially completed tests, cross-checker was able to reproduce LDB results reported by the proponent. However, for AI/RA results there is a minor deviation is observed compare to the results reported by proponent. Comparative results of cross-checker results vs results reported by the proponent:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over N0493 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.03% | -0.02% |  |  |
| Class C | 0.00% | 0.05% | -0.02% |  |  |
| Class E | 0.01% | -0.01% | 0.08% |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.01% | 0.11% | 0.03% |  |  |
| Class F (optional) |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over N0493 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.01% | -0.15% | -0.07% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.02% | -0.03% | -0.17% |  |  |
| Class F (optional) | 0.00% | 0.00% | 0.00% |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over N0493 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.00% | 0.00% | 0.00% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% |  |  |
| Class F (optional) |  |  |  |  |  |

At the same time, method proposed N0493 utilizes BIF during the RDO process. This is an encoder optimization algorithm for BIF, which is similar to the method proposed in JVET-M0428 for deblocking filter. Simulation results reported in JVET-M0428, shows that coding gain 0.58%/0.71%/0.66% was achieved by this encoder optimization technique. The method of JVET-M0428 was adopted to the VTM4.0 software but was not enabled in the CTC. It is argued by the cross-checker, that for fairness of comparisons, simulation results reported in CE1-2.3 with encoder optimization should be compared against the encoder optimization for deblocking. Alternatively, RDO for BIF can be disabled for comparison against the VTM4.0 anchor.

To assess performance of the BIF in conditions similar to deblocking filter (no filtering during the RDO optimization), cross-checker run software by disabling encoder BIF filtering in RDO.

Achieved simulation results of BIF w/o RDO vs. VTM4.0 anchor, are reported below.

Note: reported time estimates are in-accurate.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over inloop EBIF, RDO off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.00% | 0.01% | 100% | 115% |
| Class A2 | -0.05% | 0.06% | 0.02% | 97% | 107% |
| Class B | 0.00% | -0.04% | -0.03% | 100% | 118% |
| Class C | -0.31% | -0.06% | -0.06% | 100% | 114% |
| Class E | -0.19% | -0.03% | -0.03% | 100% | 121% |
| Overall | -0.10% | -0.02% | -0.02% | 100% | 115% |
| Class D | -0.21% | -0.03% | -0.03% | 100% | 112% |
| Class F (optional) |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over inloop EBIF, RDO off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 | 0.19% | -0.20% | -0.13% | 95% | 100% |
| Class B | -0.02% | -0.18% | -0.10% | 99% | 103% |
| Class C | -0.18% | -0.17% | -0.29% | 101% | 106% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.25% | -0.36% | -0.32% | 101% | 105% |
| Class F (optional) |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over inloop EBIF, RDO off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.03% | 0.27% | 0.03% | 100% | 100% |
| Class E | 0.57% | 0.65% | 0.28% | 101% | 101% |
| Overall |  |  |  |  |  |
| Class D | -0.28% | 0.22% | -0.22% | 101% | 112% |
| Class F (optional) |  |  |  |  |  |