| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0823 |
| --- | --- |

| Title: | AHG17: First octet of NAL unit header without forbidden_zero_bit |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger, BD Choi (Tencent) Miska M. Hannuksela, Kashyap Kammachi-Sreedhar (Nokia) | Email: | stewe@stewe.org miska.hannuksela@nokia.com |
| Source: | Nokia, Tencent |  |  |

_____________________________

# Abstract

This contribution is a hybrid between JVET-N0051 (Tencent), JVET-N0067 (Nokia), as well as ideas and opinions captured duing the discussion in the HLS Breakout meeting that took place March 19 afternoon and March 20, and also addresses the concerns voiced in JVET-N0050 (Tencent).  Specifically, proposed is

a) to replace the forbidden_zero_bit as the mechanism to avoid MPEG-2 stream layer PES start code emulation with the mechanism aligned with the spirit of JVET-N0067 (basically disallowing values for the first octet of the Nal Unit header between 188 and 255);

b) using the bits previously employed for temporal ID (TID) as a demultiplexing point for those NAL unit types where temporal ID is meaningless, according to JVET-N0051;

c) creating NAL unit types for reference and non-reference trailing and STSA picture NAL units (according to real-time discussions in the HLS break-out, March 20 in the morning). The semantics of the non-reference trailing and STSA NAL units are left outside of the scope of this contribution.

This document focusses on syntax and semantics on the proposed solution; for justification and discussion please see the aforementioned documents and the HLS breakout meeting reports.  Note that the documents assumes the APS having a meaningful TID, and it also proactively reserves NAL unit types for VPS and DPS even if those are not yet agreed.

Assigned so far are 10 NAL unit types (NUTs) where the TID matters, (reference/non-reference trailing/STSA NUTs, RASL/RADL NUTs, AUD, APS, prefix and postfix SEIs),  9 NUTs where TID does not matter (parameter sets, markers).  Further we propose to set aside 8 codepoints NUTs as “unspecified” for external use.

Proposed herein is the interpretation of the first octet of the NAL unit header (NUH).  Whether or not a second or more octets in the NUH are necessary for all or some NUHs is left open.

# Proposed specification text

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| nuh_first_octet | u(8) |
| … // remaining part of the NAL unit header |  |
| } |  |

nuh_first_octet is interpreted as follows:

nuh_first_octet equal to 0 is reserved.  Informative note: reserved to avoid Annex B start code emulation

nuh_first_octet beween 180 and 187, inclusive, are unspecified, and a decoder shall discard NAL units with that value. Informative note: External specification can use these NUTs  for their purposes.  The specification does not guaratee conflict resolution between applications using such NAL units with unspecified NAL types.

nuh_first_octet between 188 through 255 inclusive (1011 1100b through 1111 1111b inclusive) are reserved.  Informative note: reserved to avoid ITU-T H.222/MPEG-2 part 1 PES start code emulation.

The remaining values of nuh (1..180) are interpreted as follows:

Informative Note: the text below effectively leads to two variants of the first octet of the NAL unit header:

- In the first format, the 5 most significant bits identify a nal unit type and the remaining three bits identify the temporal ID associated with that NAL unit.  This first format is used for NAL unit types where the temporal ID carries significance.

- In the second format, all 8 bits identify the nal unit type, and no temporal ID is carried in the bitstream for that NAL unit.  This second format is used for NAL unit types which conceptually do not have an associated temporal ID, or where the temporal ID can be inferred, as the case may be.

Let the variable NalUnitTypeCategory = nuh_first_octet >> 3	// five MSBs

The variable NalUnitType is derived as follows:

- If NalUnitTypeCategory is between 0 and 15, inclusive, NalUnitType is set equal to NalUnitTypeCategory, and TemporalID is set equal to ( nuh_first_octet & 7 ) − 1, and NalUnitType is interpreted as per table xxx1.

- Otherwise, if NalUnitTypeCategory is between 16 and 18 inclusive, NalUnitType is set to nuh_first_octet, TemporalId is set equal to 0, and NalUnitType is interpreted as per table xxx2.

- Otherwise, NalUnitType is set to nuh_first_octet, and TemporalId is set equal to 0.

The values of NalUnitType in the range of 152 to 165, inclusive, are reserved non-VCL NAL units. The values of NalUnitType in the range of 166 to 179, inclusive, are reserved VCL NAL units.

Replace all occurrences of nal_unit_type with NalUnitType in VVC draft 4.

Table xxx1: NALUnitTypeCategory to NAL unit type mapping for NalUnitTypeClass in the range of 0 .. 15

| NalUnit Type Category | Name of NalUnitType | nuh_first_octet | Content of NAL unit and RBSP structure | NAL unit type class |
| --- | --- | --- | --- | --- |
| 0 | AUD_NUT | 00000 TID | Access unit delimiter access_unit_delimiter_rbsp( ) | Non VCL |
| 1 | TRAIL_R_NUT | 00001 TID | Coded tile group of non-STSA trailing picture, reference, tile_group_layer_rbsp( ) | VCL |
| 2 | TRAIL_N_NUT | 00010 TID | Coded tile group of a non- STSA trailing picture, non-reference tile_group_layer_rbsp( ) | VCL |
| 3 | STSA_R_NUT | 00011 TID | Coded tile group of an STSA picture tile_group_layer_rbsp( ) | VCL |
| 4 | STSA_N_NUT | 00100 TID | Coded tile group of an STSA picture tile_group_layer_rbsp( ) | VCL |
| 5 | RASL_NUT | 00101 TID | Coded tile group of a RASL picture tile_group_layer_rbsp( ) | VCL |
| 6 | RADL_NUT | 00110 TID | Coded tile group of a RADL picture tile_group_layer_rbsp( ) | VCL |
| 7..9 | RSV_VCL_7 .. RSV_VCL_9 | 00111 TID .. 01001 TID | Reserved non-IRAP VCL NAL unit types | VCL |
| 10 | APS_NUT | 01010 TID | Adaptation parameter set adaptation_parameter_set_rbsp( ) | Non VCL |
| 11 | PREFIX_SEI_NUT | 01011 TID | Supplemental enhancement information sei_rbsp( ) | Non VCL |
| 12 | POSTFIX_SEI_NUT | 01100 TID | Supplemental enhancement information sei_rbsp( ) | Non VCL |
| 13..15 | RSV_NVCL_13 .. RSV_NVCL_15 | 01101 TID .. 01111 TID | Reserved non-VCL NAL unit types | Non VCL |

Table xxx2: Mapping of NalUnitTypeCategory to NalUnitType, for NalUnitTypeCategory 16..18

| NalUnit Type Category | Nuh_first_ octet & 0x07 | nuh_first_ octet | Name of NAL Unit Type | Content of NAL Unit Type and RBSP structure | NAL Unit Type Class |
| --- | --- | --- | --- | --- | --- |
| 16 | 1 | 10000 001 | IDR_W_RADL | Coded tile group of an IDR picture tile_group_layer_rbsp( ) | VCL |
| 16 | 2 | 10000 010 | IDR_N_LP | Coded tile group of an IDR picture tile_group_layer_rbsp( ) | VCL |
| 16 | 3 | 10000 011 | CRA_NUT | Coded tile group of a CRA picture tile_group_layer_rbsp( ) | VCL |
| 16 | 0, 4 .. 7 | 10000 000, 10000 100 .. 10000 111 | RSV_TID0_VCL_0, RSV_TID0_VCL_4 .. RSV_TID0_VCL_7 | Reserved VCL NAL unit types | VCL |
| 17 | 1 | 10001 001 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | Non VCL |
| 17 | 2 | 10001 010 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | Non VCL |
| 17 | 3 | 10001 011 | VPS_NUT | TBD | Non VCL |
| 17 | 4 | 10001 100 | DPS_NUT | TBD | Non VCL |
| 17 | 5 | 10001 101 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | Non VCL |
| 17 | 6 | 10001 110 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | Non VCL |
| 17 | 0, 7 | 10001 000, 10001 111 | RSV_TID0_NVCL_0, RSV_TID0_NVCL_7 | Reserved non-VCL NAL unit types | Non VCL |
| 18 | 0 .. 3 | 10010 000 .. 10010 011 | RSV_TID0_VCL_A .. RSV_TID0_VCL_D | Reserved VCL NAL unit types | VCL |
| 18 | 4 .. 7 | 10010 100 .. 10010 111 | RSV_TID0_NVCL_A .. RSV_TID0_NVCL_D | Reserved non-VCL NAL unit types | Non VCL |

# Patent rights declaration(s)

Nokia and Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).