| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0823v2 |
| --- | --- |

| Title: | AHG17: First octet of NAL unit header without forbidden_zero_bit |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger, BD Choi (Tencent) Miska M. Hannuksela, Kashyap Kammachi-Sreedhar (Nokia) | Email: | stewe@stewe.org miska.hannuksela@nokia.com |
| Source: | Nokia, Tencent |  |  |

_____________________________

# Abstract

This contribution is a hybrid between JVET-N0051 (Tencent), JVET-N0067 (Nokia), as well as ideas and opinions captured duing the discussion in the HLS Breakout meeting that took place March 19 afternoon and March 20, and also addresses the concerns voiced in JVET-N0050 (Tencent).  Specifically, proposed is

a) to replace the forbidden_zero_bit as the mechanism to avoid MPEG-2 stream layer PES start code emulation with the mechanism aligned with the spirit of JVET-N0067 (basically disallowing values for the first octet of the Nal Unit header between 188 and 255);

b) using the bits previously employed for temporal ID (TID) as a demultiplexing point for those NAL unit types where temporal ID is meaningless, according to JVET-N0051.

This document focusses on syntax and semantics on the proposed solution; for justification and discussion please see the aforementioned documents and the HLS breakout meeting reports.  Note that the documents assumes the APS having a meaningful TID, and it also proactively reserves NAL unit type for DPS even if it is not yet agreed.

Assigned so far are 8 NAL unit types (NUTs) where the TID matters, (trailing, STSA, RASL, RADL VCL NUTs, AUD, APS, prefix SEI and postfix SEI non-VCL NUTs),  8 NUTs where TID does not matter (parameter sets, markers).  Further we propose to set aside 8 codepoints NUTs as “unspecified” for external use.

Proposed herein is the interpretation of the first octet of the NAL unit header (NUH).  Whether or not a second or more octets in the NUH are necessary for all or some NUHs is left open.

The following changes were implemented in version 2 of the contribution:

- A range of reserved VCL NAL unit types was assigned to IRAP VCL NAL unit types, and another range was assigned to non-IRAP VCL NAL unit types.

- Non-reference trailing and STSA picture NAL unit types as well as VPS NAL unit type were excluded, as agreed in the HLS BoG review of version 1 of the contribution.

- Editorial changes were done, with the intent to align to the style used in the VVC specification.

- VVC Draft 4 specification text on nal_unit_type and nuh_temporal_id_plus1 was incorporated as much as possible. For example, informative notes were carried over. The proposed text is now asserted to be suitable for replacing the entire semantics of nal_unit_type and nuh_temporal_id_plus1.

# Proposed specification text

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| nuh_first_octet | u(8) |
| … // remaining part of the NAL unit header |  |
| } |  |

nuh_first_octet specifies the type of RBSP data structure contained in the NAL unit and a temporal identifier for the NAL unit as specified in Tables XYZ1, XYZ2, and XYZ3. [Ed. (MH): If the nal_unit_header( ) won't contain more than one byte, nuh_first_octet could be renamed e.g. to nuh_octet.]

nuh_first_octet shall not be equal to 0 in bitstreams conforming to this version of this Specification.

NOTE 1 – nuh_first_octet equal to 0 is disallowed to avoid interpreting it as a part of a start code prefix according to Annex B.

NAL units that have nuh_first_octet in the range of 180..187, inclusive, for which semantics are not specified, shall not affect the decoding process specified in this Specification.

NOTE 2 – NAL unit types in the range of 180..187 may be used as determined by the application. No decoding process for these values of nuh_first_octet is specified in this Specification. Since different applications might use these NAL unit types for different purposes, particular care must be exercised in the design of encoders that generate NAL units with these nuh_first_octet values, and in the design of decoders that interpret the content of NAL units with these nuh_first_octet values. This Specification does not define any management for these values. These nuh_first_octet values might only be suitable for use in contexts in which "collisions" of usage (i.e., different definitions of the meaning of the NAL unit content for the same nuh_first_octet value) are unimportant, or not possible, or are managed – e.g., defined or managed in the controlling application or transport specification, or by controlling the environment in which bitstreams are distributed.

nuh_first_octet shall not be in the range of 188 to 255, inclusive, in bitstreams conforming to this version of this Specification.

NOTE 3 – nuh_first_octet values in the range of 188 to 255, inclusive are disallowed to avoid emulating Program Elementary Stream start codes specified in ITU-T H.222.0 | ISO/IEC 13818-1.

NOTE 4 – There are effectively two variants of the internal structure of nuh_first_octet:

- In the first format, the five most significant bits identify a NAL unit type and the remaining three bits identify the TemporalId associated with that NAL unit. This first format is used for NAL unit types where TemporalId carries significance or where non-zero TemporalId values are possible.

- In the second format, all eight bits identify the NAL unit type, and no TemporalId is carried in the bitstream for that NAL unit. This second format is used for NAL unit types for which TemporalId is inferred to be equal to 0.

The variable NalUnitTypeCategory is derived as follows:

NalUnitCategory = nuh_first_octet >> 3

The variables NalUnitType and TemporalId are derived from nuh_first_octet value as specified in Table XYZ1.

[Ed. (MH): Replace all occurrences of nal_unit_type with NalUnitType in VVC draft 4.]

The semantics and of NalUnitType values and the mapping from NalUnitType values to the type of RBSP data structures are specified in Tables XYZ1, XYZ2, and XYZ3.

For purposes other than determining the amount of data in the decoding units of the bitstream (as specified in Annex C), decoders shall ignore (remove from the bitstream and discard) the contents of all NAL units that use reserved values of NalUnitType.

NOTE 5 – This requirement allows future definition of compatible extensions to this Specification.

Table XYZ1 – Derivation of NalUnitType and TemporalId from nuh_first_octet

| nuh_first_octet | NalUnitType | TemporalId | Content of NAL unit |
| --- | --- | --- | --- |
| 1..127 | NalUnitTypeCategory | ( nuh_first_octet & 7 ) − 1 | Specified in Table XYZ2 |
| 128..151 | nuh_first_octet | 0 | Specified in Table XYZ3 |
| 152..165 | nuh_first_octet | 0 | Reserved non-VCL NAL unit types |
| 166..179 | nuh_first_octet | 0 | Reserved VCL NAL unit types |
| 180..187 | nuh_first_octet | 0 | Unspecified non-VCL NAL unit types |

Table XYZ2 – NAL unit type codes and NAL unit type classes for NalUnitTypeCategory values 0..15

| NalUnit Type | Name of NalUnitType | nuh_first_octet | Content of NAL unit and RBSP structure | NAL unit type class |
| --- | --- | --- | --- | --- |
| 0 | AUD_NUT | 00000 TID | Access unit delimiter access_unit_delimiter_rbsp( ) | Non VCL |
| 1 | TRAIL_NUT | 00001 TID | Coded tile group of non-STSA trailing picture, tile_group_layer_rbsp( ) | VCL |
| 2 | STSA_NUT | 00010 TID | Coded tile group of an STSA picture tile_group_layer_rbsp( ) | VCL |
| 3 | RASL_NUT | 00011 TID | Coded tile group of a RASL picture tile_group_layer_rbsp( ) | VCL |
| 4 | RADL_NUT | 00100 TID | Coded tile group of a RADL picture tile_group_layer_rbsp( ) | VCL |
| 5..9 | RSV_VCL_5 .. RSV_VCL_9 | 00101 TID .. 01001 TID | Reserved non-IRAP VCL NAL unit types | VCL |
| 10 | APS_NUT | 01010 TID | Adaptation parameter set adaptation_parameter_set_rbsp( ) | Non VCL |
| 11 | PREFIX_SEI_NUT | 01011 TID | Supplemental enhancement information sei_rbsp( ) | Non VCL |
| 12 | POSTFIX_SEI_NUT | 01100 TID | Supplemental enhancement information sei_rbsp( ) | Non VCL |
| 13..15 | RSV_NVCL_13 .. RSV_NVCL_15 | 01101 TID .. 01111 TID | Reserved non-VCL NAL unit types | Non VCL |

Table XYZ3 – NAL unit type codes and NAL unit type classes for NalUnitTypeCategory values 16..18

| NalUnit Type Category | NalUnitType | nuh_first_ octet | Name of NAL Unit Type | Content of NAL Unit Type and RBSP structure | NAL Unit Type Class |
| --- | --- | --- | --- | --- | --- |
| 16 | 129 | 10000 001 | IDR_W_RADL | Coded tile group of an IDR picture tile_group_layer_rbsp( ) | VCL |
| 16 | 130 | 10000 010 | IDR_N_LP | Coded tile group of an IDR picture tile_group_layer_rbsp( ) | VCL |
| 16 | 131 | 10000 011 | CRA_NUT | Coded tile group of a CRA picture tile_group_layer_rbsp( ) | VCL |
| 16 | 128, 132..135 | 10000 000, 10000 100 .. 10000 111 | RSV_IRAP_VCL_0, RSV_IRAP_VCL_4 .. RSV_IRAP_VCL_7 | Reserved IRAP VCL NAL unit types | VCL |
| 17 | 137 | 10001 001 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | Non VCL |
| 17 | 138 | 10001 010 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | Non VCL |
| 17 | 139 | 10001 100 | DPS_NUT | TBD | Non VCL |
| 17 | 140 | 10001 101 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | Non VCL |
| 17 | 141 | 10001 110 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | Non VCL |
| 17 | 136, 142, 143 | 10001 000, 10001 110 10001 111 | RSV_TID0_NVCL_0, RSV_TID0_NVCL_6, RSV_TID0_NVCL_7 | Reserved non-VCL NAL unit types | Non VCL |
| 18 | 144..147 | 10010 000 .. 10010 011 | RSV_TID0_VCL_A .. RSV_TID0_VCL_D | Reserved non-IRAP VCL NAL unit types | VCL |
| 18 | 148..151 | 10010 100 .. 10010 111 | RSV_TID0_NVCL_A .. RSV_TID0_NVCL_D | Reserved non-VCL NAL unit types | Non VCL |

NOTE 6 – A clean random access (CRA) picture may have associated RASL or RADL pictures present in the bitstream.

NOTE 7 – An instantaneous decoding refresh (IDR) picture having NalUnitType equal to IDR_N_LP does not have associated leading pictures present in the bitstream. An IDR picture having NalUnitType equal to IDR_W_RADL does not have associated RASL pictures present in the bitstream, but may have associated RADL pictures in the bitstream.

When NalUnitType is in the range of RSV_IRAP_VCL_0 to RSV_IRAP_VCL_7, inclusive, the coded tile group belongs to an IRAP picture.

The value of TemporalId shall be the same for all VCL NAL units of an access unit. The value of TemporalId of a coded picture or an access unit is the value of the TemporalId of the VCL NAL units of the coded picture or the access unit.

When NalUnitType is in the range of 0 to 15, inclusive, TemporalId shall be greater than or equal to the TemporalId of the access unit containing the NAL unit.

NOTE  8 – When the NAL unit is a non-VCL NAL unit, the value of TemporalId is equal to the minimum value of the TemporalId values of all access units to which the non-VCL NAL unit applies. When NalUnitType is equal to PREFIX_SEI_NUT or SUFFIX_SEI_NUT, TemporalId may be greater than or equal to the TemporalId of the containing access unit, as an SEI NAL unit may contain information that applies to a bitstream subset that includes access units for which the TemporalId values are greater than the TemporalId of the access unit containing the SEI NAL unit.

# Patent rights declaration(s)

Nokia and Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).