| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0827 |
| --- | --- |

| Title: | Cross check of JVET-N0392 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | K. Misra | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

# Abstract

This is cross-check report for JVET-N0392. The software package was provided by the proponent and was studied. The software changes seem consistent with the description in JVET-N0392. The changes were evaluated for 4:4:4 and 4:2:2 sequences for frame count listed in JVET-N0392, using encode configurations of JVET Common Test Condition (CTC) QPs for SDR video. All-Intra (AI), Random Access (RA) and Low Delay B (LDB) encoder configurations. It is reported that the R-D data generated by the crosschecker exactly matches that provided by the proponent for AI and LDB configuration (Note: As of writing of this report four data points of LDB are not available). For RA configuration there is a small difference in the 4th decimal place due to a difference in how average values were computed by proponent and crosschecker.

# Summary of cross check

The software package was provided by the proponent and studied. The base code used as starting point is the VTM software commit: fa107d012bbc0e55521c3822239662995b0657a6 [fa107d0]. The software changes seem consistent with the description in JVET-N0392 [1]. The following changes were observed in the software:

- The RExt__HIGH_BIT_DEPTH_SUPPORT macro is enabled

- Some change in data type used for coefficient coding variables

- Under macro EXT_CHROMA_FORMATS_SUPPORT_AFFINE: For 4:4:4 chroma format the affine motion compensation process of luma is used for all components

- Under macro EXT_CHROMA_FORMATS_SUPPORT_CCLM: 2, 3, 4, 5, 6 tap values are defined for chroma format 4:2:2 and 4:4:4. Availability checks seem to be modified.

- Under macro EXT_CHROMA_FORMATS_SUPPORT_DMVR: Starting point derivation is based on chroma format

The source code was compiled using GCC 7.3.1, and its performance was cross-checked for 4:4:4 and 4:2:2 sequences for frame count listed in JVET-N0392 for the All-Intra (AI), Random Access (RA) and Low Delay B (LDB) encoder configurations described JVET Common Test Condition (CTC) QPs for SDR video [3] (Note: As of writing of this report four data points of LDB are not available).

The full R-D results generated by the cross-check can be found in attached excel documents.

As noted in abstract above: R-D data generated by the crosschecker exactly matches that provided by the proponent for AI and LDB configuration. For RA configuration there is a small difference in the 4th decimal place due to a difference in how average values were computed by proponent and crosschecker.

The proponents of JVET-N0671 [2] also shared their source code. The provided JVET-N0671 software exactly matched that provided by the proponents of JVET-N0392.

# References

[1] A. Filippov, V. Rufitskiy, T. Solovyev, R. Chernyak, J. Chen, “4:4:4 and 4:2:2 chroma formats support for VVC,” JVET-N0392, Geneva, CH, March 2019

[2] A. Filippov, V. Rufitskiy, T. Solovyev, R. Chernyak, J. Chen (Huawei), X. Zhao, X. Li, X. Xu, S. Liu (Tencent), H. Wang, Y.-H. Chao, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm), “Support of 4:4:4 and 4:2:2 chroma formats in VVC,” JVET-N0671, Geneva, CH, March 2019

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Marrakech, MA, Jan 2019