| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0828-v2 |
| --- | --- |

| Title: | [AHG13] Compression performance analysis for 4K and 8K HLG test sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | +81-3-5494-3353 iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution reports compression performance of 4K and 8K HLG sequences which are under preparation for distribution as an ultra-high-definition/wide-color-gamut/high-dynamic-range standard test sequences from Japanese academic/industry society, The Institute of Image Information and Television Engineers (ITE). The test sequences were captured and mastered in BT.2100-HLG, 4:2:2, 10bit. Since most of the 4K and 8K sequences are captured by 4K and 8K cameras placed in parallel, the characteristics of each 4K and 8K pair are quite similar. A total of 17 test sequence pairs with a wide range of temporal and spatial characteristics such as portrait, sports, water surface, closed caption, etc, are encoded by HM-16.20 and VTM-4.0. It is reported that luma BD-rate improvement over HM-16.20 for 4K and 8K test sequences are -29.1% and -30.8%, respectively. The experimental results show the compression performance for 8K is almost equivalent to that for 4K.

In addition to performance report of whole VTM-4.0 package, individual tool-off results vs. VTM-4.0 anchor were also provided in this contribution. Although most of the tools works as expected, some undesirable performance improvement in tool-off test was observed. Further analysis using non-CTC sequences are also suggested in tool reporting procedure.

From the individual tool performance analysis, it is found that the parameter setting for LMCS in HDR CTC for HLG needs to be corrected. 1 line configuration change is suggested to disable LMCS for the HLG sequences. It is reported that the suggested CTC change gives 0.9% BD-rate gain for HLG CTC over the current CTC.

Specification of the test sequences

The 4K and 8K test sequences used for this experiment are the test materials “Ultra-high-definition/wide-color-gamut/high-dynamic-range standard test sequences (UHDTV-C)” which are under preparation for distribution by Japanese adacemic/industry society, the Instutute of Image Information and Television Engineers (ITE). NOTE that these 4K and 8K test sequences are only provided to the licensee and not allowed to redistribute.

For more details, readers may refer to the following address and contact information.

Web site: https://www.ite.or.jp/content/test-materials/uhdtv_hdr/

Contact: ite@ite.or.jp

A total of 17 test sequence pairs are captured by 8K and 4K cameras placed in parallel. The specifications of the 8K camera used for shooting and signal formats are listed Table 1 and Table 2, respectively.

Table 1. Specification of cameras

|  | 8K | 4K |
| --- | --- | --- |
| Model | SONY UHC-8300 | SONY HDC-4300 |
| Censor | 1.25-inch 3-CMOS | 2/3-inch 3-CMOS |
| Color space | ITU-R Rec. BT.2020 |  |
| Frame rate | 59.94/60 Hz (progressive) |  |

Table 2. Video signal format

|  | 8K | 4K |
| --- | --- | --- |
| Resolution | 7680x4320 | 3840x2160 |
| Chroma format | YCbCr 4:2:2 |  |
| Bit depth | 10bit |  |
| Transfer characteristics | ITU-R Rec. BT-2100-HLG |  |

Simulations

Since the test sequences were captured in YCbCr 4:2:2 format, down-sampling process from 4:2:2 to 4:2:0 were applied, where {1, 6, 1} filter were used for horizontal down-sampling.

Although the number of frames of the original test sequences was 900 for both 8K and 4K sequences, the representative 65 frames (=1 IRAP) were selected and encoded by HM-16.20 and VTM-4.0 due to the limitation of computational resources and time. Encoding structure was randomaccess configuration which follows HDR CTC (e.g. QP={22, 27, 32, 37}, IntraPeriod=64, Luma delta QP and Chroma QP offset were disabled). The thumbnail, histogram, and chromaticity diagram of the first frame of the selected 65 frames for each sequence were illustrated in the appendix of this contribution. In performance analysis, the last frame (64th frame) encoded as I-frame was excluded to keep the I-frame ratio 1 frame in 64 frames.

Table 3 and Table 4 show the compression performance of 4K and 8K HLG test sequences over HM-16.20, respectively. In average, 29.1% and 30.8% bitrate saving could be observed for 4K and 8K sequences, respectively, with approx. 5x encoding runtime and 1.1x decoding runtime.

Table 3 Compression performance of 4K HLG test sequences over HM-16.20

| Sequence name | Y | U | V |
| --- | --- | --- | --- |
| c01_Fireworks_willow_4K | -19.29% | -30.02% | -42.53% |
| c02_Fireworks_longshot_4K | -30.19% | -36.28% | -39.33% |
| c03_Fireworks_scrollingtext_4K | -37.08% | -39.15% | -40.99% |
| c04_Fireworks_chrysanthemum_4K | -27.23% | -36.61% | -38.97% |
| c05_Fireworks_barrage_4K | -26.84% | -44.70% | -45.36% |
| c06_Drama_standingup_4K | -31.95% | -35.03% | -36.82% |
| c07_Drama_sea_4K | -20.65% | -23.20% | -24.92% |
| c08_Drama_sunset_4K | -21.16% | -58.86% | -47.93% |
| c09_SwimRace_breaststroke | -21.17% | -35.56% | -39.50% |
| c10_SwimRace_crawl_4K | -23.78% | -45.55% | -56.47% |
| c11_SwimRace_backstroke_4K | -28.17% | -40.96% | -42.60% |
| c12_Volleyball_fixed_4K | -32.83% | -30.09% | -31.89% |
| c13_Volleyball_crawlingtext_4K | -29.47% | -28.32% | -31.16% |
| c14_Volleyball_follow_4K | -42.77% | -43.70% | -46.74% |
| c15_Paddock_fixed_4K | -27.24% | -23.04% | -21.73% |
| c16_Paddock_follow_4K | -34.40% | -39.53% | -42.34% |
| c17_HorseRace_homestretch_4K | -40.14% | -48.43% | -45.60% |
| Average | -29.08% | -37.59% | -39.70% |
| Encoding time |  | 547% |  |
| Decoding time |  | 111% |  |

Table 4. Compression performance of 8K HLG test sequences over HM-16.20

| Sequence name | Y | U | V |
| --- | --- | --- | --- |
| c01_Fireworks_willow_8K | -21.77% | -37.46% | -37.59% |
| c02_Fireworks_longshot_8K | -35.09% | -46.19% | -36.36% |
| c03_Fireworks_scrollingtext_8K | -37.63% | -44.41% | -40.05% |
| c04_Fireworks_chrysanthemum_8K | -27.63% | -33.99% | -36.35% |
| c05_Fireworks_barrage_8K | -28.48% | -46.76% | -44.77% |
| c06_Drama_standingup_8K | -33.01% | -27.66% | -31.43% |
| c07_Drama_sea_8K | -24.31% | -33.79% | -50.79% |
| c08_Drama_sunset_8K | -21.86% | -23.06% | -37.08% |
| c09_SwimRace_breaststroke | -25.72% | -53.96% | -47.69% |
| c10_SwimRace_crawl_8K | -23.87% | -45.56% | -53.54% |
| c11_SwimRace_backstroke_8K | -26.54% | -38.86% | -51.53% |
| c12_Volleyball_fixed_8K | -34.02% | -36.30% | -42.00% |
| c13_Volleyball_crawlingtext_8K | -35.74% | -37.67% | -43.05% |
| c14_Volleyball_follow_8K | -40.37% | -47.18% | -46.18% |
| c15_Paddock_fixed_8K | -27.06% | -22.86% | -23.10% |
| c16_Paddock_follow_8K | -37.06% | -44.30% | -43.98% |
| c17_HorseRace_homestretch_8K | -43.57% | -46.32% | -45.39% |
| Average | -30.81% | -39.20% | -41.82% |
| Encoding time |  | 498% |  |
| Decoding time |  | 106% |  |

Individual tool evaluation

To verify the efficiency of the individual coding tool adopted in VTM-4.0, tool-off test for representative 5 sequences (c05, c07, c10, c14, c15) were conducted. Table 5 shows individual tool-off test results in sequence basis, where performance improvements more than 0.1% were marked red.

From the tool-off test results, it is found that the parameter setting for LMCS (previously known as reshaper) in HDR CTC for HLG needs to be corrected. Specifically, in HDR-HLG CTC, the parameter “ReshapeSignalType” is currently set equal to 0 (RESHAPE_SIGNAL_SDR). It means that the procedure for SDR contents is applied to HLG contents regardless of the difference of transfer characteristics. In the current VTM-4.0 implementation, when ReshapeSignalType is set equal to RESHAPE_SIGNAL_HLG, no reshaping procedure applies. The suggested change is to add the following line to the HLG sequence configuration file:

ReshapeSignalType : 2 // RESHAPE_SIGNAL_HLG

Remark that this individual tool evaluation was only tested by 64-frame 5 sequences, so that it is not reasonable to take a specific tool for an ineffective tool. Instead, it is suggested that tool reporting procedure using non-CTC sequences is also required to further investigate individual tool performance.

Table 5. Individual tool-off test results for 8K sequences (sequence basis)

|  | DualITree | CCLM | MTS | ALF | Affine | SubPuMC | IMV |
| --- | --- | --- | --- | --- | --- | --- | --- |
| c05_Fireworks_barrage | 1.05% | 6.29% | 0.68% | 4.14% | 0.09% | 0.21% | 0.21% |
| c07_Drama_sea | 1.42% | 4.26% | 0.67% | 3.27% | -0.03% | 0.89% | 0.41% |
| c10_SwimRace_crawl | 0.08% | 3.41% | 0.51% | 1.89% | 0.10% | -0.03% | 0.24% |
| c14_Volleyball_follow | 1.74% | 1.30% | 0.55% | 7.80% | 1.58% | 0.49% | 2.41% |
| c15_Paddock_fixed | 0.37% | 1.51% | 1.01% | 3.48% | 0.82% | 0.66% | 0.47% |
|  | Triangle | BDOF | CIIP | BWA | DMVR | Reshape | TS |
| c05_Fireworks_barrage | 0.16% | 0.39% | 0.26% | 0.80% | -0.30% | 0.33% | 0.08% |
| c07_Drama_sea | 0.09% | 0.15% | 0.49% | 0.31% | 0.11% | 0.02% | -0.01% |
| c10_SwimRace_crawl | -0.05% | -0.04% | 3.68% | 0.01% | -0.13% | -1.60% | -0.01% |
| c14_Volleyball_follow | 0.77% | -0.15% | 0.48% | 2.76% | -0.11% | -1.08% | 0.02% |
| c15_Paddock_fixed | 0.27% | 0.38% | 0.13% | 2.27% | -0.38% | -0.07% | 0.10% |

Table 6. Individual tool-off test results for 8K sequences

| Tool off test | Y | U | V | enc | dec |
| --- | --- | --- | --- | --- | --- |
| CST (DualITree=0) | 0.08% | 6.48% | 10.64% | 101% | 101% |
| CCLM (LMChroma=0) | 0.61% | 22.82% | 33.38% | 100% | 101% |
| MTS (MTS=0) | 0.75% | 0.22% | -0.05% | 97% | 99% |
| ALF (ALF=0_ | 4.10% | 5.20% | 3.22% | 101% | 103% |
| AFF (Affine=0) | 0.50% | 0.54% | 0.78% | 88% | 97% |
| SbTMVP (SubPuMvp=0) | 0.46% | 0.29% | 0.26% | 101% | 100% |
| AMVR (IMV=0) | 0.64% | 1.47% | 1.90% | 88% | 101% |
| TPM (Triangle=0) | 0.22% | 0.46% | 0.55% | 88% | 102% |
| BDOF (BIO=0) | 0.17% | -0.11% | -0.05% | 95% | 92% |
| CIIP (MHIntra=0) | 0.96% | 1.02% | 1.85% | 97% | 99% |
| BPWA (GBi=0) | 1.21% | 1.36% | 1.34% | 94% | 101% |
| DMVR (DMVR=0) | -0.20% | 0.07% | 0.31% | 98% | 91% |
| ILRS (LumaReshapeEnable=0) | 0.10% | -4.54% | -6.78% | 87% | 95% |
| TSM (TransformSkip=0) | 0.05% | -0.05% | -0.04% | 100% | 101% |

Results of HLG CTC sequence with ReshapeSignalType=2

The results for HLG CTC sequences with ReshapeSignalType=2 is shown in Table 7. In averge, 0.9% BD-rate improvement was observed.

Table 7. Results of suggested CTC change over current HDR CTC

|  |  | BD-rate |  |  |
| --- | --- | --- | --- | --- |
|  |  | psnrY | psnrU | psnrV |
| HDR-A | DayStreet | -0.3% | -1.7% | -1.6% |
|  | FlyingBirds2 | -0.2% | -1.1% | -1.2% |
|  | PeopleInShoppingCenter | -1.7% | -6.6% | -6.6% |
|  | SunsetBeach2 | -1.3% | -5.1% | -5.4% |
|  | Average | -0.9% | -3.6% | -3,7% |
|  | Enc time |  | 89% |  |
|  | Dec time |  | 97% |  |

Conclusion

This contribution report VTM-4.0 compression performance of 4K and 8K HLG test sequences over HM-16.20. The simulation results indicate VTM-4.0 works as expected for both 4K and 8K HLG test sequences. In addition to performance analysis of whole VTM-4.0 package, individual tool performance analysis was also conducted. Although most of the tools works as expected, some undesirable performance improvement in tool-off test was observed. Further analysis using non-CTC sequences are also suggested in tool reporting procedure. From the individual tool performance analysis, it is found that the parameter setting for LMCS in HDR CTC for HLG needs to be corrected. 1 line configuration change is suggested to disable LMCS for the HLG sequences. It is observed that the suggested CTC change gives 0.9% BD-rate gain for HLG CTC over the current CTC.

Patent rights declaration(s)

NHK does not have any current or pending patent rights relating to the technology described in this contribution.

- Annex. Thumbnails, histogram and chromaticity diagram

| c01_Fireworks_willow_4K, c01_Fireworks_willow_8K |  |  |
| --- | --- | --- |
| 4K thumbnail 8K thumbnail | 4K histgram 8K histogram | 4K chromaticity 8K chromaticity |
| c02_Fireworks_longshot_4K, c02_Fireworks_longshot_8K |  |  |
| 4K thumbnail 8K thumbnail | 4K histgram 8K histogram | 4K chromaticity 8K chromaticity |
| c03_Fireworks_scrollingtext_4K, c03_Fireworks_scrillingtext_8K |  |  |
| 4K thumbnail 8K thumbnail | 4K histgram 8K histogram | 4K chromaticity 8K chromaticity |

| c04_Fireworks_chrysanthemum_4K, c04_Fireworks_chrysanthemum_8K |  |  |
| --- | --- | --- |
| 4K thumbnail 8K thumbnail | 4K histgram 8K histogram | 4K chromaticity 8K chromaticity |
| c05_Fireworks_barrage_4K, c05_Fireworks_barrage_8K |  |  |
| 4K thumbnail 8K thumbnail | 4K histgram 8K histogram | 4K chromaticity 8K chromaticity |
| c06_Drama_standingup_4K, c06_Drama_standingup_8K |  |  |
| 4K thumbnail 8K thumbnail | 4K histgram 8K histogram | 4K chromaticity 8K chromaticity |

| c07_Drama_ sea_4K, c07_Drama_sea_8K |  |  |
| --- | --- | --- |
| 4K thumbnail 8K thumbnail | 4K histgram 8K histogram | 4K chromaticity 8K chromaticity |
| c08_Drama_sunset_4K, c08_Drama_sunset_8K |  |  |
| 4K thumbnail 8K thumbnail | 4K histgram 8K histogram | 4K chromaticity 8K chromaticity |
| c09_SwimRace_breaststroke_4K, c09_SwimRace_breaststroke_8K |  |  |
| 4K thumbnail 8K thumbnail | 4K histgram 8K histogram | 4K chromaticity 8K chromaticit1 |

| c10_SwimRace_crawl_4K, c10_SwimRace_crawl_8K |  |  |
| --- | --- | --- |
| 4K thumbnail 8K thumbnail | 4K histgram 8K histogram | 4K chromaticity 8K chromaticity2 |
| c11_SwimRace_backstroke_4K, c11_SwimRace_backstroke_8K |  |  |
| 4K thumbnail 8K thumbnail | 4K histgram 8K histogram | 4K chromaticity 8K chromaticity |
| c12_Volleyball_fixed_4K, c12_Volleyball_fixed_8K |  |  |
| 4K thumbnail 8K thumbnail | 4K histgram 8K histogram | 4K chromaticity 8K chromaticity |

| c13_Volleyball_crawlingtext_4K, c13_Volleyball_crawlingtext_8K |  |  |
| --- | --- | --- |
| 4K thumbnail 8K thumbnail | 4K histgram 8K histogram | 4K chromaticity 8K chromaticity |
| c14_Volleyball_follow_4K, c14_Volleyball_follow_8K |  |  |
| 4K thumbnail 8K thumbnail | 4K histgram 8K histogram | 4K chromaticity 8K chromaticity |
| c15_Paddock_fixed_4K, c15_Paddock_fixed_8K |  |  |
| 4K thumbnail 8K thumbnail | 4K histgram 8K histogram | 4K chromaticity 8K chromaticity |

| c16_Paddock_follow_4K, c16_Paddock_follow_8K |  |  |
| --- | --- | --- |
| 4K thumbnail 8K thumbnail | 4K histgram 8K histogram | 4K chromaticity 8K chromaticity |
| c17_HorseRace_homestretch_4K, c17_HorseRace_homestretch_8K |  |  |
| 4K thumbnail 8K thumbnail | 4K histgram 8K histogram | 4K chromaticity 8K chromaticity |

--------------------------------------------------------------------------------
Courtesy of The Institute of Information and Television Engineers
--------------------------------------------------------------------------------