| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0835-v2 |
| --- | --- |

| Title: | Core Experiment Viewing Test Procedure and Results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mathias Wien Lehrstuhl für Bildverarbeitung RWTH Aachen University Philippe Hanhart InterDigital | Tel: Email: | +49-241-80-27867 wien@lfb.rwth-aachen.de Philippe.Hanhart@InterDigital.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This contribution provides a report on the subjective testing performed at the 14th JVET meeting. Subjective tests have been conducted for CE1-6, CE5-1, CE5-2, CE11-1, and CE11-2. Upon completion, the results were provided to the chairs and the CE coordinators for consideration in the discussion of the respective CEs. The report includes a description of the evaluation method, the testing procedure and the test results for the above-mentioned CEs. MOS plots for the results of all performed tests are reported in the document. The corresponding data are provided in the accompanying Excel sheet.

# Introduction

Expert viewing tests were conducted during the 14th JVET meeting at Geneva for the core experiments CE1-6, CE5-1, CE5-2, CE11-1, and CE11-2, which contained one or more sub-CEs. Calls for participation in the subjective viewing were issued on the JVET reflector and verbally during JVET Track A sessions. For each test campaign, 12 experts were invited from the list of experts who volunteered. The dates and timing of the viewing sessions are reported in the results section of this document. The resulting MOS values are provided in the accompanying Excel sheet. The testing procedure follows the method used for visual testing during the 13th JVET meeting in Marrakech, JVET-M0906 [1].

# Test arrangement

The sequences were presented on a LG OLED55C8PLA display using a Linux PC with a pre-installed video player capable of playing UHD 60Hz video at full frame rate and stable output. Three expert viewers were seated in front of the screen at a distance of 1.6 time the height of the screen. The room windows were fully shaded. Before the viewing sessions the light in the room was turned off. The YUV files for the tests were decoded on the test PC, with the proponents and cross-check partners verifying the correctness of the decoded YUV files and the assigned YUV file names by comparison of md5sums.

The CE descriptions defined test sets with a low number of sequences at various resolutions (720p, 1080p, 4K). It was decided to present all sequences scaled to the full UHD resolution of the screen. Nearest neighbor interpolation was used for up-scaling.

# Test procedure

## Test session setup

The viewing compared the reconstruction quality of the proposals under test to the reconstruction quality of the VTM 4 anchor.

In the following, video A and video B denote the anchor and the proposal for the same sequence and same QP for a test point of the core experiment under evaluation. The assignment of anchor and proposal to A or B was randomized and not revealed to the test subjects. A viewing session consisted of 12 to 17 test cells, with each test cell constructed as follows:

- “A” (on a mid-gray background, 0.5 s presentation)

- Video A

- “B” (on a mid-gray background, 0.5 s presentation)

- Video B

- “A” (on a mid-gray background, 0.5 s presentation)

- Video A

- “B” (on a mid-gray background, 0.5 s presentation)

- Video B

- “Vote N” (on a mid-gray background, 5 s presentation)

For a video sequence duration of 10 s, the duration of one test cell thereby accumulated to 47 s.

The sequence of test cells included each test cell once. The test subjects were asked to rate the video sequences presented in each test cell as follows:

- “A was much better than B”

- “A was better than B”

- “B was better than A”

- “B was much better than A”

The used voting sheet is presented in the figure below. The sheet records the CE, test session, ID of the test subject and position in front of the screen.

Fig. 1: Voting sheet

In each viewing test, the order of the video sequences was fixed and looped over the available test points (QP values, sub-CEs). Thereby it was asserted that not two test points for a video sequence would be presented subsequently. The order of the available test points was randomized over the session.

## Session arrangement

The viewing tests were carried out with groups of three experts. The tests were drafted to have a maximum assessment time below 15 min per session. For tests with more than one session, the assessment of two groups was interleaved: While one group was under test, the other group had a break. In the break, the groups were advised to enter the voting results of their previous session into a prepared Excel sheet and verify the correctness of this transfer. This arrangement was set up to minimize the test duration for the participating experts while providing them sufficient recreation over the testing period.

## Voting scores

For evaluation, the votes of the test subjects were mapped to numbers as follows:

- +3	: “Proposal much better than anchor”

- +1	: “Proposal better than anchor”

- -1	: “Anchor better than proposal”

- -3	: “Anchor much better than proposal”

From these values, the MOS and the 95% confidence intervals were calculated. A resulting MOS value of zero indicate that anchor and proposal were rated to have the same visual quality.

# Test results

## CE1 Post-prediction and post-reconstruction filtering

The description of the core experiment can be found in JVET-M1021 [2]. SubCEs CE1-6.1 and CE1-6.2 include visual testing. The bitrates for the anchors and proposals were considered to be matching for all test points. The following sequences and QPs were used for subjective performance comparisons.

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| S02 | Tango2 | 3840x2160p | 60 | 600 | RA | 34, 39 |
| S03 | Cactus | 1920x1080p | 50 | 500 | RA | 34, 39 |
| S01 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 34, 39 |

It is noted that the numbering above diverges from the definition in JVET-M1021 but matches the decoded YUV files.

The viewing was conducted on Fri. 2019-03-22, with results of CE1-6.1 and CE1-6.2 assessed in one session. The session comprised 12 test cells. 11 test subjects in four groups participated in the viewing, with Group 3 having two participants. The viewing time per session was approximately 9:30 min.

|  | Start | Group1 | Group2 | Group3 | Group4 |
| --- | --- | --- | --- | --- | --- |
| S1 | 19:20:00 | 3 |  |  |  |
|  | 19:35:00 |  | 3 |  |  |
|  | 19:50:00 |  |  | 2 |  |
|  | 20:05:00 |  |  |  | 3 |
|  | 20:20:00 |  |  |  |  |

### MOS plots

## CE5 Adaptive Loop Filter

The description of the core experiment can be found in JVET-M1025 [3]. The definition of CE5-1 and CE5-2 include visual testing. The bitrates for the anchors and proposals were considered to be matching for all test points. The following sequences and QPs were used for subjective performance comparisons:

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| S02 | Johnny | 1280x720p | 60fps | 600 | LDP | 37, 32 |
| S04 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 37, 32 |
| S06 | Cactus | 1920x1080 | 50fps | 500 | LDP | 37, 32 |

The viewing was conducted on Thu, 2019-03-21, jointly assessing the results of CE5-1 and CE5-2. The session comprised 36 test cells split into 3 test sessions. 11 test subjects in four groups participated in the viewing, with Group 4 having two participants. The viewing time per session was approximately 9:30 min.

|  | Start | Group1 | Group2 | Group3 | Group4 |
| --- | --- | --- | --- | --- | --- |
| S1 | 18:30:00 | 3 |  |  |  |
|  | 18:45:00 |  | 3 |  |  |
| S2 | 19:00:00 | 3 |  |  |  |
|  | 19:15:00 |  | 3 |  |  |
| S3 | 19:30:00 | 3 |  |  |  |
|  | 19:45:00 |  | 3 |  |  |
| S1 | 20:00:00 |  |  | 3 |  |
|  | 20:15:00 |  |  |  | 2 |
| S2 | 20:30:00 |  |  | 3 |  |
|  | 20:45:00 |  |  |  | 2 |
| S3 | 21:00:00 |  |  | 3 |  |
|  | 21:15:00 |  |  |  | 2 |
|  | 21:30:00 |  |  |  |  |

### MOS plots

## CE11 Deblocking

The description of the core experiment can be found in JVET-M1031 [4]. The definition of CE11-1 and CE11-2 include visual testing on two different sets of sequences with ALF being switched on and off. The bitrates for the anchors and proposals were considered to be matching for all test points. It was remarked by participants of the tests that the QP39 rate points showed significant compression artifacts for some of the sequences.

The following sequences and QPs were used for subjective performance comparisons in CE11-1:

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| S01 | FoodMarket | 3840x2160p | 60fps | 600 | RA | 34, 39 |
| S02 | Campfire | 3840x2160p | 30fps | 300 | RA | 34, 39 |
| S03 | Kimono | 1920x1080p | 24fps | 240 | LDB | 34, 39 |
| S04 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 34, 39 |
| S05 | Red Kayak | 1920x1080p | 30fps | 300 | RA | 34, 39 |

The following sequences and QPs were used for subjective performance comparisons in CE11-2:

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Redkayak | 1920x1080p | 30fps | 300 | RA | 34, 39 |
| 02 | Cactus | 1920x1080p | 50fps | 500 | LDB | 34, 39 |
| 03 | FourPeople | 1280x720p | 60fps | 600 | LDP | 34, 39 |
| 04 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 34, 39 |

### ALF on

The viewing was conducted on Fri. 2019-03-22, jointly assessing the results of CE11-1 and CE11-2. The session comprised 34 test cells split into 2 test sessions. 12 test subjects in four groups participated in the viewing. The viewing time per session was approximately 13:20 min.

|  | Start | Group1 | Group2 | Group3 | Group4 |
| --- | --- | --- | --- | --- | --- |
| S1 | 09:20:00 | 3 |  |  |  |
|  | 09:39:00 |  | 3 |  |  |
| S2 | 09:58:00 | 3 |  |  |  |
|  | 10:17:00 |  | 3 |  |  |
| S1 | 10:36:00 |  |  | 3 |  |
|  | 10:55:00 |  |  |  | 3 |
| S2 | 11:14:00 |  |  | 3 |  |
|  | 11:33:00 |  |  |  | 3 |
|  | 11:52:00 |  |  |  |  |

#### CE11-1 MOS plots

#### CE11-2 MOS plots

### ALF off

The viewing was conducted on Sat. 2019-03-23. Only the results of CE11-2 were assessed. The session comprised 24 test cells split into 2 test sessions. 12 test subjects in four groups participated in the viewing. The tests for Groups 3 and 4 were conducted in separate sessions as test subjects had to be acquired from the JVET track sessions. The viewing time per session was approximately 11:30 min.

|  | Start | Group1 | Group2 |
| --- | --- | --- | --- |
| S1 | 15:00:00 | 3 |  |
|  | 15:15:00 |  | 3 |
| S2 | 15:30:00 | 3 |  |
|  | 15:45:00 |  | 3 |

#### CE11-2 MOS plots

# Thanks

- Thanks to all volunteers who participated in the tests.

- Thanks to Andrew Segall for providing the test PC, video player, and support.

- Thanks to EBU for providing OLED display.

- Thanks to Vittorio Baroncini for his guidance.

- Thanks to ITU for providing the facilities.

# Conclusions

With respect to the conduction of visual tests for CEs, the following aspects are noted:

- The quantizer settings should be selected such that general compression artifacts in the sequences under test do not overrule the observation of the visual impact of the tool under test. E.g. in the CE11 tests at this meeting, QP 39 was considered to imply too low quality by multiple test subjects, especially for the Class E sequences.

- CE viewing sessions may need to include sequences of different resolution if the CE definition includes a corresponding sequence selection. At this meeting, all sequences were presented full-screen, with nearest-neighbor interpolation used for eventually required up-scaling. This aspect should be considered when drafting the CE definition.

- CE participants should upload bitstreams, decoder binaries, and a decoding script to the dedicated CE directory on the JVET ftp site. The binaries and scripts should be adapted to the operating system of the test PC (e.g., Ubuntu 16.04 with gcc 5.5.0 at this meeting). The data should be available at least three days before the meeting to allow for upfront transfer to the test PC. 
The recommended YUV file name convention is as follows, with “P” denoting the proposal, and “A” denoting the anchor. The subCE identification should only use dashes: 

CE<xx-y-z>_S<ii>_QP<jj>_{P|A}.yuv, 		e.g.: CE05-1-2_S02_QP34_P.yuv

The script should include the generation of a md5sum log-file for verification of the decoded results. The correctness of the assignment of the filenames should be verified and reported by the cross-checkers of the respective experiment. It is further suggested to always activate the picture hash SEI message in the uploaded bitstreams to help identification.

# References

- V. Baroncini, A. Norkin, A. M. Kotra, K. R. Andersson, K. Misra, H. Jang, C.M. Tsai, D. Rusanovskyy, “Subjective assessment of CE11 proposals,“ Document JVET-M0906, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, 13th meeting, Jan. 2019.

- J. Ström, S. Ikonin, V. Seregin, “Description of Core Experiment 1 (CE1): Post-prediction and post-reconstruction filtering,” Document JVET-M1021, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, 13th meeting, Jan. 2019.

- C.-Y. Chen, V. Seregin, “Description of Core Experiment 5 (CE5): Adaptive loop filter,” Document JVET-M1025, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, 13th meeting, Jan. 2019.

- A. Norkin, A. M. Kotra, “Description of Core Experiment 11 (CE11): Deblocking,” Document JVET-M1031, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, 13th meeting, Jan. 2019.