| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0838-v1 |
| --- | --- |

| Title: | BoG report on intra prediction and mode coding (CE3-related) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera | Email: | geertv@qti.qualcomm.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG reviewed proposals related to the intra prediction and mode coding topic (CE3-related). In this report, recommendations are included for the creation of CE3-subtests, a list of proposals to be further presented and reviewed in track A, including recommendations for adoption of proposals.

BoG recommendations:

- Continue CE3-subtest on MPM list harmonization with following proposals:

JVET-N0170: “Explicitly signal non angular modes”

JVET-N0185 (presentation requested in track A): “Unified MPM list for intra mode coding”

JVET-N0303: “Simplified unified luma intra mode coding”

JVET-N0394: “Unified MPM list based on CE3-3.3 and CE3-3.5.1”

JVET-N0412: “Modification on the intra luma mode coding process”

JVET-N0433: “Unification of MPM derivation for luma intra modes”

JVET-N0450: “Unified MPM list construction”

JVET-N0451: “Harmonization of MPM lists”

There was discussion on test conditions, including defining a ‘fairer’ codebase to evaluate the MPM harmonization proposals in addition to the VTM anchor, as well as criteria for complexity assessment.

- On block size restrictions and CE3-2 related topics, presentation of following proposals in track A for further discussion:

JVET-N0308 (recommended for adoption): “Restriction of the maximum CU size for ISP to 64x64”

JVET-N0082: “Constrained partitioning of chroma intra CBs”

JVET-N0390: “Study of CCLM restrictions in case of separate luma/chroma tree”

JVET-N0396: “Enabling parallel reconstruction of small intra-coded blocks”

JVET-N0453: “On chroma 2xN and Nx2 intra prediction in single tree”

- Create CE3-subtest on ISP simplifications including tests from following proposals:

JVET-N0159: “Relaxation method of processing dependency in ISP for small blocks”

JVET-N0186: “Simplification on Intra sub-partition coding mode”

JVET-N0313: “Low latency intra sub-partitions”

JVET-N0330: “Intra sub-partition coding without thin partitions”

JVET-N0372: “ISP with independent sub-partitions for certain block sizes”

JVET-N0432: “On 1xN and 2xN subpartitions in intra subpartition coding”

- On other ISP topics, presentation in track A of following proposals for further discussion:

JVET-N0358: “Harmonization of CBF coding for intra sub-partition coding mode”

JVET-N0437: “On allowing non-MPM modes for ISP and non-zero reference line”

- On other intra topics:

JVET-N0129 (recommended for adoption): “PDPC simplification”

JVET-N0222 (editorial adoption): “CIIP intra mode propagation to intra block”

JVET-N0435 (recommended for adoption): “Harmonization between WAIP and intra smoothing filters”

Create CE test on CCLM model improvement:

JVET-N0524: “Modified beta derivation in CCLM”

Editorial adoption:

The CE3 BoG met during these times:

20 March 2019, from 15:30-18:15 and from 18:45-21:00

21 March 2019, from 11:00-13:00 and from 19:00-22:00

22 March 2019, from 9:00-11:30 and from 18:45-20:15

# BoG Notes

## MPM list unification test condition discussion

- Option 1: Precisely list modes to test

- Option 2: Allow to test a number of modes or a number of lists

Are we defining new intra search method or new anchor; if method is tested with modified search, how does method perform in VTM?

Option 1 seems preferred. Problem is what modes to test.

We could run full RD intra search, enable parallel intra.

Other option is not inserting MPM candidates.

One proponent mentioned separating the MPM insertion (VTM MPM list) into the full RD from the coding cost computation.

Ideal solution would be using single list with fixed number of candidates and same index coding.

One anchor is VTM. In addition:

Suggested ‘fair’ anchor:

For MRLidx=0 perform search with SATD cost (planar, DC, all angular modes). For ISP, from the SATD cost list keep best 4 modes and remove non-ISP MPM modes, followed by full RD for ISP with up to 4 candidates for both horizontal and vertical ISP. For MRLidx>0, perform search with SATD for MPM list candidates using the MRLidx>0 MPM list (6 cand. from VTM). After SATD testing (MRLidx=0 and MRLidx>0) perform full RD testing of 4 candidates. Total full RD checks is up to 12.

To be tried in VTM4 during meeting (Frank, Ling)

Continued on 22 March:

1.1% AI loss is observed with similar runtime as VTM4

5 modes have also been tested, with observed losses

It is suggested to add L, A, PL resulting in -0.12% coding benefit with 10% increased encoder runtime. It is commented that it will benefit ISP, because PL is inserted, resulting in more ISP blocks.

One suggestion was 3 modes with lowest bit cost are inserted in the list for full RD.

Suggested ‘fair’ test: For MRLidx=0 perform search with SATD cost (planar, DC, all angular modes). For ISP, from the SATD cost list keep best 4 modes and remove tester’s non-ISP modes, followed by full RD for ISP with up to 4 candidates for both horizontal and vertical ISP. For MRLidx>0, perform search with SATD for MPM list candidates using the MRLidx>0 tester’s MPM list (up to 6 cand.). After SATD testing (MRLidx=0 and MRLidx>0) perform full RD testing of 4 candidates. Total full RD checks is up to 12.

The tested method shall not include other search method optimizations. Separate test results need to be provided for this case. The test result shall be disregarded if a proposed method includes such an optimization and separate results are not provided with and without the optimization.

Discussion on enabling additional modes for MRLidx>0 and ISP, such as planar and DC:

The tests shall use a common codebase which has minimal code changes to remove restrictions preventing the use of such modes for MRLidx>0 and ISP. A software patch is to be shared during the current meeting (LGE, Huawei).

Complexity aspects discussion:

|  | VTM-4.0 |
| --- | --- |
| # MPMs (regular) | 6 |
| # MPMs (MRL) | 6 |
| # MPMs (ISP) | 6 |
| # common MPMs among the lists |  |
| Does MPM list construction depend on MRL index or ISP mode? | Yes |
| Re-index if fewer | No |
| Context-coded syntax elements (#contexts) | MPM_flag (1) |
| Neighbour information for context modelling |  |
| Number of bits needed per neighbouring block for context modelling |  |
| Max layers of if conditions (*) |  |
| Max number of comparison operators in worst case, including the number of bits to compare (*) |  |
| # cycles (*) |  |
| Lookup table size if any |  |

(*) additional hardware expert opinions are requested

## MPM harmonization related proposals

BoG recommends continuing the CE3 subtest on MPM harmonization with following proposals:

JVET-N0170

JVET-N0185 (revisit requested in track A)

JVET-N0303

JVET-N0394

JVET-N0412

JVET-N0433

JVET-N0450

JVET-N0451

JVET-N0170 CE3-related: Explicitly signal non angular modes with encoding changes [J. Yao, J. Zhu, W. Cai, K. Kazui (Fujitsu)] [late]

This contribution presents to change the method of adding additional modes into the R-D check list to find a better trade-off between the gain and the encoding run-time with respect to the results of explicitly signal non angular modes (CE3-3.5.1). Instead of adding left intra mode(L) and above intra mode(A) into the R-D check list (VTM-4.0) we add modes with lowest bit cost for mode signaling depend on L and A. Such proposed encoding changes is implemented upon CE3-3.5.1. The results reportedly show an average luma BD rate performance of (−0.18%, −0.16%, −0.12%) for (Y, Cb, Cr) AI, (−0.xx%, −0.xx%, −0.xx%) for (Y, Cb, Cr) RA without increase of encoding running time over VTM-4.0.

No dependency with ISP mode and ref_idx for MPM derivation and index coding, by switching signalling order of MPM_flag and ISP and MRL syntax elements. Number of MPMs is 6 always. Also proposes encoding only changes to test 3-3.5.1.

It is questioned whether the switching of signalling order is an harmonization. The proponent claims that it simplifies the signalling.

BoG recommends CE testing MPM list construction method JVET-N0170 in the new CE3 subtest on MPM harmonization, replacing the current CE3-3.5.1 test. Separate results will be provided for the proposed encoder change.

Cross-check in JVET-N0745.

JVET-N0185 CE3-related: A unified MPM list for intra mode coding [B. Wang, A. M. Kotra, S. Esenlik, H. Gao, J. Chen (Huawei)]

This contribution reports the result of a method using a unified 6-MPM list in the intra mode coding, regardless of whether multiple reference line (MRL) or intra sub-partition (ISP) coding mode is applied or not. Compared to VTM4.0, this method achieve 0.06% coding gain in AI, with 101% encoding and 100% decoding time observed.

6 MPMs for regular, MRL, ISP.

1st bin of MPM index is coded with 3 contexts. There is dependency here.

DC, planar is used for all MPM list cases (ISP, MRL). The modes are enabled in these cases. Some modifications are implemented to enable planar mode for MRL, PDPC. What transforms used in those cases, for example, DC mode for ISP.

Cross-check in JVET-N0750 (not uploaded).

One comment is that there is 0.2% loss in class F (RA).

The proponent and other experts requested to revisit this proposal in track.

Initial recommendation of the BoG is to define a CE test for this proposal in addition to method in current CE3-3.1.1 test.

JVET-N0303 CE3-related: Simplified unified luma intra mode coding [S. Blasi, A. Seixas Dias, G. Kulupana (BBC)]

In JVET-M0528, a unification of the luma MPM list construction process is proposed. A flag is used to indicate if an intra mode is angular or not. Non-angular modes are coded independently using separate syntax elements. The MPM list is constructed only for angular modes. This contribution proposes to further modify the luma intra coding of angular modes so that it may not be necessary to compute the MPM under certain conditions. In particular, in case the top and left neighbouring blocks are encoded with non-angular modes, the MPM list as in JVET-M0528 only contains predefined default values. This contribution proposes to avoid using such MPM list filled with default values, and instead encode the intra mode directly. Moreover, an additional modification to coding of directional modes is proposed. In the VVC WD, the process of decoding a mode that is not in the MPM list still depends on the computation of the MPM list and sorting of the MPM modes. This contribution proposes to remove this dependency altogether, allowing the process of decoding modes that are not in the MPM list to be performed with no need to evaluate the MPM list. The proposed changes, implemented in VTM-4.0 and evaluated under common test conditions, reportedly provide average luma BD-rate gains of -0.19% for 101% encoder complexity and 98% decoder complexity in the all-intra configuration. It is also reported that for class A1 sequences in AI configuration the proposed scheme provides average BD rate luma gains of -0.46% for no additional encoder complexity and 98% decoder complexity.

Built on top of CE3-4 unification method.

Proposal method 1: In case that no angular mode is available, skip MPM computation and encode intra mode idx as intra luma remainder, using 6 to 7 bits. Avoid usage of this list and only compute MPM list when at least one angular mode is available.

There is some encoder runtime increase (2-3%). The proponent claims that a more efficient implementation would show no increase. It was commented that an additional full RD check is happening when L and A are planar (VTM4 would add 1, while proposal adds 2).

Proposal method 2: Implemented on top of method 1. If angular mode is not in the MPM list, parsing still depends on the computation of the MPM list. Proposed to decouple this. All angular modes are allowed to be coded with intra_luma_mpm_remainder.

BoG recommends to define two CE tests for method 1 and method 1+2, excluding the intra search modification.

JVET-N0305 CE3-related: Luma intra mode coding with restricted MPM derivation [S. Blasi, A. Seixas Dias, G. Kulupana (BBC)]

In the VVC WD, the process of decoding a mode that is not in the MPM list depends on the computation of the MPM list and sorting of the MPM modes. This contribution proposes to remove this dependency, allowing the process of decoding modes that are not in the MPM list to be performed with no need to evaluate the MPM list. An average BD-rate of 0.06% is reported for AI configuration with an encoder complexity of 98%. The reported decoder complexity is 95% on average.

Results provided on top of VTM4 MPM construction. Proposed to remove dependency on parsing modes that are not in MPM list. Some complexity savings in decoder (decoding times 94% are reported for AI).

It is simplification of VTM4 MPM construction.

It is commented that there is parsing dependency in case this is applied to method 1.

JVET-N0531 Crosscheck of JVET-N0305 (CE3-related: Luma intra mode coding with restricted MPM derivation) [S. De-Luxán-Hernández (HHI)] [miss] [late]

JVET-N0394 CE3-related: Unified MPM list based on CE3-3.3 and CE3-3.5.1 [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

In this proposal, a unified MPM list derivation is proposed, which is combination of CE3-3.3.2/CE3-3.3.3 and CE3-3.5.1. With the proposed method, the angular MPMs used in CE3-3.3.2/CE3-3.3.3 are replaced by first 5 or 6 angular MPMs derived in CE3-3.5.1. Furthermore, the modification of MPM list construction together with the encoder optimization employed in proposal JVET-N0450 are also combined in this proposal. It is reported that the combined test 7 obtains 0.25% coding gain for the AI configuration on top of VTM-4.0 with around 96% encoding time and 100% decoding time.

Features from CE3-3.3 and CE3-3.5.1 are combined.

BoG recommends defining CE test for combination, while encoder optimizations are not tested.

JVET-N0583 Cross-check of JVET-N0394: CE3-related: Unified MPM list based on CE3-3.3 and CE3-3.5.1 [X. Xiu (Kwai Inc.)] [miss] [late]

JVET-N0412 CE3-related: Modification on the intra luma mode coding process [Y. Kidani, K. Kawamura, K. Unno, S. Naito (KDDI)]

In the current VVC WD, the intra luma mode coding process is complicated due to the adoption of various intra predictions such us multiple reference line (MRL) and intra prediction and intra sub-partition (ISP) prediction. For this matter, various simplification approaches are proposed in previous meeting, and as one of those approach, the simplification method with new syntax identifying whether valid or invalid of angular and planar prediction was proposed and are going to be evaluated in CE3-3.4 and CE3-3.5 of this meeting. In this contribution, simplification of MPM list construction by partial unifying the lists is proposed. The simulation results show the BD-rate saving for luma by -0.11% and -0.03% in AI and RA over VTM-4.0 respectively. In the case of fully R-D encoder checking all MPMs, the BD-rates of proposal 1 become -0.13% and -0.05% in AI and RA respectively.

Builds upon CE3-3.4.

Option 1: 1 MPM list are removed

Option 2: in addition 3 MPM lists are removed

It was commented that there is dependency on ISP flag. Question is whether this is harmonization in this case.

BoG recommends testing option 2 in CE test.

JVET-N0433 CE3-related: Unification of MPM derivation for luma intra modes [A. K. Ramasubramonian, G. Van der Auwera, L. Pham Van, M. Karczewicz (Qualcomm)]

This document proposes a combined MPM list derivation for luma intra mode coding. A set of six MPM candidates and two replacement modes are derived with starting point as the MPM list in VTM-4.0 for blocks predicted from reference line with index equal to 0. Modes that are disallowed for certain coding blocks are removed and replacement modes are added to the end of the MPM list as needed. This is the case for a block predicted from a reference line with index greater than 0, where both DC and planar modes are disallowed;in this case two replacement modes are added at the end of the MPM list, so that the MPM list contains 6 modes. Similarly, for a block coded with ISP, the DC mode is disallowed and in this case one replacement mode is added to the end of the list. The signalling of the MPM index, contexts and encoder RD-checks are identical to that in VTM-4.0. It is asserted that the proposed derivation is simpler and provides gains on top of the VTM-4.0. The BD-rate performance of the proposed modifications with respect to VTM-4.0 are as follows: (−0.11%, −0.03%, −0.03%) for (Y, Cb, Cr) with runtimes of 101% EncT and 101% DecT for the AI configuration and (−0.03%, 0.02%, −0.16%) for (Y, Cb, Cr) with runtimes of 99% EncT and 99% DecT for the RA configuration. Under the modified encoder test condition described in Section 6.3 of JVET-M1023, the BD-rate performance of the proposed modifications with respect to VTM-4.0 are as follows: (−0.04%, −0.07%, 0.01%) for (Y, Cb, Cr) with runtimes of 101% EncT and 100% DecT for the AI configuration and (−0.01%, −0.03%, 0.03%) for (Y, Cb, Cr) with runtimes of 101% EncT and 100% DecT for the RA configuration.

BoG recommends defining one CE test for the proposed method.

JVET-N0562 Crosscheck of JVET-N0433 (CE3-related: Unification of MPM derivation for luma intra modes) [M. G. Sarwer (MediaTek)] [miss] [late]

JVET-N0450 CE3-3.4-related: unified MPM list construction [F. Bossen, K. Misra (Sharp)]

This contribution considers a variation of experiment CE3-3.4. CE3-3.4 proposes an approach to coding intra modes by signaling DC and planar modes separate from the angular modes. The angular modes are signalled using a MPM list. Here, normative constants are modified in that MPM list construction process and are asserted to have the benefit of: (i) enabling an implementation that is even more compact than the one described in JVET-N0436, (ii) reducing the number of pages required to express the list construction process to approximately 1 page of specification text, (iii) provide a coding efficiency benefit, and (iv) reduce encoder run time. Specifically, the coding efficiency relative to VTM 4 for the proposed method, including some encoder adjustments, is -0.27% (AI) and -0.09% (RA) with a slight reduction in encoder run time of about 3% (AI). Additional results are provided where dependency on the ISP mode is removed. It is asserted that this results in further simplification without an impact on coding efficiency (-0.26% for AI instead of -0.27%).

It was remarked that AI class F does not have loss and what is the reason? It is likely additional contexts.

Without ISP dependency, the coding gain is similar.

Spec text was asserted to be compact. It was commented that it is difficult to read.

Proponent proposes to additionally allow DC, planar mode for all cases (incl. MRL, ISP).

BoG recommends in CE test to substitute CE3-3.4 with the proposed method that does not have the ISP dependency. An additional CE test would be defined that allows DC, planar mode be used for MRL, ISP.

In this case the test conditions would need to be agreed regarding the changes in VTM to allow using DC, planar mode for MRL, ISP. Two proponents suggest to remove the current restrictions from the VTM, which can be done with minimal code changes.

JVET-N0704 Crosscheck of JVET-N0450 (CE3-3.4-related: unified MPM list construction) [S. Blasi, A. Seixas Dias, G. Kulupana (BBC)] [miss] [late]

JVET-N0451 CE3-3.1.2: Harmonization on MPM list [J. Heo, L. Li, J. Choi, J. Choi, S. Yoo, J. Lim, S. Kim (LGE)]

This contribution reports the results of CE3-3.1.2, which is about the unification of MPM list generation described in JVET-M0815. All sub-tests use the same MPM list which is generated from the MPM list construction process for zero reference line. In CE3-3.1.2.a and CE3-3.1.2.b, one or two MPM candidates are removed for non-zero reference line and intra sub-partition (ISP) mode. In CE3-3.1.2.c, a unified MPM generation process is used for all intra coding including zero and non-zero reference line and ISP mode.

It was requested to present additional test results b and c included in this document.

Test b: allowing planar for MRL

Test c: allowing DC, planar for all

BoG recommends to continue CE3-3.1.2a as a new CE test, as well as additionally the proposed tests b, c.

## Other MPM topics

No presenter for following proposal (Dahua):

JVET-N0157

JVET-N0157 Non-CE3: MPM List Modification for Zero Reference Line [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

This contribution proposes five modifications to modify the MPM list order for zero reference line. All the proposed methods maintain almost the same MPM structure as VTM4.0, but give the priority to the Planar mode or DC mode under certain conditions.

No presenter

JVET-N0183 Non-CE3: Shared MPM list [A. M. Kotra, S. Esenlik, J. Chen, B. Wang, H. Gao (Huawei)]

This contribution proposes to extend the “shared merge, IBC” list concept to intra luma most probable mode (MPM) list construction.  This feature enables to share the same MPM list for all leaf coding units of one ancestor node in the CU split tree. Therefore, the shared MPM list feature relaxes the worst-case scenario of construction of luma MPM list for blocks as small as 4 x 4 samples. Moreover, it enables parallel intra mode derivation of blocks belonging to a given shared region.

Worst case becomes 1 MPM list per 32 samples. All blocks in shared MPM region can derive intra mode in parallel.

0.01% luma BD-rate average for AI is reported.

Doesn’t use MRL, ISP list construction if block belongs to region.

Other case is to have 3 MPM lists per 32 samples (constructed in parallel).

Proponent prefers to have single list to be shared.

Parallel region is same as in shared merge list.

It was commented that current VTM4 has no issue with hardware implementation of the MPM lists.

BoG recommends no action.

JVET-N0641 Crosscheck of JVET-N0183 (AHG16/Non-CE3: Shared MPM list) [Y.-C. Lin (MediaTek)] [miss] [late]

JVET-N0230 Non-CE3: Simplified intra mode candidates for ISP [J. Choi, J. Heo, S. Yoo, L. Li, J. Choi, J. Lim, S. Kim (LGE)]

In VTM4.0, Intra Sub-Partitions (ISP) method requires two additional 6 MPM list construction processes (one for horizontal split mode and the other for vertical split mode) to derive the intra mode candidates. This contribution proposes to remove the two additional 6 MPM lists construction process which are dedicated to ISP mode, and to assign a default intra prediction mode list based on ISP split mode. More precisely, planar and horizontal intra mode are assigned for horizontal split mode, and planar and vertical intra mode are assigned for vertical split mode.

Removes two additional 6 MPM lists construction process for ISP mode. Only 2 fixed intra mode candidates are used for ISP, signaled with one bit. MPM flag is used to identify what mode to use. The reported loss is small.

It is commented that it may not be a simplification, because one context is used. Also meaning of MPM flag is reused. There is also modification to transform. In addition, there are modifications to encoder search by relaxing the speedup constraint.

BoG recommends no action.

JVET-N0665 Crosscheck of JVET-N0230 (Non-CE3: Simplified intra mode candidates for ISP) [J. Lee, S.-C. Lim, H. Lee, J. Kang (ETRI)] [miss] [late]

JVET-N0426 Non-CE3: History-based intra MPM default angular modes derivation [Z. Zhang, P. Wennersten, R. Yu, J. Ström, R. Sjöberg (Ericsson)]

This contribution presents a History-based intra MPM list default angular modes derivation method for intra coding. An intra modes buffer is applied to store the intra prediction mode for decoded blocks. The intra modes buffer is updated following FIFO rule. Only angular intra prediction mode is stored in the intra modes buffer. Only unique mode is stored in the intra modes buffer. The intra modes buffer is updated through entire slice. The proposed method has a luma BDR impact for AI/RA/LDB of -0.05%/-0.06%/-0.03% compared to VTM4.0 and decoding time impact for AI/RA/LDB of 100%/100%/100% compared to VTM4.0.

Small gains reported for increase in complexity and not harmonization, which is the current direction that has been identified for MPM topics. BoG recommends no action.

## Block size restriction topic

BoG recommendations:

- Presentation of following proposals in track A for further discussion:

JVET-N0082

JVET-N0390

JVET-N0396

JVET-N0453

- Adoption of following proposal:

JVET-N0308

JVET-N0082 CE3-related: Constrained partitioning of chroma intra CBs [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

BoG recommends presentation in track A.

JVET-N0693 Crosscheck of JVET-N0082 (CE3-related: Constrained partitioning of chroma intra CBs) [C. Rosewarne (Canon)] [miss] [late]

JVET-N0164 CE3-related: Size restriction for CCLM [F. Chen, L. Wang (Hikvision)]

BoG recommends no action. By itself is not enough.

JVET-N0563 Crosscheck of JVET-N0164 (CE3-related: Size restriction for CCLM) [J. Choi (LGE)] [miss] [late]

JVET-N0215 Non-CE3: Intra prediction mode restriction in small blocks [D. Y. Lee, T. H. Kim, G. H. Park (KHU)]

BoG recommends no action. Performance gain is small.

JVET-N0660 Cross check of Non-CE3: Intra prediction mode restriction in small blocks (JVET-N0215) [M. W. Park (Samsung)] [late]

JVET-N0308 AHG16/Non-CE3: Restriction of the maximum CU size for ISP to 64x64 [S. De-Luxán-Hernández, B. Bross, T. Nguyen, V. George, B. Stabernack, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

128x64 or 64x128 blocks, luma is divided, but not chroma resulting in 64x32 and 32x64 chroma blocks. Not really used. Proposed to disable.

Similar proposals: JVET-N0362 (CE6), JVET-N0469.

BoG recommends adoption.

JVET-N0672 Cross-check of JVET-N0308 (AHG16/Non-CE3: Restriction of the maximum CU size for ISP to 64x64) [X. Cao (Hikvision)] [miss] [late]

JVET-N0376 AHG16/CE3-related: CCLM mode restriction for increasing decoder throughput [J.-Y. Jung, Y.-L. Lee (Sejong Univ.), D.-Y. Kim, W. J. Jeong (Chips&Media), S.-C. Lim, J. Kang (ETRI)]

Similar to JVET-N0164. BoG recommends no action. By itself is not enough.

JVET-N0705 Cross-check of JVET-N0376 (AHG16/CE3-related: CCLM mode restriction for increasing decoder throughput) [G. Laroche (Canon)] [miss] [late]

JVET-N0390 AHG16/non-CE3: Study of CCLM restrictions in case of separate luma/chroma tree [E. François, F. Galpin, F. Le Léannec, T. Poirier (Technicolor)]

CCLM in dual tree. Problem is latency in processing chroma samples. Restrictions proposed on CCLM in dual tree. Chroma blocks <=16x16 and luma blocks within <=32x32 region. Performance penalty 0.28%(Y), 2.57%(U), 2.56%(V) for AI, 0.05%(Y), 1.25% (U), 1.04% (V) for RA. Comment is that dual tree seemed to work because of CCLM. This proposal addresses different block size restrictions from proposals above and support is voiced for further presentation in track.

BoG recommends presentation in track.

Cross-check in JVET-N0792

JVET-N0396 CE3-related: Enabling parallel reconstruction of small intra-coded blocks [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, H. Huang, M. Karczewicz (Qualcomm)]

Directly related CE3-2.1.3.

BoG recommends presentation in track.

JVET-N0682 Crosscheck of JVET-N0396 (CE3-related: Enabling parallel reconstruction of small intra-coded blocks) [?? (??)] [miss] [late]

JVET-N0453 AHG16/Non-CE3: on chroma 2xN and Nx2 intra prediction in single tree [Y. Zhao, A. Karabutov, H. Yang, J. Chen (Huawei)]

Related to JVET-N0082. Slightly better RD performance.

BoG recommends presentation in track A.

JVET-N0686 Crosscheck of JVET-N0453 Test 1 (AHG16/Non-CE3: on chroma 2xN and Nx2 intra prediction in single tree) [C. Rosewarne (Canon)] [late]

JVET-N0465 CE3-related: Restrict 2xN, Nx2 chroma processing for dual tree structure [H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)]

Related to CE3-2.2.1. Addresses dual tree small block size restriction only.

BoG recommends no action.

JVET-N0588 Crosscheck of JVET-N0465 (CE3-related: Restrict 2xN, Nx2 chroma processing for dual tree structure) [T. Zhou (Sharp)] [miss] [late]

JVET-N0469 CE3-related: Disable ISP mode for 128xN, Nx128 Block [H. Jang, J. Heo, J. Nam, J. Lim, S. Kim (LGE)]

See JVET-N0308

## ISP topic

BoG recommendations:

- CE3 sub-test on ISP simplification including tests from following proposals:

JVET-N0159

JVET-N0186

JVET-N0313

JVET-N0330

JVET-N0372

JVET-N0432

- Presentation in track A of following proposals for further discussion:

JVET-N0358

JVET-N0437

JVET-N0159 Non-CE3: Relaxation method of processing dependency in ISP for small blocks [K. Unno, K. Kawamura, Y. Kidani, S. Naito (KDDI)]

Addresses 1xN, 2xN complexity topic.

BoG recommends presentation in track.

JVET-N0530 Crosscheck of JVET-N0159 (Non-CE3: Relaxation method of processing dependency of ISP for small blocks) [S. De- Luxán-Hernández (HHI)] [miss] [late]

JVET-N0186 CE3-related: Simplification on Intra sub-partition coding mode [B. Wang, A. M. Kotra, S. Esenlik, H. Gao, J. Chen (Huawei)]

One test disables 1xN, other test disables 1xN, 2xN.

BoG recommends presentation in track.

JVET-N0728 Crosscheck of JVET-N0186 (AHG16/Non-CE3: Simplification on Intra sub-partition coding mode) [Y.-W. Chen (Kwai Inc.)] [miss] [late]

JVET-N0224 CE3-related: ISP support for 4:2:2 and 4:4:4 [L. Li, M. Koo, J. Heo, J. Nam, J. Lim, S. Kim (LGE)]

Requested to be moved to section 4.4.3 in the meeting report on chroma sampling and chroma format.

JVET-N0313 CE3-related: Low latency intra sub-partitions [R. Vanam, Y. He (InterDigital)]

Addresses prediction dependency. Employs predicted blocks instead of reconstructed block.

BoG recommends presentation in track.

JVET-N0719 Crosscheck of JVET-N0313 (Low latency intra sub-partitions) [R. Jullian (Technicolor)] [late]

JVET-N0330 Non-CE3: Intra sub-partition coding without thin partitions [T.-C. Ma, X. Xiu, Y.-W. Chen, X. Wang (Kwai Inc.)]

Proposes to remove 1xN, 2xN, Nx1 partitions.

BoG recommends presentation in track.

JVET-N0339 Non-CE3: Unification on WAIP for normal and ISP intra prediction [T.-C. Ma, X. Xiu, Y.-W. Chen, X. Wang (Kwai Inc.)]

Wide-angle harmonization proposed to depend on sub-partition dimensions, not CB.

It was commented it redefines wide-angle. However, intra prediction works on TB. There is inconsistency.

Similar proposals: JVET-N0452, JVET-N0427

It was commented that the benefit of this change is unclear.

BoG recommends no action.

JVET-N0581 Crosscheck of JVET-N0339 (Non-CE3: Unification on WAIP for normal and ISP intra prediction) [J. Heo, H. Jang, J. Lim, S. Kim (LGE)] [late]

JVET-N0358 Non-CE3: Harmonization of CBF coding for intra sub-partition coding mode [Z. Zhang, K. Andersson, R. Sjöberg (Ericsson)]

Currently there is restriction that at least one cbf has to be one, last one is inferred. Propose to signal all cbf for all sub-partition. Currently, some cases not available to encoder, however, it is claimed that experiments show that all cbf equal to 0 has benefits and is frequently selected. Proposal has about 0.04% (AI), 0.00% (RA).

It is commented that there may be encoder speedup, but seems within error margins. Overall, seems neutral regarding runtimes.

BoG recommends presentation in track.

JVET-N0372 Non-CE3: ISP with independent sub-partitions for certain block sizes [S. De-Luxán-Hernández, B. Bross, T. Nguyen, V. George, B. Stabernack, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

Enables ISP for 4x4, modifies 4x8 and 8x4. Parallel processing possible.

BoG recommends presentation in track.

JVET-N0536 Crosscheck of JVET-N0372 (Non-CE3: ISP with independent sub-partitions for certain block sizes) [X. Zhao (Tencent)] [miss] [late]

JVET-N0401 Non-CE3/Non-CE8: Enable Transform Skip in CUs using ISP [S. De-Luxán-Hernández, T. Nguyen, B. Bross, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

To be reviewed in CE8-related.

JVET-N0629 Crosscheck of JVET-N0401 (Non-CE3/Non-CE8: Enable Transform Skip in CUs using ISP) [T.-C. Ma (Kwai Inc.)] [miss] [late]

JVET-N0427 CE3-related: Harmonization between ISP and WAIP [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)] [late]

See JVET-N0339.

JVET-N0627 Crosscheck of JVET-N0427 (CE3-related: Harmonization between ISP and WAIP) [T.-C. Ma (Kwai Inc.)] [miss] [late]

JVET-N0432 AHG16/Non-CE3: On 1xN and 2xN subpartitions in intra subpartition coding [A. K. Ramasubramonian, G. Van der Auwera, T. Hsieh, V. Seregin, L. Pham Van, M. Karczewicz]

BoG recommends presentation in track.

JVET-N0726 Crosscheck of JVET-N0432 (AHG16/Non-CE3: On 1xN and 2xN subpartitions in intra subpartition coding) [N. Choi (Samsung)] [miss] [late]

JVET-N0437 Non-CE3: On allowing non-MPM modes for ISP and non-zero reference line [F. Bossen, K. Misra (Sharp Labs of America)]

MRLidx>0 and ISP limited to MPMs only. Proposed to remove restrictions, including DC and planar.

Currently it is normative limitation, perhaps future encoders can make use of it. Is there proof available that those modes are beneficial at all? There are reportedly some gains from doing this. However, perhaps gain comes from signaling changes.

BoG recommends presentation in track.

Cross-check in JVET-N0802. Reported that there is loss in performance if there is no encoder search change.

JVET-N0452 Non-CE3: Harmonization between WAIP and ISP [J. Heo, H. Jang, L. Li, J. Choi, J. Lim, S. Kim (LGE)]

See JVET-N0339

JVET-N0628 Crosscheck of JVET-N0452 (Non-CE3 : Harmonization between WAIP and ISP) [T.-C. Ma (Kwai Inc.)] [miss] [late]

JVET-N0475 Non-CE3/6: Enabling Transform Skip for ISP [T.-C. Ma, X. Xiu, Y.-W. Chen, X. Wang (Kwai Inc.)]

To be reviewed in CE8-related.

## Various intra topics

BoG recommendations:

- Adoption of the following proposals:

JVET-N0129, JVET-N0435

- Create CE test on CCLM model improvements:

JVET-N0524

- Editorial changes:

JVET-N0222

For following documents, there was no presenter (Dahua):

JVET-N0139

JVET-N0151

JVET-N0154

JVET-N0155

JVET-N0129 Non-CE3: PDPC simplification [N. Choi, M. W. Park, K. Choi (Samsung)]

Proposed to replace rounding operation with floor operation for interpolation of second boundary in near-diagonal mode case. It is reported that there is no impact on coding performance.

BoG recommends adoption.

JVET-N0644 Cross-check of JVET-N0129 (Non-CE3: PDPC simplification) [A. K. Ramasubramonian (Qualcomm)] [miss] [late]

JVET-N0132 Non-CE3: Simplification of DC Prediction [J.-Y. Huo, J.-L. Wang, X.-W. Li, Y.-Z. Ma, W. Zhang, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

Proposed to use 2 points for square block, 1 point for non-square block. It is reported that there is no performance loss.

It is commented that DC is one of the least complex modes and it may be not needed to simplify. What is the change in use of the DC mode? Loss is observed for the Campfire sequence.

BoG recommends no action.

JVET-N0657 Cross-check of JVET-N0132 (Non-CE3: Simplification of DC Prediction) [X. Ma (Huawei)] [miss] [late]

JVET-N0139 CE3-related: Intra Angular Prediction and Modified PDPC Based on Two Reference Lines [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

No presenter.

JVET-N0143 Non-CE3: Simplification of multi-line intra [T. Hashimoto, E. Sasaki, T. Ikai (Sharp)]

Proposes removing buffering of upper-left region. It is claimed that line buffer size is reduced.

It is commented that the simplification may not be significant.

BoG recommends no action.

JVET-N0151 CE3-related: Modified Chroma Derived Mode [D. Jiang, J. Lin, F. Zeng, C.Fang (Dahua)]

No presenter.

JVET-N0154 Non-CE3: Chroma Intra Default Modes Modification Based on Order of Luminance Modes [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

No presenter.

JVET-N0155 Non-CE3: Chroma Intra Default Modes Modification Based on Statistics of Luminance Modes [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

No presenter.

JVET-N0216 Non-CE3: Determination of wide-angle mode using the size of a coding block [D. Kim, J. Jung, G. Ko, J. Son, J. Kwak (WILUS)]

Related to WAIP and ISP subpartition inconsistency between usage of CB versus TB dimensions to determine angles used for prediction. Here, it is proposed to also address this issue in case CB size is larger than max TB size.

Similarly, the BoG recommends no action.

JVET-N0218 CE3-related: Chroma Intra Mode Coding with Lumped Luma Directions and Replacement for DM mode [J. Park, B. Jeon (SKKU)]

First it is proposed to replace mode 66 with the mode 2 diagonal mode if non-DM for blocks with W>H. Secondly, it is proposed for smaller chroma blocks, for example, to reduce the high angular resolution (AI -0.05% (Y), -0.05% (U), -0.06% (V); RA is still running).

BoG recommends no action.

JVET-N0723 Cross check of JVET-N0218 (CE3-related: Chroma Intra Mode Coding with Lumped Luma Directions and Replacement for DM mode) [N. Choi (Samsung)] [miss] [late]

JVET-N0222 CE3-related: CIIP intra mode propagation to intra block [L. Li, J. Nam, J. Heo, J. Lim, S. Kim (LGE)]

It is claimed that there is mismatch between spec and SW. In the spec, during luma MPM list derivation the CIIP intra mode is propagated to the current intra block used, while in the software it is not. It is proposed for the spec to match the software, and it simplified the spec.

It is commented that mismatches are better reported as spec bug.

BoG recommends for the spec to match the software (editorial).

JVET-N0223 CE3-related: reference sample filtering simplification [L. Li, J. Heo, J. Lim, S. Kim (LGE)]

It is claimed that there is mismatch between spec and SW regarding reference sample filtering. However, it is commented by two experts that there is no mismatch.

Proposed to simplify the conditions for reference sample filtering. It is reported there is -0.02% BD-rate change in AI on average.

It is commented that what is proposed is not a simplification. There do not seem to be other benefits, besides simplifying the spec text. However, the spec text should be rewritten to clarify the reference sample filtering conditions first.

BoG recommends no action.

JVET-N0229 Non-CE3: CCLM prediction for 4:2:2 and 4:4:4 color format [J. Choi, J. Heo, J. Lim, S. Kim (LGE)]

Requested to be moved to section 4.4.3 in the meeting report on chroma sampling and chroma format.

JVET-N0248 Non-CE3: Combined-Hypothesis Intra-Prediction with Unified Intra Mode Coding [G. Kulupana, A. Seixas Dias, S. Blasi (BBC)]

A new combined-hypothesis mode is proposed combing planar with angular mode. Last meeting, there were concerns expressed regarding parsing dependency and this was solved by using JVET-M0528 (CE3-3.4). It is reported that BD-rate benefit is -0.22% (AI) with 105% encoder runtime increase. It is commented that M0528 gives 0.1% gain, so really the BD-rate benefit is -0.12%. The gain/encoder runtime trade-off seems not good.

BoG recommends no action.

Cross-check in JVET-N0774

JVET-N0333 CE3-related: MMLM only cross-component prediction [H.-J. Jhu, Y.-W. Chen, X. Wang (Kwai Inc.)]

MMLM was not retained from CE3-1.

BoG recommends no action.

JVET-N0607 Cross-check of JVET-N0333, "CE3-related: MMLM only cross-component prediction" [?? (??)] [miss] [late]

JVET-N0342 Non-CE3: Decoder-side Intra Mode Derivation with Prediction Fusion [M. Abdoli, E. Mora, T. Guionnet, M. Raulet (ATEME)]

Sample gradient analysis applied on reference samples is used to derive intra mode. In this proposal, prediction fusion has been added, which allows to use more than one mode without signaling. A histogram allows to select three modes (3 tallest), those are applied separately resulting in 3 prediction blocks that are combined in a weighted averages. Coding benefit is -0.4% gain for AI with 7% decoder runtime increase, while encoder runtime is increased 21% due to mode checking.

Method is applied to all block sizes. This may require many cycles at the decoder side. Suggested to disable 4x4 blocks.

It was commented that a full set of results are required, in particular RA. Further study is suggested to reduce runtimes.

BoG recommends no action.

JVET-N0590 Crosscheck of JVET-N0342 (Decoder-side Intra Mode Derivation with Prediction Fusion) [?? (??)] [miss] [late]

JVET-N0371 Non-CE3: directional intra prediction with varying angle [G. Rath, F. Urban, F. Racapé (Technicolor)]

The block is split into two subparts during prediction stage. Applied when MRLidx is 1 or 3. One of the subparts has angular mode +1 or -1 the signalled angular mode. Coding benefit is -0.11% AI, 108% encoder runtime. The trade-off between coding gain and runtimes seems not interesting at this time.

BoG recommends no action.

JVET-N0591 Crosscheck of JVET-N0371 (Non-CE3: directional intra prediction with varying angle) [P. Merkle (HHI)] [late]

JVET-N0435 Non-CE3: Harmonization between WAIP and intra smoothing filters [P.-H. Lin, Y.-C. Yang (Foxconn)]

In current VTM, for modes 2, 34, 66 only integer sample copy is used, while for wide-angle modes there are several modes that could benefit from integer sample copy (8 modes), while in VTM the zero-phase of the interpolation filters (DCT-IF, Gaussian) is used. Intra reference sample conditions are unmodified.

It is commented that this is useful harmonization. There is no impact on the coding gain, although the proposed change is not completely lossless. The reason is that PDPC is applied to potentially smoothed samples, while in current VTM those samples are unfiltered.

Support is voiced to adopt this proposal, because it can be considered as a harmonization or cleanup of the current inconsistency regarding the treatment of the integer-slope angles.

BoG recommends adoption.

Cross-check in JVET-N0777

JVET-N0524 Non-CE3: Modified beta derivation in CCLM [M. Ikeda, T. Suzuki (Sony)] [miss] [late]

Proposed to modify beta derivation to improve coding efficiency using the average luma and chroma value, which is somewhat more complex than the current min/max method. Coding -0.11% (Y), -0.40% (U), -0.39% (V) for AI; -0.08% (Y), -0.35% (U), -0.45% (V). This meeting the number of reference samples used in the CCLM model derivation is reduced. The proponent suggests to study this method together with the adopted simplification.

Two experts voice support to create a CE test to study this method together with the adopted CCLM simplification of number of reference samples.

JVET-N0694 Crosscheck of JVET-N0524 (Non-CE3: Modified beta derivation in CCLM) [Y. Kidani, K. Kawamura, K. Unno, S. Naito (KDDI)] [late]