| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0840-v1 |
| --- | --- |

| Title: | BoG report on Neural Network based Filter for Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yiming Li | Email: | userlym@whu.edu.cn |
| Source: | BoG |  |  |

_____________________________

# Abstract

This BoG met on 23 Mar 2019 from 09:00 to 12:00 to review the CE13-related contributions of the 14th JVET meeting. The subjective viewing session and the discussion of the next CE plan are included in this BoG.

-The following BoG recommendations were made:

- Adoptions:

  - X

- CE tests:

  - JVET-N0133, JVET-N0712

- Open issues:

  - X

-For the methods in this CE, several problems are suggested to be investigated and answered in the next CE:

1) In inference stage, investigating the performances change when the parameters are in the different bit-depths. Some typical values of the bit depth include 4/6/8/16. The method to get the integer parameters from floating point value depends on the proponents.

2) Investigating the difference of the performance and subjective quality when treating the NN filter as the in-loop filter and post filter. The behavior of post filter might be indicated in SEI message. Since the subjective viewing should be considered in this subtest, full test is preferred instead of only testing the first RAS.

3) Investigating the performance impact when using the following two different methods to get the integer parameters. When doing this investigation, bit-depth of the converted integer parameters should be the same:

Method 1: The parameters that obtained in the training stage is floating point value first. And then they are converted to integer value from the floating point value.

Method 2: Using quantization-aware training to get the integer parameters directly.

4) Investigating the performance change when the test QP is far more different from the training QP. The difference of test QP and training QP should be larger than 5.

-General comments on the test sequences.

In general, no much difference can recognized.

Discussion of CE13-related/AHG9 proposals

JVET-N0133 CE13-related: Integrated in-loop filter based on CNN[S. Wan, M.-Z.Wang, H. Gong, C.-Y. Zou(NPU), Y.-Z. Ma, J.-Y. Huo(Xidian Univ.), Y.-F. Yu, Y. Liu(OPPO)]

This contribution uses CNN filter to replace all the traditional filter. The experiments are only conducted in AI configuration, which can be seen as the post processing filter. The proponent of this network will use it as an in-loop filter in the next stage. It is also suggested it can be treated as a post filter even in RA/LD configuration.

The input of the network includes: Y/U/V reconstruction, QP map of Y/UV components, and CU partition map.

Performance: -5.72%, -8.95%, -10.77% BD-rate savings for Y, Cb and Cr components compared with VTM4.0 under AI configuration.

It is asked the performance change if the CU partition map is not as the input of the network. The proponent claims the performance will drop about 0.1db in validation stage. The influence of the performance in BD-rate is unknown.

There is no CU level adaptive on/off in this version. The adaptive on/off will be added in the next version.

The 30% encoding time decrease is not reliable.

JVET-N0712 CE13-related: Adaptive CNN based in-loop filtering with boundary weights [K. Kawamura, Y. Kidani, S. Naito (KDDI)]

The filter is signaled as the side information in this contribution, which is similar as CE13-1.1 or CE13-1.2.

The weights according to the distance to the CU boundary are used. Different with JVET-N0133, the boundary information (weights table) is used to control the strength of the neural network filter. In the proposed design, the maximum influence range of CNN filter is restricted in 4 lines near the boundary. For the block smaller than 16*16, there is no zero-value weight. Zero-value weight will only occurs in the center of the blocks larger than 16*16.

The weights in intra is stronger than inter. It is asked, considering the phenomenon that the weight in inter is smaller than intra, does it mean the NN filter may do better in intra? Just guess, no data for evidence.

It is asked how much gain if using this weight? The proponent doesn’t have data about that.

It is pointed there is much more gain in chroma component, it is suggested to put the gain in luma component (e.g., by QP shift). This phenomenon also occurs in other sequence-dependent methods. The reason may due to luma information can help to increase the quality of chroma component.

It is asked why the performance is higher in high resolution? The reason is side information.

It is asked why the decoding time is much smaller than the CE test(CE13-2.6/7). The proponent claims it may due to the hit rate of this is contribution is lower. It is noticed the performance decreased to half of the CE test.

Batch normalization is used in this contribution.

It is asked how long of the training time? The proponent says maybe several hours.

JVET-N0065 AHG17 & AHG9: Comments on carriage of coding tool parameters in Adaptation Parameter Set[M. Li, P. Wu (ZTE)]

In 13th JVET meeting, Adaptation Parameter Set (APS) has been introduced into Versatile Video Coding (VVC) standard text. A few years ago, the APS was once adopted into High Efficiency Video Coding (HEVC) standard for carrying coding tool parameters, such as Adaptive Loop Filter (ALF) parameters; but the APS was removed from HEVC standard along with the removal of ALF as a coding tool at a final HEVC standardisation stage. The APS was designed to carry coding tool parameters as a picture level adaptive nature and alternatively the APS data can also be maintained as unchanged for the whole video sequence for avoiding resending them unnecessarily. This contribution is for information only and it is commented and recommended to use the APS to carry the coding tool data, such as neural networks parameters and affine linear weighted intra prediction parameters, etc., if some associated coding tools are indeed to be adopted into VVC standard. If more coding tool data need to be carried by APS in the future, it commented that further study on updating parameters of multiple tools using APS would be needed.

It is asked can APS support prediction between different APS? There is no restriction of the packing method in APS. For example, one can transmit the delta value.

APS has been used for ALF. It is picture level set.

For the case that parameters transmitted in the bitstream, the parameters are updated each RAS, currently it is put in tile group header. It can also be put into APS. Maybe it is better to take APS into consideration in the future.

For the case that index transmitted in the bit stream. It may be changed every tile group. Thus, it is also better to take the APS into consideration in the future.

Subjective Viewing Session

The informal viewing session is conducted at 12:30 after this BOG

In general, no much difference can recognized.

Comments on CE13-2.2: It is similar as the anchor, some details are different.

Comments on CE13-2.4: The proposed method has better quality in some part of the picture, while in some other part, the anchor is better. In the area including face (lips), the proposed method is better in some frames.

Comments on CE13-2.7: The comparison is between CE13-2.7a(CTU level adaptive on/off) and CE13-2.7b(always on). No difference can be seen.

Discussion of the Next CE Plan

It was suggested: it may be helpful to use a simpler network (e.g, keep the number of layers low), and switch the pre-trained and signaled parameters in the decoder side.

It is expected that:

-When the resolution is low, using pre-trained parameters to save the side information:

1) If there is only one set of the parameters, decoder can directly use it. (unlikely happen)

2) If there is more than one set of the parameters, encoder can just signal the index of which set of parameters need to use. Decoder will use the index to choose the correct set of the parameters.

-When the resolution is high, using on-line training or off-line training may depend on the one-pass or two-pass use case and coding configurations.

CE test plan: (optional)

1) In inference stage, investigating the performances change when the parameters are in the different bit-depths. Some typical values of the bit depth include 4/6/8/16. The method to get the integer parameters from floating point value depends on the proponents.

2) Investigating the difference of the performance and subjective quality when treating the NN filter as the in-loop filter and post filter. The behavior of post filter might be indicated in SEI message. Since the subjective viewing should be considered in this subtest, full test is preferred instead of only testing the first RAS.

3) Investigating the performance impact when using the following two different methods to get the integer parameters. When doing this investigation, bit-depth of the converted integer parameters should be the same:

Method 1: The parameters that obtained in the training stage is floating point value first. And then they are converted to integer value from the floating point value.

Method 2: Using quantization-aware training to get the integer parameters directly.

4) Investigating the performance change when the test QP is far more different from the training QP. The difference of test QP and training QP should be larger than 5.