| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0841 |
| --- | --- |

| Title: | BoG report on CE5-1.2 and CE5-2.1 complexity analysis |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

From the track A meeting notes:

- 5-1.2 and 5-2.1 would both solve the problem, where 5-1.2 has slightly better MOS average, however all confidence intervals are overlapping, such that they would be assessed as visually indistinguishable

- More complexity analysis to be done on these two proposals, one of them to be adopted.

BoG to analyse this.

BoG conclusion: after analysing the proposals, BoG concludes that the additional weighted averaging step, as in CE5-1.2, is an additional processing comparing to the other proposal CE5-2.1, while other aspects of the proposals are similar or the same in the implementation. There is no significant difference either in the subjective tests nor in the performance results, so BoG prefers the simpler CE5-2.1 method for the line buffer reduction.

# Introduction

VTM-4.0 operations for tLF:

| Test | Operations of classification process per luma sample |  | Operations of filtering process per luma sample |  | Operations of filtering process per chroma sample |  | Frequency change of filters (Y, Cb, Cr) | Line buffer size (L, C) | memory for storing filters, bytes | Additional operations per other units |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor | Add | 15.125 | Add | 25 | Add | 13 | 4x4 (Y) tile group (Cb) tile group (Cr) | 7 (L) 4 (C) | 10,624 | loading 332 bytes per tile group |
|  | Comp | 4.9375 | Comp | 4 | Comp | 1 |  |  |  |  |
|  | Mult | 0.125 | Mult | 13 | Mult | 7 |  |  |  |  |
|  | Shift | 2.5625 | Shift | 1 | Shift | 1 |  |  |  |  |

Proponents presented each method. Both proposals provide SIMD implementation and it was verbally reported that it matches the CE tests results.

In the both proposals, line buffer requirement is reduced to 0 for both luma and chroma components. The ALF filtering and classification require the same amount of operations as in the anchor, while additional processing is added for the blocks located at virtual boundaries (VB), this additional processing is analyzed in below.

# CE5-2.1.

Figure 1. Filtering processing at VB (CE5-2.1).

Figure 2. Modified ALF classification (CE5-2.1).

Block classification: a 4x4 block can be located above or below VB. In such cases, the gradients above/below VB are not used in the classification.

To distinguish whether a 4x4 block is at above or below VB, 2 condition checks are performed. The classification activity is changed to account different number of gradients used, it requires 1 shift and 1 add per 4x4 block. To calculate the gradients at the first/last row in 4x4 block, padding of 1 line of samples is required.

Filtering: Based on the line index at VB, padding is done for above and below sides.

Luma:

Line0: samples of 3 lines above 3 lines below are padded

Line1: samples of 2 lines above and 2 lines below are padded

Line2: samples of 1 lines above and 1 lines below are padded

Chroma:

Line0: samples of 2 lines above 2 lines below are padded

Line1: samples of 1 lines above and 1 lines below are padded

It was commented by the proponent that to avoid storing padded lines or padding process itself, in the implementation the condition check is performed per line to derive the input samples coming either from the “padded” or regular samples.

It was commented by the proponent of the alternative test, in another implementation, if no checking is performed or parallel filtering is performed for the entire 4x4 block then additional memory is needed.

It was further commented by a hardware expert (non-proponent) that “physical” copying is not performed.

Summary of the additional operations over VTM-4.0:

2 checks for above or below VB

Classification:

- 1add, 1shift for classification modification

- 1 line of samples is padded for gradient calculation

Filtering:

- 1 check for the input source per line for filtering

# CE5-1.2

Figure 3. Filtering of luma samples above the VB (CE5-1.2).

Figure 4. Filtering of luma samples below the VB (CE5-1.2).

Figure 5. Modification of accumulating gradients for a 4x4 block in VB processing (CE5-1.2).

Block classification: a 4x4 block can be located above or below VB. In such cases, the 2 lines of gradient are padded.

To distinguish whether a 4x4 block is at above or below VB, 2 condition checks are performed. To calculate the gradients at the first/last row in 4x4 block, padding of 1 line of samples is required.

Filtering: Based on the line index at VB, padding is done for above or below sides.

Luma:

Line0: samples of 3 lines above

Line1: reuse previously padded lines

Line2: reuse previously padded lines

Chroma:

Line0: samples of 2 lines above

Line1: reuse previously padded lines

It was commented by the proponent that the 3 lines of padding is performed, and the filtering is performed reusing anchor implementation.

It was commented by a hardware expert (non-proponent) that actual padding is not performed, that the input to the filtering can be thought as supplying a “pointer” to the filtering module, and it is done for the regular filtering already.

Weighted average: weights for filtered and unfiltered samples are selected based on the line index at VB. It requires 3bits x 2 weights x 4 entries = 24 bits for LUT storage, and 4 shifts with 4 adds are needed for the weighted average process. It introduces additional step in the filtering.

Summary of the additional operations over VTM-4.0:

2 checks for above or below VB

Classification:

- 1 line of samples is padded for gradient calculation

- 2 lines of gradients padding

Filtering:

- 1 check for the input source per line for filtering, or samples of 3 lines for luma and 2 lines for chroma are padded

- LUT 24 bits, 4 shifts and 4 adds for weighted averaging

# Recommendation

After analysing the proposals, BoG concludes that the additional weighted averaging step, as in CE5-1.2, is an additional processing comparing to the other proposal CE5-2.1, while other aspects of the proposals are similar or the same in the implementation. There is no significant difference either in the subjective tests nor in the performance results, so BoG prefers the simpler CE5-2.1 method for the line buffer reduction.

# References

- CE 5-1.2: “CE5-1: Adaptive loop filter with virtual boundary processing”, JVET-N0088

- CE 5-2.1: “CE5-2: Loop filter line buffer reduction”, JVET-N0180