| [1]Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 Mar. 2019 | Document: JVET-N0849-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-M0475 : Non-CE3/6: Enabling Transform Skip for ISP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck of a proposal |  |  |
| Author(s) or Contact(s): | Santiago De-Luxán-Hernández Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-511 santiago.de.luxan@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution reports the results of the crosscheck of the JVET-M0475 (Non-CE3/6: Enabling Transform Skip for ISP).

# Experimental results

The crosscheck has been performed for an intra-only (AI) and a random-access (RA) configuration with the VTM-4.0 software. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and a GCC 7.2.1 compiler. The test includes TGM sequences, given that they were tested by the proponents in [1].

Table 1: Results with an AI configuration on top of VTM-4.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.06% | -0.03% | 103% | 100% |
| Class A2 | -0.01% | 0.11% | -0.06% | 103% | 100% |
| Class B | -0.01% | 0.00% | 0.05% | 103% | 101% |
| Class C | -0.02% | -0.09% | -0.06% | 104% | 100% |
| Class E | -0.01% | 0.04% | 0.01% | 103% | 100% |
| Overall | -0.01% | 0.01% | -0.01% | 103% | 100% |
| Class D | -0.01% | 0.00% | -0.20% | 104% | 102% |
| Class F | -0.19% | -0.08% | -0.14% | 103% | 101% |
| Class TGM | -0.25% | -0.12% | -0.14% | 104% | 100% |

Table 2: Results with a RA configuration on top of VTM-4.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | -0.05% | 100% | 98% |
| Class A2 | -0.02% | -0.09% | 0.08% | 100% | 99% |
| Class B | 0.00% | -0.05% | -0.16% | 100% | 100% |
| Class C | -0.01% | -0.13% | -0.42% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.07% | -0.16% | 100% | 99% |
| Class D | -0.03% | -0.10% | -0.30% | 100% | 98% |
| Class F | -0.17% | -0.17% | -0.28% | 100% | 100% |
| Class TGM | -0.25% | -0.08% | -0.06% | 100% | 102% |

# Conclusion

Results match the ones presented by the proponents in [1]. Encoder and decoder run times are also very close to the ones showed in the original proposal.

# References

| [1] | T.-C. Ma, X. Xiu, Y.-W. Chen and X. Wang, "Non-CE3/6: Enabling Transform Skip for ISP," Document JVET-N0475, Geneva, CH, 2019. |
| --- | --- |