| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0851-v2 |
| --- | --- |

| Title: | CE4-related: On merge list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu1, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei (MediaTek) Yao-Jen Chang2, Chun-Chi Chen, Wei-Jung Chien, Marta Karczewicz (Qualcomm) | Email: | 1cw.hsu@mediatek.com 2yjchang@qti.qualcomm.com |
| Source: | MediaTek and Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents changes to VVC WD Text to remove constraints on the merge candidate derivation process. In current VVC, the decoder must check whether the number of derived merge candidates is less than the max number of merge candidates. If yes, the merge candidate derivation process continues; otherwise, it stops. However, such constraint makes a decoder unable to construct the merge candidate list in advance (e.g. simply deriving the whole set of merge candidates and selecting one pointed to by a given merge index). Accordingly, this proposal proposes to remove this constraint imposed by the max number of merge candidates in the merge candidate derivation process. On top of the proposed method, this proposal suggests a method in JVET-N0400 for TPM (Triangle Partition Mode) merge candidate size signaling which indicates the number of maximum TPM candidate size explicitly at tile-group level and specifies syntax binarization for TMP merge indices.

# Proposed WD change

The proposed WD are drafted on top of JVET-M1001-v7. The changes as summarized below are marked in yellow.

8.5.2.2 Derivation process for luma motion vectors for merge mode

….

7. When numCurrMergeCand is less than (MaxNumMergeCand - 1)5 and smrNumHmvpCand is greater than 0, the following applies:

- The derivation process of history-based merging candidates as specified in 8.5.2.6 is invoked with mergeCandList, isInSmr set equal to IsInSmr[ xCb ][ yCb ], and numCurrMergeCand as inputs, and modified mergeCandList and numCurrMergeCand as outputs.

- numOrigMergeCand is set equal to numCurrMergeCand.

- When numCurrMergeCand is less than 6 MaxNumMergeCand and greater than 1, the following applies:

- The derivation process for pairwise average merging candidate specified in clause 8.5.2.4 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, numCurrMergeCand and numOrigMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0avgCand and refIdxL1avgCand, the prediction list utilization flags predFlagL0avgCand and predFlagL1avgCand and the motion vectors mvL0avgCand and mvL1avgCand of candidate avgCand being added into mergeCandList. The bi-prediction weight index gbiIdx of candidate avgCand being added into mergeCandList is set equal to 0.

….

8.5.2.5 Derivation process for zero motion vector merging candidates

Inputs to this process are:

- a merging candidate list mergeCandList,

- the reference indices refIdxL0N and refIdxL1N of every candidate N in mergeCandList,

- the prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N in mergeCandList,

- the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList.

Outputs of this process are:

- the merging candidate list mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm of every new candidate zeroCandm added into mergeCandList during the invocation of this process,

- the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm of every new candidate zeroCandm added into mergeCandList during the invocation of this process,

- the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm added into mergeCandList during the invocation of this process.

The variable numRefIdx is derived as follows:

- If tile_group_type is equal to P, numRefIdx is set equal to NumRefIdxActive[ 0 ].

- Otherwise (tile_group_type is equal to B), numRefIdx is set equal to Min( NumRefIdxActive[ 0 ], NumRefIdxActive[ 1 ] ).

When numCurrMergeCand is less than 6 MaxNumMergeCand, the variable numInputMergeCand is set equal to numCurrMergeCand, the variable zeroIdx is set equal to 0 and the following ordered steps are repeated until numCurrMergeCand is equal to 6 MaxNumMergeCand:

….

8.5.2.6 Derivation process for history-based merging candidates

Inputs to this process are:

- a merge candidate list mergeCandList,

- a variable isInSmr specifying whether the current coding unit is inside a shared mergign candidate region,

- the number of available merging candidates in the list numCurrMergeCand.

Outputs to this process are:

- the modified merging candidate list mergeCandList,

- the modified number of merging candidates in the list numCurrMergeCand.

The variables isPrunedA1 and isPrunedB1 are both set equal to FALSE.

The array smrHmvpCandList and the variable smrNumHmvpCand are derived as follows:

smrHmvpCandList = isInSmr  ?  HmvpSmrCandList  :  HmvpCandList	(8-353)

smrNumHmvpCand = isInSmr  ?  NumHmvpSmrCand  :  NumHmvpCand	(8-354)

For each candidate in smrHmvpCandList[ hMvpIdx ] with index hMvpIdx = 1..smrNumHmvpCand, the following ordered steps are repeated until numCurrMergeCand is equal to 5 (MaxNumMergeCand - 1):

….

8.5.2.16 Updating process for the history-based motion vector predictor candidate list

Inputs to this process are:

- luma motion vectors in 1/16 fractional-sample accuracy mvL0 and mvL1,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1,

- bi-prediction weight index gbiIdx.

The MVP candidate hMvpCand consists of the luma motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction list utilization flags predFlagL0 and predFlagL1, and the bi-prediction weight index gbiIdx.

The candidate list HmvpCandList is modified using the candidate hMvpCand by the following ordered steps:

- The variable identicalCandExist is set equal to FALSE and the variable removeIdx is set equal to 0.

- When NumHmvpCand is greater than 0, for each index hMvpIdx with hMvpIdx = 0..NumHmvpCand − 1, the following steps apply until identicalCandExist is equal to TRUE:

  - When hMvpCand is equal to HmvpCandList[ hMvpIdx ], identicalCandExist is set equal to TRUE and removeIdx is set equal to hMvpIdx.

- The candidate list HmvpCandList is updated as follows:

  - If identicalCandExist is equal to TRUE or NumHmvpCand is equal to 5MaxNumMergeCand - 1, the following applies:

  - For each index i with i = ( removeIdx + 1 )..( NumHmvpCand − 1 ), HmvpCandList[ i − 1] is set equal to HmvpCandList[ i ].

  - HmvpCandList[ NumHmvpCand − 1 ] is set equal to mvCand.

  - Otherwise (identicalCandExist is equal to FALSE and NumHmvpCand is less than 5 MaxNumMergeCand - 1), the following applies:

  - HmvpCandList[ NumHmvpCand++ ] is set equal to mvCand.

….

8.6.2.2 Derivation process for luma motion vector for merge mode

This process is only invoked when merge_flag[ xCb ][ yPb ] is equal to 1 and CuPredMode[ xCb ][ yPb ] is equal to MODE_IBC, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vectors in 1/16 fractional-sample accuracy mvL.

The luma motion vector mvL is derived by the following ordered steps:

- The derivation process for merging candidates from neighbouring coding units as specified in clause 8.6.2.3 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, and the luma coding block height cbHeight as inputs, and the outputs being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2 and the motion vectors mvA0, mvA1, mvB0, mvB1 and mvB2.

- The merging motion vector candidate list, mergeMvCandList, is constructed as follows:

i = 0
if( availableFlagA1 )
	mergeMvCandList [ i++ ] = mvA1
if( availableFlagB1 )
	mergeMvCandList [ i++ ] = mvB1
if( availableFlagB0 ) 		(8-977)
	mergeMvCandList [ i++ ] = mvB0
if( availableFlagA0 )
	mergeMvCandList [ i++ ] = mvA0
if( availableFlagB2 )
	mergeMvCandList [ i++ ] = mvB2

- The variable numCurrMergeCand is set equal to the number of merging candidates in the mergeMvCandList.

- When numCurrMergeCand is less than 5 (MaxNumMergeCand - 1) and NumHmvpIbcCand is greater than 0, the derivation process of history-based merging candidates as specified in 8.6.2.5 is invoked with mergeMvCandList, and numCurrMergeCand as inputs, and modified mergeMvCandList and numCurrMergeCand as outputs.

- When numCurrMergeCand is less than MaxNumMergeCand 6 and greater than 1, the derivation process for pairwise average merging candidate specified in clause 8.6.2.4 is invoked with mergeMvCandList and numCurrMergeCand as inputs, and the outputs are assigned to mergeMvCandList and numCurrMergeCand.

[Ed. (SL): Alert that the merging motion vector candidate list may not be full. In rare case the merge index may point to an empty entry of the merging motion vector candidate array.]

- The following assignments are made:

mvL[ 0 ] = mergeMvCandList[ merge_idx[ xCb ][ yCb ] ][ 0 ]	(8-978)

mvL[ 1 ] = mergeMvCandList[ merge_idx[ xCb ][ yCb ] ][ 1 ]	(8-979)

….

8.6.2.5 Derivation process for history-based merging candidates

Inputs to this process are:

- a merge candidate list mergeCandList,

- the number of available merging candidates in the list numCurrMergeCand.

Outputs to this process are:

- the modified merging candidate list mergeCandList,

- the modified number of merging candidates in the list numCurrMergeCand.

The variables isPrunedA1 and isPrunedB1 are set both equal to FALSE.

For each candidate in HmvpIbcCandList[ hMvpIdx ] with index hMvpIdx = 1..NumHmvpIbcCand, the following ordered steps are repeated until numCurrMergeCand is equal to 5 ( MaxNumMergeCand -1):

….

8.6.2.10 Updating process for the history-based motion vector predictor candidate list

Inputs to this process are:

- luma motion vector mvL in 1/16 fractional-sample accuracy.

The candidate list HmvpIbcCandList is modified by the following ordered steps:

- The variable identicalCandExist is set equal to FALSE and the variable removeIdx is set equal to 0.

- When NumHmvpIbcCand is greater than 0, for each index hMvpIdx with hMvpIdx = 0..NumHmvpIbcCand − 1, the following steps apply until identicalCandExist is equal to TRUE:

  - When hMvpCand is equal to HmvpIbcCandList[ hMvpIdx ], identicalCandExist is set equal to TRUE and removeIdx is set equal to hMvpIdx.

- The candidate list HmvpIbcCandList is updated as follows:

  - If identicalCandExist is equal to TRUE or NumHmvpIbcCand  is equal to 5 MaxNumMergeCand -1, the following applies:

  - For each index i with i = ( removeIdx + 1 )..( NumHmvpIbcCand  − 1 ), HmvpIbcCandList[ i − 1] is set equal to HmvpIbcCandList [ i ].

  - HmvpIbcCandList[ NumHmvpIbcCand − 1 ] is set equal to mvCand.

  - Otherwise (identicalCandExist is equal to FALSE and NumHmvpIbcCand  is less than 5 MaxNumMergeCand -1), the following applies:

  - HmvpIbcCandList[ NumHmvpIbcCand ++ ] is set equal to mvCand.

….

# Suggested method of TPM candidate size signaling

This proposal suggests a method of TPM candidate size signaling as proposed in JVET-N0400. The WD text of JVET-N0400 is quoted below, in which the changes to current VVC draft text are marked in yellow.

7.3.4.1 General tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| … |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| tile_group_fpel_mmvd_enabled_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 ) |  |
| max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| … |  |
| } |  |

7.3.6.7 Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_triangle_enabled_flag && tile_group_type = = B && ciip_flag[ x0 ][ y0 ] = = 0 && cbWidth * cbHeight >= 64 && MaxNumTriangleMergeCand >= 2 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| … |  |
| } |  |

7.4.5.1 General tile group header semantics

….

max_num_merge_cand_minus_max_num_triangle_cand specifies the maximum number of triangular merge mode candidates supported in the tile group subtracted from MaxNumMergeCand. The maximum number of triangular merge mode candidates, MaxNumTriangleMergeCand is derived as follows:

MaxNumTriangleMergeCand = MaxNumMergeCand − max_num_merge_cand_minus_max_num_triangle_cand

The value of MaxNumTriangleMergeCand shall be in the range of 0, and 2 to MaxNumMergeCand, inclusive. When max_num_merge_cand_minus_max_num_triangle_cand is not signaled, MaxNumTriangleMergeCand is infered as 0. When MaxNumTriangleMergeCand is 0, no triangle merge mode is allowed for the current tile group.

….

9.5.3.1 General

….

| Table 9-9 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| …. |  |  |  |
| merge_data( ) | …. |  |  |
|  | merge_triangle_flag[ ][ ] | FL | cMax = 1 |
|  | merge_triangle_idx[ ][ ] | FL | cMax = 1 |
|  | merge_triangle_idx0[ ][ ] | TR | cMax = 4 MaxNumTriangleMergeCand - 1, cRiceParam = 0 |
|  | merge_triangle_idx1[ ][ ] | TR | cMax = 3 MaxNumTriangleMergeCand - 2, cRiceParam = 0 |
|  | merge_idx[ ][ ] | TR | cMax = MaxNumMergeCand − 1, cRiceParam = 0 |
| .… |  |  |  |

….

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).