| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0852 |
| --- | --- |

| Title: | AHG12: Tile group order restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Hendry | Email: | rickard.sjoberg@ericsson.com fnu.hendry@huawei.com |
| Source: | Ericsson, Huawei |  |  |

_____________________________

# Abstract

This document proposes a modification to a constraint in the VVC regarding the tile group order. The proposal is to replace constraint 1 from the JVET-M1001-v7 draft by constraint 2 below:

Constraint 1: The tile groups of a picture shall be in increasing order of their tile_group_address values.

Constraint 2: The decoding order of the tile groups of a picture shall be such that the tile index of the first tile in each tile group of the picture are in increasing order

The authors are aware that decisions at the 14th meeting may change what flexibility is supported in VVC, and that those decisions may make this proposal obsolete.

# Introduction

In the JVET-M-Notes_dH.docx notes of the previous meeting, the following was agreed regarding rectangular tile groups:

--- start quote ---

JVET-M00853-v2 adds the support of rectangular tile groups in addition to the existing raster scan ordered tile groups, and enables extraction of MCTSs without changing VLC NAL units.

Decision: Adopted, with constraints as follows:

- the tiles in a tile group need to be in raster-scan order.

- the tile groups need to be in increasing address order (see below: dark orange, then light blue, then pale yellow, then dark blue, then light orange, …)

- The tile group shapes shall be such that each tile, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or previously decoded tile(s). In other words, the light yellow and light green tiles in the figure below are not allowed.

The following tile group picture was drawn for illustration and discussion purposes regarding the above constraints (an accidental homage to Piet Mondrian).

--- end quote ---

# Proposal

Based on off-line discussions at the Geneva meeting, this contribution proposes the following change to the restrictions of tile group order in the JVET-M1001-v7 spec to better reflect the decision in Marrakesh:

It is a requirement of bitstream conformance that the following constraints apply:

- The value of tile_group_address shall not be equal to the value of tile_group_address of any other coded tile group NAL unit of the same coded picture.

- The tile groups of a picture shall be in increasing order of their tile_group_address values.

- The decoding order of the tile groups of a picture shall be such that the tile index of the first tile in each tile group of the picture are in increasing order

- The shapes of the tile groups of a picture shall be such that each tile, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or consisting of boundaries of previously decoded tile(s).

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).