| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 Mar. 2019 | Document: JVET-N0857-v1 |
| --- | --- |

| Title: | AHG12: Signalling for tile and brick partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Hendry 10180 Telesis Court San Diego, CA, 92121, USA Maxim Sychev Building 1/7, Altufievskoe Shosse, Moscow 127106, Russia Robert Skupin Yago Sanchez Karsten Sühring Thomas Schierl (Fraunhofer HHI) | Tel: Email: | +1-908-803-3888 yekui.wang@huawei.com +1-858-375-8065 fnu.hendry@huawei.com +7-925-008-1124 sychev.maxim@huawei.com {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Huawei Technologies Co., Ltd., Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution is a harmonized design from JVET-N0111 and JVET-N0348 for signalling tile and brick partitioning. The proposed design takes into account the design for tile and brick partitioning discussed in Track B on March 23, 2019.

The proposed design allows further splitting one or more tiles that resulted from the current HEVC-style  tiling scheme into one or more bricks (i.e., division of a tile into brick(s) by one or more horizontal cuts). Brick splits can be either uniform or be signalled in units of CTUs within a tile.

Specification text based on JVET-M1001-v7 is attached to the document.

# Introduction

This contribution is a harmonized design from JVET-N0111 and JVET-N0348 for signalling tile and brick partitioning. The proposed design takes into account the design for tile and brick partitioning discussed in Track B on March 23, 2019. The following is an excerpt from the respective notes.

It was noted that the current VVC design has less partitioning flexibility than HEVC, since HEVC allows chopping tiles into slices. Even with this proposal, that would still be the case to some extent, since HEVC slice boundary lines are not required to cut all the way through the picture, where that constraint applies in this proposal.

It was noted that allowing any individual tile to be partitioned vertically at CTU row boundaries (whether independent or dependent) should not be any more difficult than supporting HEVC, since that is just a subset of what can be done with slices. Having a partitioning as shown below should not be any more difficult than HEVC’s tiles/slices scheme.

The order of the data in the bitstream would be required to be 1, 2, 3, 4, 5, as shown (completing all small rectangles within a large rectangle before moving to the next large rectangle). To pick an initial terminology, the large rectangles could be called tiles and the smaller ones called bricks.

The syntax of the partitioning of tiles into bricks could hypothetically use a stop flag after each CTU row in a tile or could be at the tile level, tile group level, picture level, or even a higher level.

For mapping these to NAL units, each NAL unit would contain a rectangular region. It was suggested that each NAL unit would contain a tile group or a part of a tile group.

We current have a raster scan mode of tile groups. In that mode, perhaps the restriction we imposed in HEVC should be similarly imposed – a NAL unit would need to be within a tile or contain multiple tiles. Another suggestion was to not allow tiles to be divided into multiple bricks in this mode – this was agreed as a starting point.

We want to enable rewriting without modifying data within the VCL NAL units. In the rectangular mode, the PPS would contain the entire map of tiles and bricks (not using a stop bit within the NAL unit). As in the current syntax, we start the NAL unit with an ID that would be mapped by the PPS (or higher) information to identify the specific brick that starts the NAL unit.

# Proposed hierarchical tiling scheme

The proposed design takes into account the above design goals allowing further splitting one or more tiles that resulted from the current HEVC-style  tiling scheme into one or more bricks (i.e., division of a tile into brick(s) by one or more horizontal cuts). Brick splits are signalled in the tililng syntax in the Picture Parameter Set and can be either uniform or be signalled in units of CTUs within a tile.

In summary, the proposed hierarchical tiling with bricks works as follows:

- Firstly, the current HEVC-style tiling scheme is applied to partition a picture into a set of tile columns and a set of tile rows. Each of the tiles resulted from this step is referred to as a first-level tile.

- For each tile, a flag is signalled to specify whether or not the tile is further divided into one or more brick rows. The new tiles resulted from the division are referred to as bricks.

As a result, a picture would contain zero or more tiles that were not further split and zero or more bricks. A tile that has been further split would only conceptually exist, would not physically exist, and would not be counted in the total number of tiles in the picture (i.e, tile that is not further split into bricks is considered as a single brick).

- For simplicity, it is constrained that a tile group shall contain only a number of complete tiles or only a consecutive sequence of complete bricks from the division of a tile. When a tile group contains one or more bricks, it shall be a rectangular tile group and it shall not violate the availability rule.

- The scan order is specified as follows (for simplicity, it is constrained that a tile group shall exclusively consist of either a number of complete tiles or a true subset of a tile (i.e., bricks) as shown in the example illustrated in Figure 3:

  - Tiles are ordered the same as in the current tiling scheme, i.e., tile raster scan of a picture.

  - The bricks of tile tileA precedes the bricks of tile that succeeds tileA.

  - For all the bricks, their order is the brick raster scan within a tile.

  - As before, CTUs within the bricks are in the order of CTU tile scan order. Since brick splits are horizontally only, they do not affect scan order beyond the tile boundaries.

Figure 3 – Example of scan order in tile and brick

The specification text, relative to JVET-M1001-v7, is provided as an attachment to this document.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. and Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).