| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0859 |
| --- | --- |

| Title: | Sub-picture design principles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce | Tel: Email: | jill.boyce@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

A set of design principles are proposed for a sub-picture design:

- SPS signalling of number of sub-pictures and location and size of each sub-picture

- Sub-picture ID in fixed length field for each tile group, at beginning of TGH, not following any variable length coded syntax elements

- PTL signalling for sub-pictures and sub-picture sets in an SEI message

- Simple rewriting process for sub-picture merging and extraction, including rewriting of SPS and TGH, with rewritten SPS PTL from SEI message provided PTL values

- No normative conformance points for sub-pictures or sub-picture sets, only for pictures created after rewriting process

- Isolation of sub-picture concept to rewriting process and SPS and TGH, so that the rest of the decoding process below TGH is unaware of sub-pictures

# Discussion

The set of design principles listed in the abstract are proposed to be used when designing a solution for sub-pictures.

Specification text implementing the design principles can be provided shortly if requested by the group.

It is asserted that following such design principles will accomplish two key goals:

- Satisfying many of the MPEG systems requests for independently decodable and conforming “objects”

- Friendly for implementation by hardware decoders, so that sub-pictures can be supported in a Main profile of VVC

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).