| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0866-v2 |
| --- | --- |

| Title: | Non-CE6: Combined Test of JVET-N0172/JVET-N0375/JVET-N0419/JVET-N0420 on Unification of Implicit Transform Selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao, Semih Esenlik, Biao Wang, Anand Meher Kotra, Jianle Chen Hilmi E. Egilmez, Adarsh K. Ramasubramonian, Amir Said, Nan Hu, Vadim Seregin, Geert Van-der Auwera, Marta Karczewicz Sung-Chang Lim, Jungwon Kang, Jinho Lee, Hahyun Lee, Hui Yong Kim | Tel: Email: | han.gao@huawei.com hegilmez@qti.qualcomm.com sclim@etri.re.kr |
| Source: | Huawei Technologies Co.,Ltd. Qualcomm Incorporated Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

# Abstract

In VVC Draft 4.0, two separate implicit transform selection methods based on intra mode and block-shape are introduced for ISP and implicit MTS, respectively. This joint contribution describes the unified design adopted in the 14th JVET meeting, where the components of the unified design are independently proposed and tested in JVET-N0172, JVET-N0375, JVET-N0419 and JVET-N0420:

- The implicit MTS part of the adoption is proposed and tested in JVET-N0172 and JVET-N0419.

- The transform derivation for ISP is proposed and tested in JVET-N0172, JVET-N0375 and JVET-N0420.

The adopted unified method provides -0.01%, 0.00%, and -0.01% coding efficiency over VTM4.0 for AI, RA, and LDB cases under CTC, respectively. As compared to the current implicit MTS design in VTM-4.0, -0.11%, -0.02%, and 0.02% are achieved for AI, RA, and LDB cases, respectively.

# Introduction

In VVC Draft 4.0 [1], if MTS is enabled in the high level syntax, the MTS transform cores could be implicitly selected for intra block under the condition:

- If the current block is using ISP mode

- Otherwise, if the current block is intra block and both intra and inter explicit MTS is disable in the sequence level (intra shape adaptive)

The ISP implicit transform core selection is based on the intra mode of the current block, intra shape adaptive implicit transform core selection is based on the block shape.

# Problem Statement

Both ISP and intra shape-adaptive is applied only in intra block, and both tools are only using DST7 and DCT2 transform cores with same block size restriction, which means their implicit transform core selections are easily unified.

In this contribution, the proposed method proposes to unify the transform selection in ISP and implicit MTS by removing the intra-mode and block-shape dependencies in the current VTM-4.0.

# Proposed method

## Method Description

In the proposed method, if current block is ISP mode or if the current block is intra block and both intra and inter explicit MTS is on, then only DST7 is used for both horizontal and vertical transform cores. The size restriction is unchanged.

In this method, the dependency between ISP mode implicit transform core selection and intra mode is removed. The shape adaptive implicit transform core selection and the ISP implicit transform core selection are simplified. The proposed method provides coding gain without introducing additional worst case.

## Simulation Results

The simulations are on top of VTM4.0. Table 1 shows the CTC condition (MTS explicit intra on, and MTS explicit inter off) and Table 2 shows both MTS inter and MTS intra off cases with the proposed method.

The coding efficiency results in the following table are identical to the results reported in JVET-N0172 (Table 1) and JVET-N0420 (Table 2). The same method is tested over VTM-4.0.1 in JVET-N0375 (Table IV). In these tests, encoding and decoding execution times are within the measurement error range.

Table 1: Results of the proposed method over VTM-4.0 under CTC.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | 0.00% | 100% | 100% |
| Class A2 | -0.03% | 0.07% | -0.08% | 100% | 102% |
| Class B | -0.01% | 0.00% | 0.05% | 100% | 98% |
| Class C | 0.04% | -0.11% | -0.04% | 100% | 101% |
| Class E | -0.06% | -0.13% | -0.12% | 100% | 96% |
| Overall | -0.01% | -0.04% | -0.03% | 100% | 99% |
| Class D | 0.01% | -0.03% | -0.19% | 100% | 101% |
| Class F | 0.05% | -0.05% | -0.19% | 100% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | 0.04% | 100% | 100% |
| Class A2 | -0.02% | 0.04% | -0.02% | 100% | 99% |
| Class B | 0.01% | -0.07% | -0.23% | 101% | 101% |
| Class C | 0.01% | -0.04% | -0.11% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.10% | 100% | 100% |
| Class D | 0.00% | -0.36% | -0.16% | 101% | 101% |
| Class F | 0.04% | -0.17% | -0.17% | 101% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.32% | -0.76% | 100% | 99% |
| Class C | -0.05% | 0.27% | -0.10% | 101% | 102% |
| Class E | 0.06% | -0.81% | 1.53% | 102% | 105% |
| Overall | -0.01% | -0.25% | 0.04% | 101% | 102% |
| Class D | 0.02% | 0.28% | 0.39% | 101% | 98% |
| Class F | -0.05% | -0.38% | 0.32% | 100% | 102% |

The coding efficiency results in the following table are identical to the results reported in JVET-N0172 (Table 2) and JVET-N0419 (Table 1). In these tests, encoding and decoding execution times are within the measurement error range.

Table 2: Results of the proposed method over VTM-4.0 with MTS=0 and ImplicitMTS=1.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 with MTS=0 and ImplicitMTS=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.40% | -0.33% | 101% | 96% |
| Class A2 | -0.12% | -0.33% | -0.42% | 101% | 98% |
| Class B | -0.12% | -0.40% | -0.36% | 101% | 96% |
| Class C | 0.05% | -0.37% | -0.16% | 101% | 95% |
| Class E | -0.18% | -0.43% | -0.53% | 101% | 99% |
| Overall | -0.11% | -0.39% | -0.35% | 101% | 96% |
| Class D | 0.04% | -0.20% | -0.12% | 101% | 94% |
| Class F | -0.02% | -0.30% | -0.10% | 100% | 96% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-4.0with MTS=0 and ImplicitMTS=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.31% | -0.21% | 98% | 97% |
| Class A2 | 0.03% | -0.04% | -0.06% | 99% | 100% |
| Class B | -0.06% | -0.18% | 0.01% | 101% | 100% |
| Class C | 0.00% | 0.24% | -0.09% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.06% | -0.07% | 100% | 99% |
| Class D | -0.05% | 0.05% | -0.26% | 100% | 99% |
| Class F | 0.00% | -0.16% | -0.35% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 with MTS=0 and ImplicitMTS=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | 0.05% | -0.33% | 99% | 101% |
| Class C | -0.07% | 0.07% | -0.12% | 100% | 100% |
| Class E | 0.22% | -1.37% | 0.01% | 101% | 98% |
| Overall | 0.02% | -0.30% | -0.18% | 100% | 100% |
| Class D | -0.02% | 0.14% | -0.05% | 100% | 99% |
| Class F | -0.15% | 0.03% | 0.13% | 100% | 99% |

## Draft Specification Changes

The draft text changes based on [1] of the proposed method is highlighted as follow:

#### 8.7.4.1 General

…

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT or (sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are equal to 0), trTypeHor and trTypeVer are derived as followsspecified in Table 8-15 depending on intraPredMode.

  - trTypeHor = ( nTbW >= 4  &&  nTbW <= 16  ) ? 1 : 0	(8-1028)

  - trTypeVer = ( nTbH >= 4  &&  nTbH <= 16  ) ? 1 : 0	(8-1029)

- Otherwise, if cu_sbt_flag is equal to 1, trTypeHor and trTypeVer are specified in Table 8-14 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

- Otherwise (sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are equal to 0), trTypeHor and trTypeVer are derived as follows:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16  &&  nTbW <= nTbH ) ? 1 : 0	(8-1028)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16  &&  nTbH <= nTbW ) ? 1 : 0	(8-1029)

…

Table 8-15 – Specification of trTypeHor and trTypeVer depending on predModeIntra

| predModeIntra | trTypeHor | trTypeVer |
| --- | --- | --- |
| INTRA_PLANAR, INTRA_ANGULAR31, INTRA_ANGULAR32, INTRA_ANGULAR34, INTRA_ANGULAR36, INTRA_ANGULAR37 | ( nTbW >= 4 && nTbW <= 16 ) ? 1 : 0 | ( nTbH >= 4 && nTbH <= 16 ) ? 1 : 0 |
| INTRA_ANGULAR33, INTRA_ANGULAR35 | 0 | 0 |
| INTRA_ANGULAR2, INTRA_ANGULAR4,…,INTRA_ANGULAR28, INTRA_ANGULAR30, INTRA_ANGULAR39, INTRA_ANGULAR41,…,INTRA_ANGULAR63, INTRA_ANGULAR65 | ( nTbW >= 4 && nTbW <= 16 ) ? 1 : 0 | 0 |
| INTRA_ANGULAR3, INTRA_ANGULAR5,…, INTRA_ANGULAR27, INTRA_ANGULAR29, INTRA_ANGULAR38, INTRA_ANGULAR40,…,INTRA_ANGULAR64, INTRA_ANGULAR66 | 0 | ( nTbH >= 4 && nTbH <= 16 ) ? 1 : 0 |

# Conclusion

The proposed method is unifying the ISP and intra shape adaptive implicit transform core selection providing coding gain without introducing additional worst case, which is recommended to include into the next version of VVC draft.

# References

- “Versatile Video Coding (Draft 4)”, JVET-M1001, B. Bross, J. Chen, S. Liu

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Electronics and Telecommunications Research Institute (ETRI) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).