| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0867-v1 |
| --- | --- |

| Title: | Harmonized proposal on HRD parameters for temporal scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon (Panasonic) Monzastr. 4c 63225 Langen, Germany Robert Skupin Yago Sanchez (HHI) Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49-6103-766240 virginie.drugeon@eu.panasonic.com {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Panasonic, Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document presents a harmonized proposal of JVET-N0062 and JVET-N0351 for signalling of HRD parameters for temporal scalable bitstreams.

It is asserted that the HEVC solution for specifying and signalling the HRD parameter for temporal scalability (non-scalable nested SEI messages per AU for each temporal layer) comes at an undesirable signalling and implementation burden. Furthermore, it led external organizations such as DVB to avoid the specified solution and impose further constraints on bitstream structure that are detrimental to coding efficiency. Detailed problem descriptions can be found in documents JVET-N0062 and JVET-N0351.

The proposal is to specify and signal HRD parameters in temporal scalable VVC bitstreams for each AU in a single picture timing SEI message, without the need for non-scalable nesting SEI messages.

# Introduction

This document presents a harmonized proposal of JVET-N0062 and JVET-N0351 for signalling of HRD parameters for temporal scalable bitstreams.

It is asserted that the HEVC solution for specifying and signalling the HRD parameter for temporal scalability (non-scalable nested SEI messages per AU for each temporal layer) comes at an undesirable signalling and implementation burden. Furthermore, it led external organizations such as DVB to avoid the specified solution and impose further constraints on bitstream structure that are detrimental to coding efficiency. Detailed problem descriptions can be found in documents JVET-N0062 and JVET-N0351.

The proposal is to specify and signal HRD parameters in temporal scalable VVC bitstreams for each AU in a single picture timing SEI message, without the need for non-scalable nesting SEI messages.

# Proposal

## Buffering Period SEI message

The modifications suggested below are based on the syntax and semantics of buffering period SEI message specified in Annex D of HEVC.

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| bp_seq_parameter_set_id | ue(v) |
| concatenation_flag | u(1) |
| au_cpb_removal_delay_delta_minus1 | u(v) |
| … |  |
| au_cpb_removal_delay_deltas_present_flag | u(1) |
| if( au_cpb_removal_deltas_present_flag ) { |  |
| num_cpb_removal_delay_deltas_minus1 | ue(v) |
| for( i = 0; i <= num_cpb_removal_delays_deltas _minus1 ; i++ ) |  |
| au_cpb_removal_delay_delta[ i ] | u(v) |
| } |  |
| if( NalHrdBpPresentFlag ) { |  |
| for( i = 0; i < CpbCnt; i++ ) { |  |
| nal_initial_cpb_removal_delay[ i ] | u(v) |
| nal_initial_cpb_removal_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( VclHrdBpPresentFlag ) { |  |
| for( i = 0; i < CpbCnt; i++ ) { |  |
| vcl_initial_cpb_removal_delay[ i ] | u(v) |
| vcl_initial_cpb_removal_offset[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

au_cpb_removal_delay_deltas_present_flag specifies the presence of CPB removal delay deltas. When au_cpb_removal_delay_deltas_present_flag is equal to 1, the buffering period SEI message contains CPB removal delay deltas. When au_cpb_removal_delay_deltas_present_flag is equal to 0, no CPB removal delay deltas are present in the buffering period SEI message.

num_cpb_removal_delay_deltas_minus1 plus 1 specifies the number of syntax element au_cpb_removal_delay_delta[ i ] following in the buffering period SEI message. The value of num_cpb_removal_offsets_minus1 shall be in the range of 0 to 15, inclusive.

au_cpb_removal_delay_delta[ i ] specifies the CPB removal delay delta of the i-th entry in the CPB removal delay deltas list.

## Picture Timing SEI message

The modifications suggested below are based on the syntax and semantics of the picture timing SEI message specified in Annex D of HEVC.

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| if( frame_field_info_present_flag ) { |  |
| pic_struct | u(4) |
| source_scan_type | u(2) |
| duplicate_flag | u(1) |
| } |  |
| if( CpbDpbDelaysPresentFlag ) { |  |
| au_cpb_removal_delay_minus1[ maxNumSubLayersMinus1 ] | u(v) |
| for( i = nuh_temporal_id_plus1 – 1; i < maxNumSubLayersMinus1; i++ ) { |  |
| sub_layer_delays_present_flag[ i ] | u(1) |
| if( sub_layer_delays_present_flag[ i ]) { |  |
| au_cpb_removal_delay_delta_enabled[ i ] | u(1) |
| if ( au_cpb_removal_delay_delta_enabled[ i ] ) |  |
| au_cpb_removal_delay_delta_idx[ i ] | u(v) |
| else |  |
| au_cpb_removal_delay_minus1[ i ] | u(v) |
| } |  |
| } |  |
| pic_dpb_output_delay | u(v) |
| } |  |
| } |  |

au_cpb_removal_delay_minus1[ i ] specifies the number of clock ticks between the CPB removal time of the access unit associated with the picture timing SEI message and the preceding access unit in decoding order that contained a buffering period SEI message that applies to a OpTid equal to i.

sub_layer_delays_present_flag[ i ] equal to 1 specifies that one of au_cpb_removal_delay_minus1[ i ] or au_cpb_removal_delay_delta_idx[ i ] is present for the the sub-layer with TemporalId equal to i. sub_layer_delays_present_flag[ i ] equal to 0 specifies that neither au_cpb_removal_delay_minus1[ i ] nor au_cpb_removal_delay_delta_idx[ i ] are present for the sub-layer with TemporalId equal to i. If sub_layer_delays_present_flag[ i ] is not present, it is infered to be equal to 0.

au_cpb_removal_delay_delta_enabled[ i ] equal to 1 specifies the presence of the au_cpb_removal_delay_delta_idx[ i ] in the picture timing SEI message. If au_cpb_removal_delay_delta_enabled[ i ] is not present, it is infered to be equal to 0.

au_cpb_removal_delay_delta_idx[ i ] specifies the index of the au_cpb_removal_delay_delta list element that applies to a OpTid equal to i. If au_cpb_removal_delay_delta_idx[ i ] is not present, it is infered to be equal to 0.

pic_dpb_output_delay is used to compute the DPB output time of the picture. It specifies how many clock ticks to wait after removal of an access unit from the CPB before the decoded picture is output from the DPB.

NOTE 8 – A picture is not removed from the DPB at its output time when it is still marked as "used for short-term reference" or "used for long-term reference".

The length of the syntax element pic_dpb_output_delay is given in bits by dpb_output_delay_length_minus1 + 1.

The output time derived from the pic_dpb_output_delay of any picture that is output from an output timing conforming decoder shall precede the output time derived from the pic_dpb_output_delay of all pictures in any subsequent CVS in decoding order.

The picture output order established by the values of this syntax element shall be the same order as established by the values of PicOrderCntVal.

For pictures that are not output by the "bumping" process because they precede, in decoding order, an IRAP picture with NoRaslOutputFlag equal to 1 that has no_output_of_prior_pics_flag equal to 1 or inferred to be equal to 1, the output times derived from pic_dpb_output_delay shall be increasing with increasing value of PicOrderCntVal relative to all pictures within the same CVS.

The following text changes are relative to the on the HRD description proposed in JVET-N0350.

–	When access unit n is not the first access unit of a buffering period, the nominal removal time of the access unit n from the CPB is specified by:

AuNominalRemovalTime[ n ] = AuNominalRemovalTime[ firstPicInCurrBuffPeriod ] +
		ClockTick * AuCpbRemovalDelayVal	(C-11)

where AuNominalRemovalTime[ firstPicInCurrBuffPeriod ] is the nominal removal time of the first access unit of the current buffering period and AuCpbRemovalDelayVal is the value of AuCpbRemovalDelayVal derived according to au_cpb_removal_delay_minus1 in the picture timing SEI message, selected as specified in clause C.1, associated with access unit n derived as follows:

–	If sub_layer_delays_present_flag[ OpTid ] is equal to 0, AuCpbRemovalDelayVal is set equal to the value of au_cpb_removal_delay_minus1[ maxNumSubLayersMinus1 ] plus 1 in the picture timing SEI message, selected as specified in clause C.1, associated with access unit m and picDpbOutputDelta is set equal to 0.

–	Otherwise, if au_cpb_removal_delay_delta_enabled[ OpTid ] is equal to 0, AuCpbRemovalDelayVal is set equal to the value of au_cpb_removal_delay_minus1[ OpTid ] plus 1 in the picture timing SEI message, selected as specified in clause C.1, associated with access unit m and picDpbOutputDelta is set equal to au_cpb_removal_delay_minus1[ OpTid ] – au_cpb_removal_delay_minus1[ maxNumSubLayersMinus1 ].

–	Otherwise, AuCpbRemovalDelayVal is set equal to the value of au_cpb_removal_delay_minus1[ maxNumSubLayersMinus1 ] plus 1 in the picture timing SEI message + au_cpb_removal_delay_delta[ au_cpb_removal_delay_delta_idx[ OpTid ] in the buffering period SEI message, selected as specified in clause C.1, associated with access unit m and picDpbOutputDelta is set equal to au_cpb_removal_delay_delta[ au_cpb_removal_delay_delta_idx[ OpTid ] ].

DpbOutputTime[ n ] = AuCpbRemovalTime[ n ] + ClockTick * (picDpbOutputDelay – picDpbOutputDelta)

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).