| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0870 |
| --- | --- |

| Title: | BoG report on 360 video |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce | Tel: Email: | jill.boyce@intel.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

BoG met 26 Mar 2019 from 11:30 to 12:45.

The BoG recommends;

- Adopt semantic restrictions to reference wraparound from JVET-N0070

  - limit the range of sps_ref_wraparound_offset_minus1

  - define an inference value when sps_ref_wraparound_enabled_flag not present

  - replace the wrapping modulus operation “ClipH” with a conditional offset

The BoG has identified for further study:

- Define an SEI message for a generalized cube map format, with parameterized model to define projection mapping which can represent several of the cube based formats supported in 360Lib, and add corresponding support in 360Lib for the general parameterized model

It is also suggested to copy the HEVC ERP SEI message.

# Contribution review

### 4.4.2 360° video coding (6)

JVET-N0070 AHG16/AHG8: Proposed Cleanup for Reference Wraparound [B. Heng, M. Zhou, W. Wan (Broadcom)]

The following document discusses potential issues with the motion compensation reference wrapping tool. It is asserted that the current specification of this tool has some unintended corner cases that impose undue complexity on decoders.

This contribution proposes to limit the range of sps_ref_wraparound_offset_minus1 as follows:

sps_ref_wraparound_offset_minus1 shall be in the range of (CtbSizeY/MinCbSizeY) + 1 to ( pic_width_in_luma_samples / MinCbSizeY ) − 1, inclusive.

And if (CtbSizeY/MinCbSizeY + 1) > ( pic_width_in_luma_samples / MinCbSizeY  − 1), sps_ref_wraparound_enabled_flag is not signalled in SPS, it is inferred to be equal to 0.

In addition, this contribution proposes to replace the wrapping modulus operation “ClipH” with a conditional offset:

ClipH(o, W, x) = x  + o;x  < 0x -o;x  > W-1x;otherwise

And add a clipping operation:

xInti = Clip3( 0, picW − 1, ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ))

so that any reference location that would have wrapped around more than once will get clipped within the picture boundary instead.

The goal for this contribution is to not restrict the intended use cases, but to limit hardware complexit.

BoG recommends to adopt all 3 aspects of the contribution.

JVET-N0219 Coding of 360° video in HEC using different numbers of tiles per face [J. Sauer, M. Bläser (RWTH Aachen)]

This document describes the impact of tiles on the coding of 360° video. For the anchor VTM 4.0.1 with 360Lib 9.0 was used and configured to use a single tile for the whole frame / all faces. Different number of tiles for a single cube face were used for comparison.

The BoG on coding of 360° video recommends disabling of loop filters at face boundaries. This can be achieved by configuring each face to be in a different tile group and setting loop_filter_across_tile_groups_enabled_flag to zero. Another approach is to implement a high level signaling of sample rows and columns at which loop filters shall be disabled.

The BD-Rate changes for different numbers of tiles per face, reported in this document, may be helpful for the ongoing discussion on this.

The adoption of JVET-N0438 should address this use case.

JVET-N0232 [AHG8] Coding performance of 360° Videos in PHEC format with different face sizes [A. DSouza, C. Pujara, R. Gadde, W. Choi, K. Choi, K.P. Choi (Samsung)] [late]

Omnidirectional videos are one of the categories of videos considered for evaluating the performance of Versatile Video Coding. Cubemap based formats are shown to perform better than other projection formats for encoding 360° Videos. In this contribution, the coding performance of PHEC is computed at different face sizes. It is observed that the coding efficiency improves as face size increases. It is also noted that the bitrate and decoding complexity also increase linearly as face size increases.

Recommends changing the CTC to adjust the face size of from 1280 to 1536.

It was noted that such a change would mean a meaningful difference in pixel rate vs. ERP, and that would bump up to another PTL level. It would also increase the encoder and decoder runtime by ~25%.

No change to the CTC at this time. Contributions are always free to provide additional results in addition to the CTC.

JVET-N0233 [AHG8] Alternate face arrangement for PHEC [A. DSouza, C. Pujara, R. Gadde, W. Choi, K. Choi, K.P. Choi (Samsung)] [late]

Currently AHG6 on 360° video coding tools uses PHEC as an anchor for Common Test Conditions (CTC). The layout of the faces is 3x2 layout with the top and bottom rows continuous and discontinuous boundary at the middle of the frame. In this contribution, we propose an alternate face arrangement of PHEC. It is shown that the alternate face arrangement shows gains for some of the sequences as compared to the current anchor layout.

# Discussion

It is noted that the VVC design has no current mechanism for signaling projection formats. HEVC has ERP and CMP SEI messages. For the HEVC CMP SEI message, in combination with the region-wise packing SEI, very flexible mapping can be done, perhaps too flexible given the overhead for such flexibility.

There is a config file provided with the 360Lib software for a single cube face layout that corresponds to the CTC, but the software is flexible enough to support other layouts via config file options.

It is noted that there are no coding tools in the design now that require knowledge within the codec of projection format characteristics.

The SEI message framework will be adopted to the VVC design with other adoptions at this meeting.

Further study encouraged on definition of an SEI message with the following features:

- cube map layout indication that could apply for any cube-based projection format, including padding

- projection mapping function with parameters for one or more projection formats supported in 360Lib

  - Group would need to consider which ones to include

It would also be useful to have support for ERP. For ERP support, could either copy/paste the HEVC SEI message into VVC or could make the SEI message described above be general purpose enough to also support ERP.

Whatever is designed for VVC could be back-ported to HEVC if desired.

Study overhead comparison of use of region-wise packing vs. a more targeted signaling with less flexibility.

Should the SEI have user-defined parameters for the projection mapping equations?

Group suggests to define two SEI messages:

- ERP

- General cube map

Could we define parameters and equations that could be used to represent more than one projection format supported in 360Lib. EAC would be interesting to support because of industry usage. HEC is also interesting as it is our CTC projection format.

The HEVC CMP SEI has equations that map coded picture samples to 3D space for a conventional cube map. The new SEI should follow a similar functionality but for additional projection formats based on a few parameters.

Could have a table that provides parameters corresponding to formats supported in 360Lib.

Would want to see 360Lib software that supported the parameterized mapping.

Continue 360 video AHG to the next meeting. SEI message contribution titles should indicate AHG numbers for both the 360 video AHG and the HLS AHG.