| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 Mar. 2019 | Document: JVET-N1021-v1 |
| --- | --- |

| Title: | Description of Core Experiment 1 (CE1): CABAC Initialization |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Frank Bossen Jie Dong Heiner Kirchhoffer | Email: | fbossen@sharplabs.com jiedong@qti.qualcomm.com heiner.kirchhoffer@ hhi.fraunhofer.de |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This Core Experiment (CE) covers one aspect of the arithmetic coding engine of VVC: the method for initializing the state of each context.

# Introduction

Several initialization methods were proposed at the 14th JVET meeting to remove a mapping table used in draft 4 of VVC. These methods, including possible variants and combinations, are tested using a common training algorithm.

# Participant list

| Affiliation | Participants | P | C |
| --- | --- | --- | --- |
| HHI | Heiner Kirchhoffer | X | X |
| Qualcomm | Jie Dong Amir Said | X | X |
| Sharp | Frank Bossen | X | X |

# Description of Experiments

## Linear, 6 bits

### JVET-N0301

The allowed values for variable initValue are: 0≤initValue≤63. The initialization procedure is as follows:

m = ( ( initValue >> 3 )  * 3 ) - 12

n  = ( ( ( initValue  &  7 ) * 5 ) << 5 ) + 32

preCtxState = m * qp + n

pStateIdx0 = Clip3( 16, 1008, preCtxState )

pStateIdx1 = Clip3( 1, 16383, preCtxState << 4 )

### JVET-N0425

The detailed operations of initialization process are showed as below, where slopeIdx and offsetIdx are integers from 0 to 7.

m = slopeIdx × 10 – 45

n = (offsetIdx << 4) + 14

preCtxState = Clip3(1, 127, ((m × (QP – 32)) >> 4) + n)

pStateIdx0 = preCtxState << 3

pStateIdx1 = preCtxState << 7

## Linear, 8 bits

### JVET-N0425

The detailed operations of initialization process are showed as below, where slopeIdx and offsetIdx are integers from 0 to 15.

m = slopeIdx × 5 – 45

n = (offsetIdx << 3) + 7

preCtxState = Clip3(1, 127, ((m × (QP – 32)) >> 4) + n)

pStateIdx0 = preCtxState << 3

pStateIdx1 = preCtxState << 7

## Logarithmic, 8 bits

### JVET-N0381

The initialization process is as follows:

From the 8 bit table entry initValue, two 4-bit variables slopeIdx and offsetIdx are derived as follows:

slopeIdx = initValue  >>  4

offsetIdx = initValue & 15

The variables m and n, used in the initialization of context variables, are derived from slopeIdx and offsetIdx as follows:

m = slopeIdx - 8

n = ( offsetIdx  - 8 ) * 25

The variable preCtxStateIdx is derived from TileGroupQpY, which is derived as provided above,  as follows:

preCtxStateIdx = m * Clip3( 0, 51, TileGroupQpY) + n

mps = preCtxStateIdx < 0  ? 0 : 1

preCtxStateIdx = 3 * min(255, mps ? preCtxStateIdx  : –preCtxStateIdx-1)

state = ((255 – (   preCtxStateIdx & 127)) * 32) >> (preCtxStateIdx    >> 7)

state = mps ? 16383 – p : p

The values of pStateL and pStateH are derived as follows:

pStateL = state >> 4

pStateH = state

A variant using a 6-bit representation of initValue may also be tested.

# Test conditions

For all experiments, the common test conditions as specified in JVET-N1010 are used, with modifications as noted below.

## Training method

For training, bitstreams obtained with VTM 5.0 using common test conditions are used. This includes:

- Classes A, B, C, D, E, F

- All intra, random access, low delay P, low delay B

- QPs 22, 27, 32, 37

For random access, a single bitstream per data point should be used (segments are concatenated to avoid duplication of I frames).

Brute force search is used to determine for each context model which combination of initialization values and shiftIdx (window size) minimizes a rate criterion on the set of bitstreams described above.

### Some definitions

The set of candidate initial values shall be given as X and it depends on the initialization method. For example, the set of values may be {0..255} as in VTM 5.0.

The set of training bitstreams shall be denoted B, the set of candidate values for shiftIdx shall be denoted S, and the set of allowed values for initType shall be denoted T=0, 1, 2.

For each context c, each initType t, each candidate initial value x, each candidate shiftIdx s, and each bitstream b, a rate rc,t,x,s,b is determined. The rate r is estimated using the ProbModelTables::m_binFracBits table defined in VTM 5.0 and it corresponds to the number of bits that are produced from encoding the associated bins.

The variable Lb shall denote the bitstream size of bitstream b in bits. The size of the bitstream is calculated using the file size (after removal of SEI messages, if any).

For each context c, each initType t, each candidate initial value x, each candidate shiftIdx s, and each bitstream b, a normalized rate

dc,t,x,s,b=rc,t,x,s,bL(b)

is determined.

### Optimization method for init values and shiftIdx

For each context c, each initType t, each candidate shiftIdx s, the best candidate initial value

xminc,t,s=arg minx∈Xb∈Bdc,t,x,s,b

is determined.

For each context c, each candidate shiftIdx s, and all associated best candidate initial values xminc,t,s,  the normalized rates of all initTypes and all bitstreams are summed up, yielding a sum

d2c,s=t∈Tb∈Bdc,t,xminc,t,s,s,b.

For each context c, the best shiftIdx

sminc=arg mins∈Sd2c,s

is determined. The output of the optimization is the best shiftIdx sminc and the associated best candidate initial values xminc,t,sminc.

## Implementation of the training method

The training method is implemented in three steps as described in the following. Software for each step is made available in the official CE1 software repository.

### Extraction of bins

For each bitstream, a modified VTM decoder writes all bins of all context models into a single uncompressed binary file (denoted bin file) with the following properties:

- Bins associated with a particular frame occur in an uninterrupted sequence.

- Bins associated with a particular frame and with a particular context model occur in an uninterrupted sequence.

Furthermore, the modified decoder creates for each bin file an accompanying xml file that contains information for locating bins in the bin file and L(b).

Fraunhofer HHI volunteered to provide such a modified VTM-5.0 decoder by xxxx

### Computation of bit rates

A C++ tool reads a bin file and associated xml file in order to compute bit rates resulting from compressing the bins of a selected context model for all frames. This corresponds to computing all rc,t,x,s,b.

Fraunhofer HHI volunteered to provide source code for this tool by xxxx. Proponents then must implement their proposed context initialization method in this tool.

### Selection of best parameters

A second tool determines the best parameters sminc and xminc,t,sminc based on the values rc,t,x,s,b derived by the rate computation tool (see previous section) and L(b) ..

XXX volunteered to provide this tool by xxxx.

## Compression Efficiency

To obtain objective rate-distortion measurements, four-point BD-rate and BD-PSNR metrics shall be used to evaluate coding efficiency. Rates and PSNR values obtained for all test points shall be provided as well. Compression efficiency, the encoder, and decoder run-time results shall also be reported with the common spreadsheet template that is being used for most JVET contributions.

Report BD rate for QP values

- 2, 7, 12, 17 (RA condition only)

- 22, 27, 32, 37

- 42, 47, 52, 57 (RA condition only)

## Runtime measurements

The overall runtimes for executing the tasks in sections 4.2.1, 4.2.2, and 4.2.3 are reported separately for each CE test.

# Timeline

General timeline for core experiments (as agreed by JVET).

| T1 | April 17, 2019 | - Final CE description uploaded |
| --- | --- | --- |
| T2 | VTM 5.0 release + 2 weeks | - Proponents complete initial integration of tools (in individual branch) - Cross checkers start study of tools / software |
| T3 | June 5, 2019 | - Proponents freeze changes to software branches - The names of the cross checkers are finalized - Cross checkers start final cross checks |
| T4 | June 18, 2019 | - Proponents provide full test results - CE documents are uploaded |

Test results and cross check results are reported to CE coordinators (for inclusion into CE report).

Proponent

Cross-Checker