| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-M1022-v1 |
| --- | --- |

| Title: | Description of Core Experiment 2 (CE2): luma-chroma dependency reduction |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Output document |  |  |  |
| Purpose: | Core experiment description |  |  |  |
| Author(s) or Contact(s): | Jie Chen Edouard François Peng Yin | Email: | jiechen.cj@alibaba-inc.com Edouard.Francois@technicolor.com pyin@dolby.com |
| Source: | CE2 coordinators |  |  |  |

_____________________________

- Abstract

This document describes Core Experiment 2 (CE2) on luma-chroma dependency reduction. The experiments are conducted after the 14th JVET meeting and the results, including cross-checks, will be reported to the 15th JVET meeting.

Introduction

As stated in the AHG16 report on Implementation studies (JVET-N0016), the support of Separate luma/chroma partitioning tree potentially introduces a long latency for chroma intra prediction when CCLM is on. In the worst case, the chroma intra prediction cannot be kicked off until the whole 64x64 luma block is reconstructed. A similar problem arises when using the chroma residual scaling (CRS) from LMDS, as in the current LMCS method, the chroma scaling process is performed at the block level, and the scaling factor is determined based on the average value of the corresponding luma prediction block.

This CE aims at investigating solutions proposed in the 14th JVET meeting to address those issues.

In CE2, tests are organized in the following sub-CE categories:

- CE2-1: Chroma residual scaling luma-chroma dependency reduction – Single-tree case (8 tests)

- CE2-2: Chroma residual scaling luma-chroma dependency reduction – Dual-tree case (7 tests)

- CE2-3: CCLM luma-chroma dependency reduction in Dual-tree case (2 tests)

Test conditions and guidelines

CTC conditions as agreed at the 14th JVET meeting (JVET-N1010) will be used for the evaluation of objective performance. The anchor is the VTM5.0.

Both the ‘All Intra’, ‘Low Delay’ and ‘Random Access’ test configurations shall be used and providing the results for each of these configurations is mandatory. Tools shall be implemented into the VTM version 5.0 software.

Proposals will primarily be compared with respect to objective compression efficiency while complexity will also be assessed.

BD-rates, encoding and decoding time comparisons, shall be reported by all proponents.

Source codes corresponding with the test descriptions contained in this document shall be uploaded to the CE2 GitLab project according to the agreed timeline

Timeline

| T1 | April 17. 2019 | CE test description finalized (3 weeks after the meeting) |
| --- | --- | --- |
| T2 | 2 weeks after 5.0 release (May 13. 2019 ) | Initial software of CE tests released; initial study by cross-checkers begins (2 weeks after 5.0 release) |
| T3 | June 5. 2019 | Software of CE tests is finalized; final cross-check begins (4 weeks before next meeting) |
| T4 | June 18. 2019 | Document upload: CE contribution document including spec txt. Full test results, cross-check reports (1 week before regular doc deadline) |

CE2-1: Chroma residual scaling luma-chroma dependency reduction – Single-tree case

Contribution documents

JVET-N0113 AHG16: Subblock-based chroma residual scaling [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

To reduce the delay of chroma sample reconstruction, the chroma residual inverse scale value is derived using the top-left 16×16 luma prediction samples corresponding to the chroma TB. The derived chroma residual inverse scale value is applied to the entire chroma TB. The worst-case delay of the chroma sample reconstruction is reduced to 16x16 luma prediction samples.

JVET-N0220 Simplification of Reshaper Implementation [T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen (Dolby)]

This contribution proposed to use the first (upper-left) sample value of the corresponding luma prediction block to replace the average value of the block as in VTM4.0. Hence, the chroma residue scale factor for the chroma TU can be computed immediately after the first corresponding luma prediction sample is available.

Figure 1. Top-left luma sample used for deriving the chroma residual scaling.

JVET-N0389 Chroma residual scaling with separate luma/chroma tree [E. François, C. Chevance (Technicolor)]

One aspect from JVET-N0389 relates to the reduction of the number of luma samples used to derive the chroma residual scaling factor. In one of the investigated options of JVET-N0389, chroma residual scaling uses luma samples neighboring the luma block co-located with the top-left sample of the chroma block, as illustrated in Figure 2.

Figure 2. Examples of neighboring samples used for chroma residual scaling in dual-tree.

JVET-N0477 On luma mapping with chroma scaling [Y. Ye, J. Chen, R. Liao (Alibaba)]

JVET-N0477 proposes three modifications to the chroma scaling process, two of them being investigated in this CE. In the first proposed modification considered in this CE, the scaling factor computation is modified to use only up to 4x4 samples of the corresponding luma block. In the third modification considered in this CE, for intra coded blocks, the top-left reference sample is used to derive the chroma scaling factor. This modification is only applied to the conventional intra coding mode. For all other modes, the proposed simplified luma averaging is used.

Defined tests

In CE2-1 tests, chroma residual scaling (CRS) is not enabled for dual-tree case (separate luma-chroma partitioning tree), which means, CRS is disabled in Intra tiles group similar as VTM5.0 anchor.

| CE2-1 – Chroma residual scaling luma-chroma dependency reduction – Single-tree case |  |  |  |
| --- | --- | --- | --- |
| Test # | Description | Tester | Cross-checker |
| CE2-1.1 | Derive the scaling factor from the average value of the top-left 16×16 luma prediction samples | Z.-Y. Lin (MediaTek) |  |
| CE2-1.2 | Derivation of the scaling factor from the first (upper-left) sample value of the corresponding luma prediction | T. Lu (tlu@dolby.com) (Dolby) |  |
| CE2-1.3.a | For intra coded blocks, derivation of the scaling factor from the top-left, top or left neighboring reconstructed luma sample | E. Francois (Technicolor) |  |
| CE2-1.3.b | For intra coded blocks, derivation of the scaling factor from the average value of neighboring reconstructed luma samples | E. Francois (Technicolor) |  |
| CE2-1.3.c | For intra coded blocks, derivation of the scaling factor from the average value of up to N neighboring reconstructed luma samples | E. Francois (Technicolor) |  |
| CE2-1.4.a | For intra coded blocks, derivation of the scaling factor from the top-left reference luma sample used for intra prediction | R.-L Liao (Alibaba) |  |
| CE2-1.4.b | For intra coded blocks, derivation of the scaling factor from the averaged value of reference luma samples used for intra prediction | R.-L Liao (Alibaba) |  |
| CE2-1.4.c | For intra coded blocks, derivation of the scaling factor from the averaged value of up to N reference luma samples used for intra prediction | R.-L Liao (Alibaba) |  |

Detailed test descriptions

CE2-1.1 (JVET-N0113)

In the CE2-1.1, the chroma scaling factor is derived the average value of the top-left 16×16 luma prediction samples.

CE2-1.2 (JVET-N0220)

In the CE2-1.2, the chroma scaling factor is derived from the first (upper-left) sample value of the corresponding luma prediction.

CE2-1.3 (JVET-N0389)

In the CE2-1.3, for conventional intra coded block, luma reconstructed samples neighboring the luma block collocated with the top-left sample of the chroma block are used to derive the chroma scaling factor.

Three options are tested:

- CE2-1.3.a, the top-left, top or left neighboring luma reconstructed sample is used.

- CE2-1.3.b, the averaged value of neighboring luma reconstructed samples is used.

- CE2-1.3.c, the averaged value of up to N neighboring luma reconstructed samples is used.

CE2-1.4 (JVET-N0477)

In the CE2-1.4, for conventional intra coded block, luma reference samples used for intra prediction of the collocated luma block are used to derive the chroma scaling factor.

Three options are tested:

- CE2-1.4.a, the top-left luma reference sample is used.

- CE2-1.4.b, the averaged value of luma reference samples is used.

- CE2-1.4.c, the averaged value of up to N luma reference samples is used.

CE2-2: Chroma residual scaling luma-chroma dependency reduction – Dual-tree case

Contribution documents

Solutions from JVET-N0389 and JVET-N0477, described in section 4, are also investigated for the dual-tree case.

Defined tests

In CE2-2 tests, chroma residual scaling (CRS) is enabled for dual-tree case (separate luma-chroma partitioning tree) and the following methods are applied to all title groups.

| CE2-2 – Chroma residual scaling luma-chroma dependency reduction – Dual-tree case |  |  |  |
| --- | --- | --- | --- |
| Test # | Description | Tester | Cross-checker |
| CE2-2.1.a | CRS on in dual-tree For all intra coded blocks, scaling factor derived from the top-left, top or left reconstructed luma sample neighboring the collocated luma block | E. Francois (Technicolor) |  |
| CE2-2.1.b | CRS on in dual-tree For all intra coded blocks, scaling factor derived from the average of reconstructed luma samples neighboring the collocated luma block | E. Francois (Technicolor) |  |
| CE2-2.1.c | CRS on in dual-tree For all intra coded blocks, scaling factor derived from the average of up to N reconstructed luma samples neighboring the collocated luma block | E. Francois (Technicolor) |  |
| CE2-2.2.a | CRS on in dual-tree For all intra coded blocks, scaling factor derived from the top-left, top or left reference luma sample neighboring the collocated luma block | E. Francois (Technicolor) |  |
| CE2-2.2.b | CRS on in dual-tree For all intra coded blocks, scaling factor derived from the average of reference luma samples neighboring the collocated luma block | E. Francois (Technicolor) |  |
| CE2-2.2.c | CRS on in dual-tree For all intra coded blocks, scaling factor derived from the average of up to N reference luma samples neighboring the collocated luma block | E. Francois (Technicolor) |  |
| CE2-2.3 | CRS on in dual-tree For intra coded blocks in dual-tree, scaling factor derived from the average of reconstructed luma samples neighboring the collocated luma block For intra coded blocks in single-tree, scaling factor derived from the average of reference luma samples neighboring the collocated luma block | E. Francois (Technicolor) |  |

Detailed test descriptions

CE2-2.1 (JVET-N0389)

In the CE2-2.1, for intra coded blocks in single-tree and dual-tree cases, luma reconstructed samples neighboring the luma block collocated with the top-left sample of the chroma block are used to derive the chroma scaling factor.

Three options are tested:

- CE2-2.1.a, the top-left, top or left neighboring luma reconstructed sample is used.

- CE2-2.1.b, the averaged value of neighboring luma reconstructed samples is used.

- CE2-2.1.c, the averaged value of up to N neighboring luma reconstructed samples is used.

CE2-2-2 (JVET-N0477)

In the CE2-2.2, for intra coded blocks in single-tree and dual-tree cases, luma reference samples used for intra prediction, neighboring the luma block collocated with the top-left sample of the chroma block are used to derive the chroma scaling factor.

Three options are tested:

- CE2-2.2.a, the top-left, top or left neighboring luma reference sample is used.

- CE2-2.2.b, the averaged value of neighboring luma reference samples is used.

- CE2-2.2.c, the averaged value of up to N neighboring luma reference samples is used.

CE2-2.3 (JVET-N0389, JVET-N0477)

CE2-2.3 tests the following combination:

- For intra coded blocks in dual-tree, the chroma residual scaling factor is derived from the average of reconstructed luma samples neighboring the collocated luma block,

- For intra coded blocks in single-tree, the chroma residual scaling factor is derived from the average of reference luma samples neighboring the collocated luma block.

CE2-3: CCLM luma-chroma dependency reduction in Dual-tree case

Contribution documents

JVET-N0390 AHG16/non-CE3: Study of CCLM restrictions in case of separate luma/chroma tree [E. François, F. Galpin, F. Le Léannec, T. Poirier (Technicolor)]

This contribution proposes constraints on CCLM activation when separate luma/chroma tree is used. In one option (“restrict16x16”), CCLM is enabled only if the chroma block is inside a 16x16 non-overlapping area in a 16x16 sample grid, and if all its co-located luma samples belong to luma blocks included in the corresponding 32x32 area. In a second option (“restrict16x32or32x16”), the chroma non-overlapping area is of size 16x32 or 32x16, and the co-located luma blocks are constrained to be inside the corresponding 32x64 or 64x32 non-overlapping area. This is illustrated in Figure 3 where CCLM is enabled as the chroma block is inside a 16x16 non-overlapping area and its collocated luma blocks are also included into the corresponding 32x32 luma non-overlapping area.

Figure 3. Example of CCLM enabled according to the first restriction option.

Defined tests

| CE2-3 - CCLM luma-chroma dependency reduction in Dual-tree case |  |  |  |
| --- | --- | --- | --- |
| Test # | Description | Tester | Cross-checker |
| CE2-3.1 | CCLM enabled if chroma block and its collocated luma blocks are inside the same 16x16/32x32 non-overlapping area | E. Francois (Technicolor) |  |
| CE2-3.2 | CCLM parameters derived from reconstructed luma/chroma samples neighboring the luma block collocated with the chroma block top-left sample | E. Francois (Technicolor) |  |

Detailed test descriptions

CE2-3.1 (JVET-N0390)

In CE2-3.1, CCLM is enabled only if the chroma block is inside a 16x16 non-overlapping area, and if all its co-located luma samples belong to luma blocks included in the corresponding 32x32 area.

CE2-3.2 (JVET-N0389)

In CE2-3.2, CCLM parameters are derived from reconstructed luma/chroma samples neighboring the luma block collocated with the top-left sample of the chroma block.

Participants

The following table enumerates participants with tools in CE2 as well as participating cross-checkers and interested persons.

| Name | Company | Email | Proponent | Cross-checker |
| --- | --- | --- | --- | --- |
| Lu, Taoran Yin, Peng Fangjun Pu Sean McCarthy | Dolby | tlu@dolby.com pyin@dolby.com fpu@dolby.com Sean.McCarthy@dolby.com | Y |  |
| Chih-Wei Hsu Ching-Yeh Chen Tzu-Der Chuang Zhi-Yi Lin Yu-Wen Huang Olena Chubach | MediaTek | cw.hsu@mediatek.com chingyeh.chen@mediatek.com peter.chuang@mediatek.com zhi-yi.lin@mediatek.com yuwen.huang@mediatek.com Olena.Chubach@mediatek.com | Y |  |
| Jie Chen Ye, Yan R.-L Liao | Alibaba | jiechen.cj@alibaba-inc.com yan.ye@alibaba-inc.com ruling.lrl@alibaba-inc.com | Y |  |
| E. Francois T. Poirier F. Le Léannec P. Andrivon | Technicolor | edouard.francois@technicolor.com tangi.poirier@technicolor.com fabrice.leleannec@technicolor.com pierre.andrivon@technicolor.com | Y |  |
| Wei-Jung Chien Han Huang Chun-Chi Chen Vadim Seregin Geert Van der Auwera Yu Han Yao-Jen Chang | Qualcomm | wchien@chunchic@qti.qualcomm.com hanhuang@chunchic@qti.qualcomm.com chunchic@qti.qualcomm.com vseregin@qti.qualcomm.com geertv@qti.qualcomm.com yuhan@qti.qualcomm.com yjchang@qti.qualcomm.com |  |  |
| Li Zhang Kai Zhang Hongbin Liu Zhipin Deng | Bytedance | lizhang.idm@bytedance.com zhangkai.video@bytedance.com liuhongbin.01@bytedance.com zhipin.deng@bytedance.com |  |  |
| Solovyev Timofey Sergey Ikonin | Huawei | solovyev.timofey@huawei.com Sergey.Ikonin@huawei.com |  |  |
| Weimin Zeng Eric Chai | Ublnx | weimin.zeng@ublnx.com eric.chai@ublnx.com |  |  |
| Saverio Blasi Gosala kulupana | BBC | saverio.blasi@bbc.co.uk gosala.kulupana@bbc.co.uk |  |  |
| Dongjae Won Jong-Ki Han | Sejong university | bigdj2002@sju.ac.kr hjk@sejong.edu |  |  |
| Po-Han Lin Yu-Ciao Yang Tsung-Hua Li | Foxconn | PohanLin@fginnov.com yuciaoyang@fginnov.com thli@fginnov.com |  |  |
| Andrew Segall | Sharp | asegall@sharplabs.com |  |  |
| Ling Li Jaehyun Lim Seunghwan Kim Jie Zhao | LGE | l.li@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com jie.zhao@lge.com |  |  |
| Ruoyang Yu | Ericsson | ruoyang.yu@ericsson.com |  |  |
| Jingya Li | Panasonic | jingya.li@sg.panasonic.com |  |  |
| Jianhua Zheng | Hisilicon | zhengjianhua@hisilicon.com |  |  |
| Jianqing Zhu Wenting Cai Jie Yao | Fujitsu | zhujianqing@cn.fujitsu.com caiwenting@cn.fujitsu.com yaojie@cn.fujitsu.com |  |  |
| Junyan Huo Yanzhuo MA | Xidian University | jyhuo@mail.xidian.edu.cn huojunyanvideo@gmail.com yzma@mail.xidian.edu.cn |  |  |
| Steve.Keating | Sony | Steve.Keating@sony.com |  |  |
| adam.wieckowski | HHI | adam.wieckowski@hhi.fraunhofer.de |  |  |