| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N1023-v2 |
| --- | --- |

| Title: | Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Ling Li Alexey Filippov | Email: | geertv@qti.qualcomm.com l.li@lge.com Alexey.Filippov@huawei.com |
| Source: | CE3 coordinators |  |  |

_____________________________

# Abstract

This document describes Core Experiment 3 (CE3) on intra prediction and mode coding. The experiments are conducted after the 14th JVET meeting and the results, including cross-checks, will be reported to the 15th JVET meeting.

# Introduction

In CE3, a total of 21 tests are defined, which are organized in the following sub-CE categories:

- CE3-1: ISP (12 tests)

- CE3-2: Small chroma block size restrictions for shared tree (3 tests)

- CE3-3: CCLM model improvements (6 tests)

# Test conditions and guidelines

CTC conditions as agreed at the 14th JVET meeting (Geneva, CH) for SDR will be used for the evaluation of objective performance (JVET-N1010). The anchor is the ‘VVC Test Model’ or VTM. Both the ‘All Intra’ and ‘Random Access’ test configurations for all sub-CEs, and additionally the ‘Low Delay B’ configuration for CE3-2 shall be used and providing the results for each of these configurations is mandatory. The VTM version 5.0 software shall be used with ‘vtm’ configuration files to produce the VTM anchor. Tools shall be implemented into the VTM version 5.0 software.

Proposals will primarily be compared with respect to objective compression efficiency while complexity will also be assessed.

BD-rates, encoding and decoding time comparisons, shall be reported by all proponents for at least the following case: #2 vs. #1. Additional test results may be provided, for example: #2 vs. #3.

Numbering is defined as follows for the case where ‘ToolA’ is replacing an existing ‘ToolB’ in VTM:

1.           VTM,						VTM anchor

2.           VTM + ToolA,			VTM with ToolA enabled (as a “tool on” test)

3.           VTM − ToolB            	VTM with ToolB disabled

Note: If ‘ToolB’ does not exist, then #3 = #1.

Source codes corresponding with the test descriptions contained in this document shall be uploaded to the following CE3 GitLab project according to the agreed timeline:

TBD

# Timeline

| T1 | 17 April 2019 | CE test description finalized |
| --- | --- | --- |
| T2 | Test model software release + 2 weeks | Initial software of CE tests released; initial study by cross-checkers begins |
| T3 | 4 June 2019 (3 weeks before the next JVET meeting) | Software of CE tests is finalized; final cross-check begins |
| T4 | 18 June 2019 (Regular document deadline -1 week) | CE contribution document upload, including specification text, complete test results |
|  | 22 June 2019 (Regular document deadline -3 days) | Cross-check reports to be sent to CE coordinators |
|  | 25 June 2019 (Regular document deadline) | CE summary report upload |

# CE3-1: ISP

## Contribution documents

JVET-N0159 Non-CE3: Relaxation method of processing dependency in ISP for small blocks [K. Unno, K. Kawamura, Y. Kidani, S. Naito (KDDI)]

JVET-N0186 CE3-related: Simplification on Intra sub-partition coding mode [B. Wang, A. M. Kotra, S. Esenlik, H. Gao, J. Chen (Huawei)]

JVET-N0313 CE3-related: Low latency intra sub-partitions [R. Vanam, Y. He (InterDigital)]

JVET-N0330 Non-CE3: Intra sub-partition coding without thin partitions [T.-C. Ma, X. Xiu, Y.-W. Chen, X. Wang (Kwai Inc.)]

JVET-N0372 Non-CE3: ISP with independent sub-partitions for certain block sizes [S. De-Luxán-Hernández, B. Bross, T. Nguyen, V. George, B. Stabernack, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-N0432 AHG16/Non-CE3: On 1xN and 2xN subpartitions in intra subpartition coding [A. K. Ramasubramonian, G. Van der Auwera, T. Hsieh, V. Seregin, L. Pham Van, M. Karczewicz]

## Defined tests

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-1.1.1 | CU-based prediction is applied for CUs whose sub-partition size is 1xN or 2xN. | K. Unno (KDDI) |  |
| 3-1.1.2 | CU-based prediction is applied for CUs whose sub-partition size is 1xN. | K. Unno (KDDI) |  |
| 3-1.2.1 | Disable partitions in ISP that can result into either 1xN or 2xN sub-blocks. An 8xN block can be split into 2 4xN blocks | B. Wang (Huawei) |  |
| 3-1.2.2 | Disable partitions in ISP that can result into 1xN sub-blocks | B. Wang (Huawei) |  |
| 3-1.3 | Apply CU-based low latency ISP to 4×8, 8×4 CUs, and the sub-partition with 1xN or 2xN partition size is disabled. | H. Yang (Interdigital) |  |
| 3-1.4.1 | Remove ISP CU if it causes 1xN sub-partition | Tsung-Chuan Ma (Kwai Inc.) |  |
| 3-1.4.2 | Remove ISP CU if it causes 1xN or 2xN sub-partition | Tsung-Chuan Ma (Kwai Inc.) |  |
| 3-1.5.1 | Test 3-1.6 extending the independent sub-partitions concept to 4×4 blocks and using 4 transforms for all cases. The throughput constraint of at least 16 samples/cycle is kept. | S. De-Luxán-Hernández (HHI) |  |
| 3-1.5.2 | Simplification of ISP by using independent sub-partitions for the vertical split of 4×N blocks so that reconstructed samples can be written in groups having a width of at least 2 samples. | S. De-Luxán-Hernández (HHI) |  |
| 3-1.5.3 | Test 3-1.5.2 extending the independent sub-partitions concept to 4×4 blocks and using 4 transforms for all cases. The throughput constraint of at least 16 samples/cycle is kept. | S. De-Luxán-Hernández (HHI) |  |
| 3-1.6 | 4xN prediction regions for 4xN and 8xN (N > 4) ISP-coded blocks with vertical subpartitions | Adarsh K. Ramasubramonian (Qualcomm) S. De-Luxán-Hernández (HHI) |  |
| 3-1.7 | Test 3-1.6 with additional restriction of only allowing vertical prediction mode for 4xN ISP-coded blocks with vertical subpartitions | Adarsh K. Ramasubramonian (Qualcomm) |  |

The following tables provide an overview of the aspects of each test. The data is provided for luma CBs that can potentially be split into narrow 1xN and 2xN vertical sub-partitions (if a 4-way ISP split would be performed), i.e., luma CBs with width equal to 4 and 8. The first table specifies the sizes of dependent sub-partitions that require reconstruction of the neighbouring sub-partition to the left for prediction of the current sub-partition. The second table specifies the transform block sizes that are applied to the dependent sub-partitions. The VTM anchor data is provided for reference. Yellow highlights indicate tested changes with respect to the VTM anchor.

- Sub-partition sizes with prediction dependency (V: vertical splitting; H: horizontal splitting)

| Test# vs. CB size | 4x4 | 4x8 | 4xN (N>=16) | 8x4 | 8xN (N>=8) |
| --- | --- | --- | --- | --- | --- |
| VTM | No ISP | V: 2x8 H: 4x4 | V: 1xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.1.1 | No ISP | V: 4x8 H: 4x4 | V: 4xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 8xN H: 8x(N/4) |
| 3-1.1.2 | No ISP | V: 2x8 H: 4x4 | V: 4xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.2.1 | No ISP | V: No ISP H: 4x4 | V: No ISP H: 4x(N/4) | V: 4x4 H: 8x2 | V: 4xN H: 8x(N/4) |
| 3-1.2.2 | No ISP | V: 2x8 H: 4x4 | V: No ISP H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.3 | No ISP | V: No ISP H: 4x8 | V: No ISP H: 4x(N/4) | V: 8x4 H: 8x4 | V: 4xN H: 8x(N/4) |
| 3-1.4.1 | No ISP | V: 2x8 H: 4x4 | V: No ISP H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.4.2 | No ISP | V: No ISP H: 4x4 | V: No ISP H: 4x(N/4) | V: 4x4 H: 8x2 | V: No ISP H: 8x(N/4) |
| 3-1.5.1 | V: 4x4 H: 4x4 | V: 4x8 H: 4x4 | V: 4xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 4xN H: 8x(N/4) |
| 3-1.5.2 | No ISP | V: 2x8 H: 4x4 | V: 2xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.5.3 | V: 4x4 H: 4x4 | V: 2x8 H: 4x4 | V: 2xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.6 | No ISP | V: 4x8 H: 4x4 | V: 4xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 4xN H: 8x(N/4) |
| 3-1.7 | No ISP | V: 4x8 (*) H: 4x4 | V: 4xN (*) H: 4x(N/4) | V: 4x4 H: 8x2 | V: 4xN H: 8x(N/4) |

(*) VER pred mode only

- Transform block sizes:

| Test# vs. CB size | 4x4 | 4x8 | 4xN (N>=16) | 8x4 | 8xN (N>=8) |
| --- | --- | --- | --- | --- | --- |
| VTM | No ISP | V: 2x8 H: 4x4 | V: 1xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.1.1 | No ISP | V: 2x8 H: 4x4 | V: 1xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.1.2 | No ISP | V: 2x8 H: 4x4 | V: 1xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.2.1 | No ISP | V: No ISP H: 4x4 | V: No ISP H: 4x(N/4) | V: 4x4 H: 8x2 | V: 4xN H: 8x(N/4) |
| 3-1.2.2 | No ISP | V: 2x8 H: 4x4 | V: No ISP H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.3 | No ISP | V: No ISP H: 4x4 | V: No ISP H: 4x(N/4) | V: 4x4 H: 8x2 | V: 4xN H: 8x(N/4) |
| 3-1.4.1 | No ISP | V: 2x8 H: 4x4 | V: No ISP H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.4.2 | No ISP | V: No ISP H: 4x4 | V: No ISP H: 4x(N/4) | V: 4x4 H: 8x2 | V: No ISP H: 8x(N/4) |
| 3-1.5.1 | V: 1x4 H: 4x1 | V: 1x8 H: 4x2 | V: 1xN H: 4x(N/4) | V: 2x4 H: 8x1 | V: 2xN H: 8x(N/4) |
| 3-1.5.2 | No ISP | V: 2x8 H: 4x4 | V: 1xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.5.3 | V: 1x4 H: 4x1 | V: 1x8 H: 4x2 | V: 1xN H: 4x(N/4) | V: 2x4 H: 8x1 | V: 2xN H: 8x(N/4) |
| 3-1.6 | No ISP | V: 2x8 H: 4x4 | V: 1xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.7 | No ISP | V: 2x8 H: 4x4 | V: 1xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |

## Test descriptions

### Relaxation method of processing dependency in ISP for small blocks (JVET-N0159 KDDI)

The proposed method that switches processing unit of prediction between sub-partition (same as TU) and CU for CUs applied ISP is tested.

If a sub-partition width is less than a threshold, prediction is processed by CU. It is same as the intra prediction process when ISP is not applied.

Otherwise (if the sub-partition width is greater than or equal to the threshold), prediction is processed by sub-partition. It is same as the ISP process in VTM-5.

The threshold value is defined as follows.

- Test 3-1.1.1: The threshold is 4 pixels.

  - CU-based prediction is applied for CUs whose sub-partition size is 1xN or 2xN.

- Test 3-1.1.2: The threshold is 2 pixels.

  - CU-based prediction is applied for CUs whose sub-partition size is 1xN.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-1.1.1 | CU-based prediction is applied for CUs whose sub-partition size is 1xN or 2xN. | K. Unno (KDDI) |  |
| 3-1.1.2 | CU-based prediction is applied for CUs whose sub-partition size is 1xN. | K. Unno (KDDI) |  |

### Simplification on intra sub-partition coding mode (JVET-N0186 Huawei)

When Intra sub-partition coding mode (ISP) is enabled, an intra block can be divided into 1xN or 2xN sub-blocks. These blocks are not friendly for hardware implementation. In these CE tests, two sub-tests are defined:

- Sub-Test 1: Disable partitions in ISP that can result into either 1xN or 2xN sub-blocks. An 8xN block can be split into 2 4xN blocks.

- Sub-Test 2: Disable partitions in ISP that can result into 1xN sub-blocks.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-1.2.1 | Disable partitions in ISP that can result into either 1xN or 2xN sub-blocks. An 8xN block can be split into 2 4xN blocks | B. Wang (Huawei) |  |
| 3-1.2.2 | Disable partitions in ISP that can result into 1xN sub-blocks | B. Wang (Huawei) |  |

### Low latency intra sub-partitions (JVET-N0313 Interdigital)

In the CU-based low latency ISP, the above reference row and left reference column for each sub-partition are from CU boundaries. For example, in Figure 1, the sub-partitions A, B, C and D are predicted using the reference samples adjoining the CU. This method is applied to 4x8 and 8x4 CUs. The sub-partition with partition size 1xN or 2xN is disabled. When 8x4 and 4x8 CUs are split, the sub-partition prediction will follow the CU-based low latency ISP approach.

Figure 1. CU-based low latency ISP.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-1.3 | Apply CU-based low latency ISP to 4×8, 8×4 CUs, and the sub-partition with 1xN or 2xN partition size is disabled. | H. Yang (Interdigital) |  |

### Intra sub-partition coding without thin partitions (JVET-N0330 Kwai Inc.)

In this CE test, the coding loss of disabling vertical thin sub-partition are studied. The ISP mode splits current coding unit into 4 or 2 sub-blocks according to the size of current block. When the ISP is enabled, depending on the selected intra direction, the reconstructed samples of one ISP subblock can be used as references to predict the samples of its following subblock. In other words, the intra prediction of one ISP subblock cannot be started until its preceding ISP subblock (according to the ISP coding order) is fully reconstructed. Hence, such dependency causes decoding throughput issue, especially for the ISP CUs with small sub-partitions, e.g., 1xN or 2xN. For example, ISP with 4xN block consumes 4 times processing cycles than non-ISP 4xN block, because it decodes each 1xN sub-block sequentially and therefore the decoding throughput of ISP 4xN block is roughly one-quarter of that of non-ISP (normal intra mode) 4xN block.

In the first test, we plan to test the impact of coding efficiency if the ISP CU causing 1xN sub-partitions is disabled.

In the second test, we plan to test the impact of coding efficiency if the ISP CU causing 1xN or 2xN sub-partition is disabled.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-1.4.1 | Remove ISP CU if it causes 1xN sub-partition | Tsung-Chuan Ma (Kwai Inc.) |  |
| 3-1.4.2 | Remove ISP CU if it causes 1xN or 2xN sub-partition | Tsung-Chuan Ma (Kwai Inc.) |  |

### ISP with independent sub-partitions for certain block sizes (JVET-N0372 HHI)

As presented in JVET-M0102, the Intra Sub-Partitions (ISP) coding mode divides luma intra-predicted blocks vertically or horizontally into 2 or 4 equal-size sub-partitions according to the CU dimensions. Table 1 shows the different possibilities.

Table 1: Number of sub-partitions created by ISP according to the CU dimensions in the current VVC Draft

| Block size | Number of sub-partitions |
| --- | --- |
| 4×4 | Not divided |
| 4×8 and 8×4 | 2 |
| All other cases | 4 |

It can be observed that this type division can lead to blocks with a width smaller than 4. The usage of such sub-partitions has been mentioned on the JVET reflector as a potential hardware concern, given that samples are typically allocated in a raster scanner fashion and certain implementations write the output of reconstructed samples in groups. In fact, typically a number of samples will be collected and written back to the external memory by a cache or buffer management component, which is part of the memory subsystem of state-of-art decoder implementations. This is important since DDR memories work with burst wise access rather than with access of small chunks of data to reach the highest throughput. Since this applies not only for the presented tool, a decoder architecture needs to take care of this behavior and has to provide a buffer functionality anyways, which can be used as well for rearranging pixel data into raster scan order when writing back.

Therefore, the complexity of implementing these sub-partitions would depend on the type of hardware implementation used. Hence, it is argued that the usage of these sub-partitions is justified by its impact on the coding efficiency.

For this reason, document JVET-N0372 investigated the impact of removing 1×N (N≥16) and 2×M (M≥8) sub-partitions from the current ISP design. To do so, the vertical split of 4×N blocks was disabled (the horizontal split was always inferred) and 8×N blocks were vertically divided into 2 sub-partitions instead of 4. The impact of these modifications led to losses of 0.14% (CTC) and 0.31% (class F) for AI, 0.04% (CTC) and 0.23% (class F) for RA and 0.02% (CTC) and 0.13% (class F) for LDB. In all three configurations there was no significant reduction in the encoder run-time.

These losses are significant and would not justify the removal of these sub-partitions. For this reason, JVET-N0372 proposed to modify the 1×N and 2×M sub-partitions in the following way:

- Vertical split of 4×N blocks: the reference samples used to predict each sub-partition are the neighbouring samples of the CU, instead of the TU. This is the same as predicting the whole 4×N  CU at once, generating a 4×N residual signal and then dividing it into 4 TUs that are transformed and entropy coded in the usual way done by ISP. The four sub-partitions are independent from each other and hence they can be processed in parallel in a single step. Figure 2 shows an example of this process. Note that now the vertical split of a 4×8 will produce 4 sub-partitions instead of 2.

- Vertical split of 8×N blocks: in this case the block is divided into 4 sub-partitions with indices from 1 to 4 as usual. However, the prediction signals of sub-partitions 2 and 4 cannot be generated using the reconstructed samples of sub-partitions 1 and 3 respectively, as shown in the example of Figure 3. Consequently, sub-partition 2 is independent from sub-partition 1 and analogously sub-partition 4 is independent from sub-partition 3. Therefore, sub-partitions 1 and 2 can be processed in parallel and the same can be said about sub-partitions 3 and 4. As a result, reconstructed samples can be written in memory at once in groups having a width of at least 4 samples. Note that now the vertical split of an 8×4 block will produce 4 sub-partitions instead of 2.

Figure 2: Example of the vertical split of a 4×16 block into four independent 1×16 sub-partitions. Only the CU neighbouring samples can be used to calculate the prediction signals.

Figure 3: Example of the vertical split of a 8×16 block into four 2×16 sub-partitions. In this case the reconstructed samples from sub-partition 2 can be used to predict sub-partition 3, but the reconstructed samples from sub-partitions 1 and 3 cannot be used to predict 2 and 4 respectively. This is equivalent to having two 4×N prediction signals.

Furthermore, if these modifications are built as a hardware implementation to avoid having to buffer the samples of sub-partitions with a width smaller than 4, there would not be any extra cost in terms of data flow in extending the same procedure to 4×4 blocks and to modifiy the number of sub-partitions of 4×8 and 8×4 blocks in the following way:

- 4×4 blocks: they will be treated in the same way as the aforementioned method used for the vertical split of 4×N blocks (analogously in the horizontal split case).

- Horizontal split of 4×8 and 8×4 blocks: they will now use 4 sub-partitions instead of 2 in a analogous way as the proposed modification for the vertical split of 8×N blocks. This also unifies the design, since it ensures that 4 sub-partitions will be used in all cases (instead of having 2 or 4 depending on the blocks size and the split type).

Note that these modifications would keep the throughput constraint of at least 16 samples/cycle. More details about the proposed modifications can be found in JVET-N0372.

Furthermore, it is also proposed to study the impact of modifying 1×N sub-partitions (leaving 2×M sub-partitions unaltered) by using analogous techniques as the ones described before. In this case reconstructed samples would be written in groups of 2 in the worst-case scenario.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-1.5.1 | Test 3-1.6 extending the independent sub-partitions concept to 4×4 blocks and using 4 transforms for all cases. The throughput constraint of at least 16 samples/cycle is kept. | S. De-Luxán-Hernández (HHI) |  |
| 3-1.5.2 | Simplification of ISP by using independent sub-partitions for the vertical split of 4×N blocks so that reconstructed samples can be written in groups having a width of at least 2 samples. | S. De-Luxán-Hernández (HHI) |  |
| 3-1.5.3 | Test 3-1.5.2 extending the independent sub-partitions concept to 4×4 blocks and using 4 transforms for all cases. The throughput constraint of at least 16 samples/cycle is kept. | S. De-Luxán-Hernández (HHI) |  |

### On 1xN and 2xN subpartitions in intra subpartition coding (JVET-N0432 Qualcomm, JVET-N0372 HHI)

This test evaluates a modification to the handling of ISP-coded coding blocks that generate 1xN and 2xN subpartitions in VTM. For coding blocks of size 4xN that are ISP-coded with vertical split, prediction is performed by considering the 4xN block as a whole. For coding blocks of size 8xN (N > 4) that are ISP-coded with vertical split, prediction is performed in two 4xN prediction regions of samples (Figure 4); the prediction of samples in the second prediction region may depend on the reconstructed samples of the first prediction region.

Figure 4: An 8xN (N > 4) coding block coded with ISP and split vertically; prediction is done on 4xN prediction regions and the transform block (TB) size is 2xN

In addition to the constraints above, only the vertical intra prediction mode may be allowed for 4xN coding blocks that are ISP-coded with vertical split; the intra mode is not signalled in this case.

Table 1 provides a summary of the prediction region sizes and transform sizes for ISP-coded coding blocks that are vertically split. The ISP-coding of other blocks is unchanged.

Table 1: Prediction region sizes and transform sizes tested for vertical ISP splits; ISP coding of other blocks is unchanged.

| Coding block sizes | Test 3-1.6 |  |
| --- | --- | --- |
|  | Prediction region sizes | Transform sizes |
| 4x8 | 4x8 | 2xN |
| 4xN (N > 8) | 4xN | 1xN |
| 8xN (N > 4) | 4xN | 2xN |

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-1.6 | 4xN prediction regions for 4xN and 8xN (N > 4) ISP-coded blocks with vertical subpartitions | Adarsh K. Ramasubramonian (Qualcomm) S. De-Luxán-Hernández (HHI) |  |

### On 1xN and 2xN subpartitions in intra subpartition coding (JVET-N0432 Qualcomm)

In addition to test 3-1.6, only the vertical intra prediction mode may be allowed for 4xN coding blocks that are ISP-coded with vertical split; the intra mode is not signalled in this case.

The following table provides a summary of the prediction region sizes and transform sizes for ISP-coded coding blocks that are vertically split. The ISP-coding of other blocks is unchanged.

Table 2: Prediction region sizes and transform sizes tested for vertical ISP splits; ISP coding of other blocks is unchanged.

| Coding block sizes | Test 3-1.7 |  |
| --- | --- | --- |
|  | Prediction region sizes | Transform sizes |
| 4x8 | 4x8 (vertical mode only) | 2xN |
| 4xN (N > 8) | 4xN (vertical mode only) | 1xN |
| 8xN (N > 4) | 4xN | 2xN |

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-1.7 | Test 3-1.6 with additional restriction of only allowing vertical prediction mode for 4xN ISP-coded blocks with vertical subpartitions | Adarsh K. Ramasubramonian (Qualcomm) |  |

# CE3-2: Small chroma block size restrictions for shared tree

## Contribution documents

JVET-N0082 CE3-related: Constrained partitioning of chroma intra CBs [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0396 CE3-related: Enabling parallel reconstruction of small intra-coded blocks [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, H. Huang, M. Karczewicz (Qualcomm)]

JVET-N0453 AHG16/Non-CE3: on chroma 2xN and Nx2 intra prediction in single tree [Y. Zhao, A. Karabutov, H. Yang, J. Chen (Huawei)]

## Defined tests

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-2.1.1 | Do not split chroma blocks of smallest chroma intra prediction units (SCIPU) - SCIPU determined from the coding mode of the first CU inside SCIPU | Zhi-Yi Lin (MediaTek) Yin Zhao (Huawei) | Chris Rosewarne (Canon) |
| 3-2.1.2 | Do not split chroma blocks of smallest chroma intra prediction units (SCIPU) - SCIPU determined from a flag in coding_tree() | Zhi-Yi Lin (MediaTek) Yin Zhao (Huawei) | Chris Rosewarne (Canon) |
| 3-2.2 | All coded blocks within a parallel-processable region (PPR) of size larger than or equal to 16 chroma samples to be reconstructed in parallel in the single-tree case by removing the dependency between blocks within the PPR. | Luong Pham Van (Qualcomm) Zhi-Yi Lin (MediaTek) | Chris Rosewarne (Canon) |

## Test descriptions

### Constrained partitioning of small chroma intra CBs (JVET-N0082 MediaTek, JVET-N0453 Huawei)

The goal of the proposed method is to disallow intra CBs smaller than 16 samples by constraining the partitioning of chroma intra CBs.

In single coding tree, it is proposed to define smallest chroma intra prediction units (SCIPUs). A SCIPU corresponds to a non-leaf node of a coding tree. Luma component of a SCIPU can be further split and chroma components of the SCIPU might not be further split. The size of the SCIPU is larger than or equal to 16 chroma samples. In a SCIPU, all CUs have to be consistently coded in intra or inter, where intra here includes regular intra modes and intra block copy (IBC) mode. When all CUs within a SCIPU is consistently coded in intra, the chroma components cannot be further split. By applying the above method, there is no 2x2/2x4/4x2 chroma intra CU. The worst case hardware processing throughput is the same as that in HEVC and is 4X of that in VTM4.0.

Two signalling methods are tested to indicate the prediction modes used in the SCIPU. In the first method, the prediction mode signalling in the first CU of the SCIPU is unchanged. According to the signalled prediction mode of the first CU of the SCIPU, the prediction modes of the rest CUs are constrained. In the second method, a flag (namely mode_cons_flag) is parsed right after the split mode syntax in the coding_tree() syntax structure corresponding to the SCIPU. If mode_cons_flag is 1, all CUs in the SCIPU are allowed to use only Intra/IBC mode, and the luma blocks split while and chroma blocks does not split; otherwise, all CUs in the SCIPU are allowed to use only Inter mode, and the luma and chroma split jointly. For CUs in an Intra-type SCIPU, cu_skip_flag and pred_mode_flag are inferred as 0 and 1 if IBC is off, or only pred_mode_flag is inferred as 1 when IBC is on. For CUs in an Inter-type SCIPU, pred_mode_flag is inferred as 0.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-2.1.1 | Do not split chroma blocks of smallest chroma intra prediction units (SCIPU) - SCIPU determined from the coding mode of the first CU inside SCIPU | Zhi-Yi Lin (MediaTek), Yin Zhao (Huawei) | Chris Rosewarne (Canon) |
| 3-2.1.2 | Do not split chroma blocks of smallest chroma intra prediction units (SCIPU) - SCIPU determined from a flag in coding_tree() | Zhi-Yi Lin (MediaTek), Yin Zhao (Huawei) | Chris Rosewarne (Canon) |

### Enabling parallel reconstruction of small chroma blocks (JVET-N0396 Qualcomm)

This method allows multiple chroma blocks within a parallel-processable region (PPR) of size larger than or equal to 16 chroma samples to be reconstructed in parallel in the single-tree case. The prediction dependency between the blocks in a 16-sample PPR is eliminated by a marking the blocks within the PPR as “unavailable” for predicting other blocks contained in this PPR. With a PPR region of 32 (8x4/4x8) samples including a 4x4 block, the 4x4 block is marked as “unavailable” for others blocks in the same region while it can use the top or left block in the region for intra prediction. The VVC reference sample substitution process automatically pads the unavailable reference samples for prediction.

Figure 5: PPR example with the minimum number of chroma samples of 16

Figure 6: PPR example with the minimum number of chroma samples of 32 with small blocks

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-2.2 | All coded blocks within a parallel-processable region (PPR) of size larger than or equal to 16 chroma samples to be reconstructed in parallel in the single-tree case by removing the dependency between blocks within the PPR. | Luong Pham Van (Qualcomm) Zhi-Yi Lin (MediaTek) | Chris Rosewarne (Canon) |

# CE3-3: CCLM model improvements

## Contribution documents

JVET-N0524 Non-CE3: Modified beta derivation in CCLM [M. Ikeda, T. Suzuki (Sony)]

JVET-N0716 CE1-related: Simplified and robust LIC parameter derivation unified with CCLM [A. Filippov, V. Rufitskiy, J. Chen (Huawei), C. Helmrich, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

## Defined tests

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-3.1 | Modified beta parameter derivation in context of the CCLM simplification (JVET-N0271) | M. Ikeda (Sony) |  |
| 3-3.2.1 | FIR filtering is applied to both luma and chroma templates to remove outliers | Vasily Rufitskiy (Huawei) |  |
| 3-3.2.2 | Test CE3-3.2.1 with calculating mean values for more robust parameter β deriving | Vasily Rufitskiy (Huawei) |  |
| 3-3.2.3 | Test CE3-3.2.2 without template filtering | Alexey Filippov (Huawei) |  |
| 3-3.2.4 | Test CE3-3.2.1 with calculating mean values for more robustly deriving both parameters α and β | Alexey Filippov (Huawei), Christian Helmrich (HHI) |  |
| 3-3.2.5 | Test CE3-3.2.4 without template filtering | Alexey Filippov (Huawei), Christian Helmrich (HHI) |  |

## Test descriptions

### Modified beta derivation in CCLM (JVET-N0524 Sony)

It is proposed that neighbouring luma and chroma averaged values are used to derive the CCLM β parameter. The averaged values were originally calculated for all neighbouring samples. In this test, however, the proposed modification is tested in context of the CCLM simplification (JVET-N0271) which was adopted in 14th Geneva meeting.

Then α derivation is not changed and β is obtained according to the averaged values (xC and yC).

α=yB-yAxB-xA

β=yC-α*xC

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-3.1 | Modified beta parameter derivation in context of the CCLM simplification (JVET-N0271) | M. Ikeda (Sony) |  |

### Robust CCLM parameter derivation (JVET-N0716 Huawei, HHI)

In the perspective of CCLM, contribution JVET-N0716 proposes to derive CCLM parameters α and β using a more robust mechanism described below:

- Template filtering: as proposed in JVET-N0410 and JVET-N0716, FIR filtering can be applied to both luma and chroma templates to remove outliers using smoothing FIR filtering to make CCLM parameter derivation process more robust. The number of neighbouring reconstructed samples used in proposed filtering and model derivation should be aligned with VTM-5.

- Mean values: as proposed in JVET-M0356, mean values ymean and xmean are computed using chroma and luma templates, respectively. To avoid division operation in the case of rectangular blocks, mean values are calculated by performing averaging separately for each (top and left) side and then both obtained results are averaged with each other to get final values of ymean and xmean. Parameter β is computed as follows:

β=ymean-α*xmean

- 3. Slope value: as proposed in JVET-M0093, JVET-M0274, and JVET-N0271, the slope parameter α can be derived not only from the smallest luma value A and the largest luma value B but also from the second-smallest luma value A' and the second-largest luma value B'. In this test, we propose to replace the averaging operations (A + A' + 1) >> 1, (B + B' + 1) >> 1 of the CCLM simplification adopted at the 14th JVET meeting by the simpler “inaccurate” averaging operations (A + A') >> 1, (B + B') >> 1 from JVET-M0093, thus saving some additions.ab

- ymeanxmean

b=ymean-a*xmean

Figure 7

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-3.2.1 | FIR filtering is applied to both luma and chroma templates to remove outliers | Vasily Rufitskiy (Huawei) |  |
| 3-3.2.2 | Test CE3-3.2.1 with calculating mean values for more robust parameter β deriving | Vasily Rufitskiy (Huawei) |  |
| 3-3.2.3 | Test CE3-3.2.2 without template filtering | Alexey Filippov (Huawei) |  |
| 3-3.2.4 | Test CE3-3.2.1 with calculating mean values for more robustly deriving both parameters α and β | Alexey Filippov (Huawei), Christian Helmrich (HHI) |  |
| 3-3.2.5 | Test CE3-3.2.4 without template filtering | Alexey Filippov (Huawei), Christian Helmrich (HHI) |  |

# Participants

The following table enumerates participants with tools in CE3 as well as participating cross-checkers and interested persons.

| Name | Company | Email | CE3-x Proponent | Cross-checker |
| --- | --- | --- | --- | --- |
| Geert Van der Auwera Vadim Seregin Nan Hu Ted Hsieh Adarsh Krishnan Ramasubramonian Luong Pham Van Chun-Chi Chen Yan Zhang Chao-Hsiung Hung Hilmi Enes Egilmez Yao-Jen Chang | Qualcomm | geertv@qti.qualcomm.com vseregin@qti.qualcomm.com nanh@qti.qualcomm.com thsieh@qti.qualcomm.com aramasub@qti.qualcomm.com lphamvan@qti.qualcomm.com chunchic@qti.qualcomm.com yzh@qti.qualcomm.com chaohsiu@qti.qualcomm.com hegilmez@qti.qualcomm.com yjchang@qti.qualcomm.com | 1, 2 |  |
| Xin Zhao Liang Zhao Jing Ye Xiang Li Shan Liu | Tencent | xinzzhao@tencent.com leolzhao@tencent.com jingye@tencent.com xlxiangli@tencent.com shanl@tencent.com |  |  |
| Fabien Racapé Gagan Rath Fabrice Urban Franck Galpin Francois Edouard Christophe Chevance | Technicolor | Fabien.Racape@technicolor.com Gagan.rath@technicolor.com Fabrice.urban@technicolor.com Franck.galpin@technicolor.com Edouard.Francois@technicolor.com christophe.chevance@technicolor.com |  |  |
| Jani Lainema Ramin Ghaznavi-Youvalari | Nokia | jani.lainema@nokia.com ramin.ghaznavi-youvalari@nokia.com |  |  |
| Virginie Drugeon Che-Wei Kuo Chong Soon Lim | Panasonic | Virginie.Drugeon@eu.panasonic.com chewei.kuo@sg.panasonic.com chongsoon.lim@sg.panasonic.com |  |  |
| Jaehyun Lim SeungHawn Kim Jin Heo Jangwon Choi Sunmi Yoo Ling Li Hyeongmun Jang Naeri Park Jungah Choi | LGE | jaehyun.lim@lge.com seunghwan3.kim@lge.com jin78.heo@lge.com jangwon84.choi@lge.com sunmi.yoo@lge.com l.li@lge.com hm.jang@lge.com naeri.park@lge.com jungah.choi@lge.com |  |  |
| Jungwon Kang Jinho Lee Sung-Chang Hahyun Lee | ETRI | jungwon@etri.re.kr jinosoul@etri.re.kr sclim@etri.re.kr hanilee@etri.re.kr |  |  |
| Jonathan Pfaff Heiko Schwarz Benjamin Bross Philipp Merkle Santiago de Luxán Hernández Philipp Helle Bjoern Stallenberger Christian Helmrich | HHI | jonathan.pfaff@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de benjamin.bross@hhi.fraunhofer.de philipp.merkle@hhi.fraunhofer.de santiago.de.luxan@hhi.fraunhofer.de philipp.helle@hhi.fraunhofer.de bjoern.stallenberger@hhi.fraunhofer.de christian.helmrich@hhi.fraunhofer.de | 1, 3 |  |
| Po-Han Lin Yu-Chiao Yang Tsung-Hua Li | Foxconn | PohanLin@fginnov.com yuciaoyang@fginnov.com thli@fginnov.com |  |  |
| Alexey Filippov Sergey Ikonin Jianle Chen Anand Meher Kotra Xiang Ma Vasily Rufitskiy Sychev Maxim Biao Wang Seungwook Hong Haitao Yang Semih Esenlik Yin Zhao | Huawei | Alexey.Filippov@huawei.com Sergey.Ikonin@huawei.com jianle.chen@huawei.com anand.meher.kotra@huawei.com maxiang6@huawei.com Vasily.Rufitskiy@huawei.com Sychev.Maxim@huawei.com biao.wang@huawei.com seuhong@outlook.com haitao.yang@huawei.com semih.esenlik@huawei.com yin.zhao@huawei.com | 1, 2, 3 |  |
| Kei Kawamura Yoshitaka Kidani Sei Naito Kyohei Unno | KDDI | ki-kawamura@kddi.com yo-kidani@kddi.com se-naitou@kddi.com ky-unno@kddi-research.jp | 1 |  |
| Peter Chuang, ChingYeh Chen, Yu-Wen Huang, Chih-Wei Hsu, Chia-Ming Tsai, Zhi-Yi Lin, Mohammed Sarwer Olena Chubach Man-Shu Chiang | MediaTek | peter.chuang@MediaTek.com chingyeh.chen@MediaTek.com yuwen.huang@MediaTek.com cw.hsu@MediaTek.com Chia-ming.Tsai@MediaTek.com Zhi-Yi.Lin@MediaTek.com Mohammed.Sarwer@MediaTek.com Olena.Chubach@mediatek.com Man-Shu.Chiang@mediatek.com | 2 |  |
| Yinji Piao Narae Choi Kiho Choi | Samsung | yj1003.piao@samsung.com narae1.choi@samsung.com kiho14.choi@samsung.com m.w.park@samsung.com |  |  |
| Ching-Chieh Lin PoHan Lin | ITRI | jacklin@itri.com pohan@itri.com |  |  |
| Steve Keating Karl Sharman Masaru Ikeda Kenji Kondo | Sony | Steve.Keating@sony.com karl.sharman@sony.com masaru.ikeda@sony.com Kenji.Kondo@sony.com | 3 |  |
| Shimpei Nemoto Shunsuke Iwamura | NHK | nemoto.s-fy@nhk.or.jp iwamura.s-gc@nhk.or.jp |  |  |
| Andre Seixas Dias Saverio Blasi Gosala Kulupana | BBC | Andre.SeixasDias@bbc.co.uk saverio.blasi@bbc.co.uk gosala.kulupana@bbc.co.uk |  |  |
| Andrew Segall Kiran Misra Yukinobu Yasugi Takeshi Chujoh Tomohiro Ikai Eiishi Sasaki Tomonori Hashimoto Frank Bossen Tianyang Zhou | Sharp | asegall@sharplabs.com misrak@sharplabs.com yasugi.yukinobu@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp eiichi.sasaki@sharp.co.jp tomonori.hashimoto@sharp.co.jp frank@bossentech.com zhou.tianyang@sharp.co.jp |  |  |
| Zhi Zhang Rickard Sjoberg | Ericsson | zhi.a.zhang@ericsson.com rickard.sjoberg@ericsson.com |  |  |
| Yongbing Lin Quanhe Yu Jianhua Zheng | HiSilicon | linyongbing@hisilicon.com yuquanhe@hisilicon.com zhengjianhua@hisilicon.com |  |  |
| Dongcheol Kim Jason Jung | WILUS | dongcheol.kim@wilusgroup.com jason.jung@wilusgroup.com |  |  |
| Xingchun Qi Shurui Ye Liying Xu | Hikvision | qixingchun@hikvision.com yeshurui@hikvision.com xuliying5@hikvision.com |  |  |
| D. Y. Kim | Chips& Media | brad.kim@chipsnmedia.com |  |  |
| Elie Mora Anthony Nasrallah | ATEME | e.mora@ateme.com a.nasrallah@ateme.com m.abdoli@ateme.com t.guionnet@ateme.com |  |  |
| Lin Tao | Tongji Univ. | lintao@tongji.edu.cn |  |  |
| Xianglin Wang Yi-Wen Chen Xiaoyu Xiu Tsung-Chuan Ma | Kwai | xianglinwang@kwai.com yiwenchen@kwai.com xiaoyuxiu@kwai.com tsung-chuanma@kwai.com jhuhong-jheng@kwai.com | 1 |  |
| Sriram Sethuraman | Ittiam | sriram.sethuraman@ittiam.com |  |  |
| Chris Rosewarne Patrice Onno Guillaume Laroche Jonathan Taquet Christophe Gisquet Jonathan Gan | Canon | chris.rosewarne@canon.com.au Patrice.Onno@crf.canon.fr guillaume.laroche@crf.canon.fr jonathan.taquet@crf.anon.fr Christophe.Gisquet@crf.canon.fr jonathan.gan@canon.com.au |  |  |
| Byeungwoo Jeon Jeeyoon Park | SKKU | bjeon@skku.edu jiyoonpark@skku.edu |  |  |
| Liu Hongbin Kai Zhang Li Zhang Weijia Zhu Cynthia Zhangna Deng Zhipin | Bytedance | liuhongbin.01@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com weijia.zhu@bytedance.com zhangna.cynthia@bytedance.com zhipin.deng@bytedance.com |  |  |
| Hari Kalva | Florida Atlantic Univ. | hari.kalva@fau.edu |  |  |
| Minhua Zhou Peisong Chen Wade Wan | Broadcom | minhua.zhou@broadcom.com peisong.chen@broadcom.com wade.wan@broadcom.com |  |  |
| Rahul Vanam Yuwen He Saurav Bandyopadhyay Daniel Luo Hua Yang Wei Chen | Interdigital | rahul.vanam@interdigital.com yuwen.he@interdigital.com Saurav.Bandyopadhyay@InterDigital.com Daniel.Luo@InterDigital.com Hua.Yang@InterDigital.com Wei.Chen@InterDigital.com | 1 |  |
| Pierrick Philippe | Orange | pierrick.philippe@orange.com |  |  |
| Weimin Zeng Eric Chai | Ubilinx | weimin.zeng@ublnx.com eric.chai@ublnx.com |  |  |
| Dan Grois | Comcast | groisz@yahoo.com |  |  |
| Jongseok Lee | Kwangwoon University | suk2080@kw.ac.kr |  |  |
| Fannie Yu | OPPO | yuyuanfang@oppo.com |  |  |
| Peng Yin | Dolby | pyin@dolby.com |  |  |
| Haechul Choi Daehyeok Gwon Heeji Han Jaeryun Choe | Hanbat University | choihc@hanbat.ac.kr skyeye0530@naver.com sarhakor@naver.com tjswo9395@naver.com |  |  |
| Grace Yu | Intel | grace.yu@intel.com |  |  |
| Yiming Li Zizheng Liu | Wuhan University | userlym@whu.edu.cn lzz8246@whu.edu.cn |  |  |
| Jianqing Zhu Kimihiko Kazui Jie Yao | Fujitsu | zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com yaojie@cn.fujitsu.com |  |  |
| Junyan Huo Yanzhuo Ma | Xidian University | jyhuo@mail.xidian.edu.cn yzma@mail.xidian.edu.cn |  |  |
| Dong-jae Won | Sejong University | bigdj2002@sju.ac.kr |  |  |
| Shuai Wan | Northwestern Polytechnical University | swan@nwpu.edu.cn |  |  |
|  | Digital Insights | yjahn@digitalinsights.co.kr |  |  |
| YuChen Sun | Alibaba | yuchen.s@alibaba-inc.com jiechen.cj@alibaba-inc.com ruling.lrl@alibaba-inc.com |  |  |
| Hong-Jheng Jhu | National Chiao Tung University | ted0958.am96@g2.nctu.edu.tw |  |  |
| Xiaozhen Zheng | DJI | xiaozhen.zheng@dji.com |  |  |
| Yong-uk Yoon | Korea Aerospace University | yuyoon@kau.kr |  |  |